# New Zealand’s 
Greenhouse Gas Inventory 
1990–2017

# Volume 2, Annexes

Fulfilling reporting requirements under the 
United Nations Framework Convention on 
Climate Change and the Kyoto Protocol

Contents

Annex 1: Key categories	11

A1.1 	Methodology used for identifying key categories	11

A1.2 	Disaggregation	12

A1.3 	Tables 4.2–4.3 of the IPCC 2006 Guidelines (General Guidance and Reporting)	12

Annex 1: References	31

Annex 2: Uncertainty analysis	32

A2.1 	Approach 1 uncertainty calculation	32

Annex 2: References	43

Annex 3: Detailed methodological information for other sectors	44

A3.1	Supplementary information for the Agriculture sector	44

A3.2	Supplementary information for the LULUCF sector	50

Annex 3: References	66

Annex 4: Methodology and data collection for estimating emissions from fossil fuel combustion	68

A4.1	Emissions from liquid fuels	71

A4.2	Emissions from solid fuels	74

A4.3	Emissions from gaseous fuels	75

A4.4	Energy balance	75

A4.5	Carbon dioxide reference approach for the Energy sector	76

Annex 4: References	78

Annex 5: Supplementary information for the KP-LULUCF sector	79

A5.1	Technical corrections to the FMRL	79

A5.2 	Natural disturbance	90

A5.3	Carbon equivalent forests	93

Annex 5: References	95

Annex 6: Additional information on the inventory system and completeness	96

A6.1 	Quality assurance and quality control processes	96

A6.2 	General assessment of completeness	99

Annex 6: References	106

Annex 7: Tokelau	107

A7.1 	Preliminary processing of raw data to obtain activity data for liquid and gaseous fuels from Tokelau	107

A7.2	Emissions estimate data and relevant supporting information by category for Tokelau	109

Annex 8: Agricultural emissions from fertilisers and by livestock type	196

A8.1 	Agricultural emissions from fertilisers	196

A8.2 	Agricultural emissions by livestock type	198

See volume 1, chapters 1–15, for the national inventory report, which is presented separately.

Tables

Table A1.3.1(a)	Results of the key category level analysis for 99 per cent of the net emissions and removals for New Zealand in 2017	12

Table A1.3.1(b)	Results of the key category level analysis for 99 per cent of the gross emissions and removals for New Zealand in 2017	15

Table A1.3.2(a)	Results of the level analysis for 99 per cent of the net emissions and removals for New Zealand in 1990 included for reference only	18

Table A1.3.2(b)	Results of the level analysis for 99 per cent of the gross emissions  for New Zealand in 1990 included for reference only	20

Table A1.3.3(a)	Results of the key category trend analysis for 99 per cent of the net emissions and removals for New Zealand in 1990–2017	23

Table A1.3.3(b)	Results of the key category trend analysis for 99 per cent of the gross emissions for New Zealand in 1990–2017	26

Table A2.1.1 	Uncertainty calculation (including LULUCF) for New Zealand’s Greenhouse Gas Inventory 1990–2017 (IPCC, 2006, Approach 1)	33

Table A2.1.2 	Uncertainty calculation (excluding LULUCF) for New Zealand’s Greenhouse Gas Inventory 1990–2017 (2006 IPCC, Approach 1)	38

Table A3.1.1	Imputation levels and sampling errors for recent Agricultural Production surveys	44

Table A3.1.2	Characterisation of major livestock subcategories (dairy cattle, non-dairy cattle, sheep and deer) in New Zealand’s Tier 2 livestock modelling	45

Table A3.1.3	Proportion of annual milk yield each month for major livestock categories	46

Table A3.1.4	Emission factors for Tier 1 enteric fermentation livestock and manure management	47

Table A3.1.5	Emission factors for New Zealand’s agriculture nitrous oxide emissions	47

Table A3.1.6	Parameter values for New Zealand’s agriculture nitrous oxide emissions	48

Table A3.1.7	Parameter values for New Zealand’s cropping emissions	49

Table A3.2.1 	Annual land-use changes (units in 000s hectares)	50

Table A3.2.2	Pre-1990 natural forest – regenerating forest yield table (tonnes C ha–1)	56

Table A3.2.3	Uncertainty analysis for the LULUCF sector	57

Table A4.1 	Gross carbon dioxide emission factors used for New Zealand’s energy sector in 2017	68

Table A4.2 	Consumption-weighted average emission factors used for New Zealand’s sub-bituminous coal-fired electricity generation for 1990 to 2017	69

Table A4.3 	Methane emission factors used for New Zealand’s energy sector for 1990 to 2017	70

Table A4.4 	Nitrous oxide emission factors used for New Zealand’s energy sector for 1990 to 2017	70

Table A4.5 	Gross calorific values (MJ/kg) for liquid fuels for 1990 to 2017	72

Table A4.6 	Carbon content (per cent mass) for liquid fuels for 1990 to 2017	72

Table A4.7 	Emission factors for European gasoline and diesel vehicles – COPERT IV model (European Environment Agency, 2007)	73

Table A4.8 	Split of energy use and non-energy use of natural gas in petajoules	75

Table A5.1.1 	Pre-1990 planted forest data used to estimate emissions for the technically corrected FMRL	82

Table A5.1.2	Summary of the technical corrections to the FMRL	86

Table A5.1.3 	Contribution of each source to the FMRLcorr	89

Table A5.3.1	Breakdown of carbon equivalent forests by domestic scheme application, 2014–17	93

Table A6.1.1 	Quality assurance and quality control processes used in preparation of the inventory	96

Table A6.2.1 	Summary of NE (not estimated) entries in 2019 submission	100

Table A6.2.2 	Emissions reported using the ‘IE’ (included elsewhere) notation key	102

Table A7.1.1 	Allocation of 2018 fuel sales	107

Table A7.1.2 	Tokelau vehicle photo survey August–December 2018	108

Tokelau CRF Table 1.A.1.a: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.1  Energy Industries][1.A.1.a  Public Electricity and Heat Production] (Part 1 of 3)	109

Tokelau CRF Table 1.A.1.a: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.1  Energy Industries][1.A.1.a  Public Electricity and Heat Production] (Part 2 of 3)	111

Tokelau CRF Table 1.A.1.a: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.1  Energy Industries][1.A.1.a  Public Electricity and Heat Production] (Part 3 of 3)	113

Tokelau CRF Table 1.A.3.b.i: [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i  Cars][Gasoline] (Part 1 of 3)	115

Tokelau CRF Table 1.A.3.b.i: [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i  Cars][Gasoline] (Part 2 of 3)	116

Tokelau CRF Table 1.A.3.b.i: [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i  Cars][Gasoline] (Part 3 of 3)	117

CRF Table 1.A.3.b.i Diesel Oil: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.3  Transport][1.A.3.b  Road Transportation][1.A.3.b.i  Cars][Diesel Oil]  (Part 1 of 3)	118

CRF Table 1.A.3.b.i Diesel Oil: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.3  Transport][1.A.3.b  Road Transportation][1.A.3.b.i  Cars][Diesel Oil]  (Part 2 of 3)	119

CRF Table 1.A.3.b.i Diesel Oil: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.3  Transport][1.A.3.b  Road Transportation][1.A.3.b.i  Cars][Diesel Oil]  (Part 3 of 3)	120

CRF Table 1.A.3.d Gas/Diesel Oil: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.3  Transport][1.A.3.d  Domestic Navigation][Gas/Diesel Oil] (Part 1 of 3)	121

CRF Table 1.A.3.d Gas/Diesel Oil: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.3  Transport][1.A.3.d  Domestic Navigation][Gas/Diesel Oil (Part 2 of 3)	122

CRF Table 1.A.3.d Gas/Diesel Oil: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.3  Transport][1.A.3.d  Domestic Navigation][Gas/Diesel Oil (Part 3 of 3)	123

CRF Table 1.A.4.b: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.4  Other Sectors][1.A.4.b  Residential] (Part 1 of 3)	124

CRF Table 1.A.4.b: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.4  Other Sectors][1.A.4.b  Residential] (Part 2 of 3)	126

CRF Table 1.A.4.b: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.4  Other Sectors][1.A.4.b  Residential] (Part 3 of 3)	128

CRF Table 1.A.4.c.iii Gas/Diesel Oil: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.4  Other Sectors][1.A.4.c  Agriculture/Forestry/Fishing] [1.A.4.c.iii  Fishing][Gas/Diesel Oil] (Part 1 of 3)	130

CRF Table 1.A.4.c.iii Gas/Diesel Oil: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.4  Other Sectors][1.A.4.c  Agriculture/Forestry/Fishing][1.A.4.c.iii  Fishing] [Gas/Diesel Oil] (Part 2 of 3)	131

CRF Table 1.A.4.c.iii Gas/Diesel Oil: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.4  Other Sectors][1.A.4.c  Agriculture/Forestry/Fishing][1.A.4.c.iii  Fishing] [Gas/Diesel Oil] (Part 3 of 3)	132

CRF Table 1.AB Gasoline: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Gasoline] (Part 1 of 3)	133

CRF Table 1.AB Gasoline: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Gasoline] (Part 2 of 3)	134

CRF Table 1.AB Gasoline: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Gasoline] (Part 3 of 3)	135

CRF Table 1.AB Gasoline: [1.  Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Gas / Diesel Oil] (Part 1 of 3)	136

CRF Table 1.AB Gasoline: [1.  Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Gas / Diesel Oil] (Part 2 of 3)	137

CRF Table 1.AB Gasoline: [1.  Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Gas / Diesel Oil] (Part 3 of 3)	138

CRF Table 1.AB Other Kerosene: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Other Kerosene] (Part 1 of 3)	139

CRF Table 1.AB Other Kerosene: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Other Kerosene] (Part 2 of 3)	140

CRF Table 1.AB Other Kerosene: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Other Kerosene] (Part 3 of 3)	141

CRF Table 1.AB LPG: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] (Part 1 of 3)	142

CRF Table 1.AB LPG: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] (Part 2 of 3)	143

CRF Table 1.AB LPG: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] (Part 3 of 3)	144

CRF Table 1.AB Lubricants: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Lubricants] (Part 1 of 3)	145

CRF Table 1.AB Lubricants: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Lubricants] (Part 2 of 3)	146

CRF Table 1.AB Lubricants: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Lubricants] (Part 3 of 3)	147

CRF Table 2.F.1.b HFC-134a Product Uses as Substitutes for ODS: [2.  Industrial Processes and Product Use][ 2.F  Product Uses as Substitutes for ODS][ 2.F.1  Refrigeration and Air conditioning][ 2.F.1.b  Domestic Refrigeration][ HFC-134a] (Part 1 of 3)	148

CRF Table 2.F.1.b HFC-134a Product Uses as Substitutes for ODS: [2.  Industrial Processes and Product Use][ 2.F  Product Uses as Substitutes for ODS][ 2.F.1  Refrigeration  and Air conditioning][ 2.F.1.b  Domestic Refrigeration][ HFC-134a] (Part 2 of 3)	149

CRF Table 2.F.1.b HFC-134a Product Uses as Substitutes for ODS: [2.  Industrial Processes and Product Use][ 2.F  Product Uses as Substitutes for ODS][ 2.F.1  Refrigeration and Air conditioning][ 2.F.1.b  Domestic Refrigeration][ HFC-134a] (Part 3 of 3)	150

CRF Table 2.F.1.f: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air Conditioning] (Part 1 of 3)	151

CRF Table 2.F.1.f: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning] (Part 2 of 3)	152

CRF Table 2.F.1.f: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning] (Part 3 of 3)	153

CRF Table 2.F.1.f HFC-32: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-32] (Part 1 of 3)	154

CRF Table 2.F.1.f HFC-32: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-32] (Part 2 of 3)	155

CRF Table 2.F.1.f HFC-32: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-32] (Part 3 of 3)	156

CRF Table 2.F.1.f HFC-125: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-125] (Part 1 of 3)	157

CRF Table 2.F.1.f HFC-125: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-125] (Part 2 of 3)	158

CRF Table 2.F.1.f HFC-125: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-125] (Part 3 of 3)	159

CRF Table 2.F.1.f HFC-134a: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-134a] (Part 1 of 3)	160

CRF Table 2.F.1.f HFC-134a: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-134a] (Part 2 of 3)	161

CRF Table 2.F.1.f HFC-134a: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-134a] (Part 3 of 3)	162

CRF Table 2.F.4.a: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.4  Aerosols][2.F.4.a  Metered Dose Inhalers] (Part 1 of 3)	163

CRF Table 2.F.4.a: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.4  Aerosols][2.F.4.a  Metered Dose Inhalers] (Part 2 of 3)	164

CRF Table 2.F.4.a: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.4  Aerosols][2.F.4.a  Metered Dose Inhalers] (Part 3 of 3)	165

CRF Table 2.F.4.a HFC-134a: [ 2.  Industrial Processes and Product Use][ 2.F  Product Uses as Substitutes for ODS][ 2.F.4  Aerosols][ 2.F.4.a  Metered Dose Inhalers][ HFC-134a]  (Part 1 of 3)	166

CRF Table 2.F.4.a HFC-134a: [ 2.  Industrial Processes and Product Use][ 2.F  Product Uses as Substitutes for ODS][ 2.F.4  Aerosols][ 2.F.4.a  Metered Dose Inhalers][ HFC-134a]  (Part 2 of 3)	167

CRF Table 2.F.4.a HFC-134a: [ 2.  Industrial Processes and Product Use][ 2.F  Product Uses as Substitutes for ODS][ 2.F.4  Aerosols][ 2.F.4.a  Metered Dose Inhalers][ HFC-134a]  (Part 3 of 3)	167

CRF Table 2.F.4.a HFC-227ea: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.4  Aerosols][2.F.4.a  Metered Dose Inhalers][HFC-227ea]  (Part 1 of 3)	168

CRF Table 2.F.4.a HFC-227ea: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.4  Aerosols][2.F.4.a  Metered Dose Inhalers][HFC-227ea]  (Part 2 of 3)	169

CRF Table 2.F.4.a HFC-227ea: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.4  Aerosols][2.F.4.a  Metered Dose Inhalers][HFC-227ea]  (Part 3 of 3)	169

CRF Table 2.G.3.a: [2.  Industrial Processes and Product Use][2.G  Other Product Manufacture and Use][2.G.3  N2O from Product Uses][2.G.3.a  Medical Applications] (Part 1 of 3)	170

CRF Table 2.G.3.a: [2.  Industrial Processes and Product Use][2.G  Other Product Manufacture and Use][2.G.3  N2O from Product Uses][2.G.3.a  Medical Applications] (Part 2 of 3)	171

CRF Table 2.G.3.a: [2.  Industrial Processes and Product Use][2.G  Other Product Manufacture and Use][2.G.3  N2O from Product Uses][2.G.3.a  Medical Applications] (Part 3 of 3)	171

CRF Table 3.A.3 Tokelau Swine: [ 3.  Agriculture][ 3.1  Livestock][ 3.A  Enteric Fermentation][ 3.A.3  Swine][ Other (please specify)][ Tokelau_Swine] (Part 1 of 3)	172

CRF Table 3.A.3 Tokelau Swine: [ 3.  Agriculture][ 3.1  Livestock][ 3.A  Enteric Fermentation][ 3.A.3  Swine][ Other (please specify)][ Tokelau_Swine] (Part 2 of 3)	173

CRF Table 3.A.3 Tokelau Swine: [ 3.  Agriculture][ 3.1  Livestock][ 3.A  Enteric Fermentation][ 3.A.3  Swine][ Other (please specify)][ Tokelau_Swine] (Part 3 of 3)	174

CRF Table 3.A.3 Tokelau Poultry [ 3. Agriculture][ 3.1  Livestock][ 3.A  Enteric Fermentation][ 3.A.4  Other livestock][ Tokelau_Poultry] (Part 1 of 3)	175

CRF Table 3.A.3 Tokelau Poultry [ 3. Agriculture][ 3.1  Livestock][ 3.A  Enteric Fermentation][ 3.A.4  Other livestock][ Tokelau_Poultry] (Part 2 of 3)	176

CRF Table 3.A.3 Tokelau Poultry [ 3. Agriculture][ 3.1  Livestock][ 3.A  Enteric Fermentation][ 3.A.4  Other livestock][ Tokelau_Poultry] (Part 3 of 3)	177

CRF Table 3.B.1.3 Tokelau Swine: [3.  Agriculture][ 3.1  Livestock][ 3.B  Manure Management][ 3.B.1  CH4 Emissions][ 3.B.1.3  Swine][ Other (please specify)][ Pigs] (Part 1 of 3)	178

CRF Table 3.B.1.3 Tokelau Swine: [3.  Agriculture][ 3.1  Livestock][ 3.B  Manure Management][ 3.B.1  CH4 Emissions][ 3.B.1.3  Swine][ Other (please specify)][ Pigs] (Part 2 of 3)	179

CRF Table 3.B.1.3 Tokelau Swine: [3.  Agriculture][ 3.1  Livestock][ 3.B  Manure Management][ 3.B.1  CH4 Emissions][ 3.B.1.3  Swine][ Other (please specify)][ Pigs] (Part 3 of 3)	180

CRF Table 3.B.1.4 Tokelau Poultry: [ 3.  Agriculture][ 3.1  Livestock][ 3.B  Manure Management][ 3.B.1  CH4 Emissions][ 3.B.1.4  Other livestock][ Tokelau_Poultry] Part 1 of 3)	181

CRF Table 3.B.1.4 Tokelau Poultry: [ 3.  Agriculture][ 3.1  Livestock][ 3.B  Manure Management][ 3.B.1  CH4 Emissions][ 3.B.1.4  Other livestock][ Tokelau_Poultry] Part 2 of 3)	182

CRF Table 3.B.1.4 Tokelau Poultry: [ 3.  Agriculture][ 3.1  Livestock][ 3.B  Manure Management][ 3.B.1  CH4 Emissions][ 3.B.1.4  Other livestock][ Tokelau_Poultry] Part 3 of 3)	183

CRF Table 5.C: [5.  Waste][5.C  Incineration and Open Burning of Waste]   (Part 1 of 3)	184

CRF Table 5.C: [5.  Waste][5.C  Incineration and Open Burning of Waste]   (Part 2 of 3)	185

CRF Table 5.C: [5.  Waste][5.C  Incineration and Open Burning of Waste]   (Part 3 of 3)	186

CRF Table 5.C.2: [5.  Waste] [5.C  Incineration and Open Burning of Waste][5.C.2  Open Burning of Waste] (Part 1 of 3)	187

CRF Table 5.C.2: [5.  Waste] [5.C  Incineration and Open Burning of Waste][5.C.2  Open Burning of Waste] (Part 2 of 3)	188

CRF Table 5.C.2: [5.  Waste] [5.C Incineration and Open Burning of Waste][5.C.2  Open Burning of Waste] (Part 3 of 3)	189

CRF Table 5.C.2.2: [5.  Waste][5.C  Incineration and Open Burning of Waste][5.C.2  Open Burning of Waste][5.C.2.2  Non-biogenic] (Part 1 of 3)	190

CRF Table 5.C.2.2: [5.  Waste][5.C  Incineration and Open Burning of Waste][5.C.2  Open Burning of Waste][5.C.2.2  Non-biogenic] (Part 2 of 3)	191

CRF Table 5.C.2.2: [5.  Waste][5.C  Incineration and Open Burning of Waste][5.C.2  Open Burning of Waste][5.C.2.2  Non-biogenic] (Part 3 of 3)	192

CRF Table 5.D: [5.  Waste][5.D  Wastewater Treatment and Discharge] (Part 1 of 3)	193

CRF Table 5.D: [5.  Waste][5.D  Wastewater Treatment and Discharge] (Part 2 of 3)	194

CRF Table 5.D: [5.  Waste][5.D  Wastewater Treatment and Discharge] (Part 3 of 3)	195

Table A8.1.1 	Direct and indirect emissions by fertiliser in 2017	196

Table A8.1.2 	Use of fertilisers in New Zealand in 1990 and 2017	197

Table A8.2.3 	Emissions from fertilisers in 1990 and 2017	198

Table A8.2.1 	Total emissions by livestock type in 1990, 2016 and 2017	198

Figures

Figure A1.1.1 	Decision tree to identify key source categories (figure 4.2, IPCC, 2006)	11

Figure A3.2.1	New Zealand’s LUCAS data management system	61

Figure A3.2.2	New Zealand’s Geospatial System components	62

Figure A3.2.3 	LUCAS Management Studio	63

Figure A3.2.4	LUCAS Gateway database	64

Figure A4.1 	Reference and sectoral approach carbon dioxide by fuel type (kt CO2)	77

Figure A5.1.1	Technical corrections to the FMRL by category	87

Figure A5.1.2	Comparison of the 2011 FMRL, technical corrections to ensure consistency with Forest management reporting, and total of technical corrections over the period to 2020	88

Figure A5.2.1 	Calculating the background level of natural disturbance during the calibration period for Afforestation and reforestation land	91

Figure A5.2.2 	Emissions from natural disturbance during the calibration period on Forest management land	92

Figure A6.1.1 	How the quality assurance and quality control processes and products align with the preparation of the inventory	97

Figure A6.1.2 	Data processsing, quality assurance and quality control processes 
during the inventory prepartion of the data from Tokelau 	98

Figure A8.1.1 	Use of fertilisers (tonnes (N)) in New Zealand in 1990 and 2017	197

# Annex 1: Key categories

## A1.1 	Methodology used for identifying key categories

The key categories in the inventory have been assessed using the Approach 1 level and trend methodologies from the 2006 IPCC Guidelines for National Greenhouse Gas Inventories (IPCC, 2006). The methodology applied was determined using the decision tree shown in figure A1.1.1. Because some categories in the inventory apply default uncertainties values for emission estimates, and developing country-specific uncertainty values is resource prohibitive, Approach 1 level and trend methodologies are used.

Figure A1.1.1 	Decision tree to identify key source categories (figure 4.2, IPCC, 2006)

For this inventory submission, the Approach 1 level and trend assessments were applied, including the Land Use, Land-Use Change and Forestry (LULUCF) sector and excluding the LULUCF sector (IPCC, 2003).

The ‘including LULUCF’ level and trend assessments are calculated as per equations 5.4.1 and 5.4.2 of Good Practice Guidance for Land Use, Land-Use Change and Forestry (IPCC, 2003). The ‘excluding LULUCF’ level and trend assessments are calculated as per equations 4.1 and 4.2 of the IPCC 2006 Guidelines (IPCC, 2006). Key categories are defined as those categories whose cumulative percentages, when summed in decreasing order of magnitude, contributed 95 per cent of the total level or trend.

## A1.2 	Disaggregation

The classification of categories follows the classification of the common reporting format (CRF) tables by:

- identifying categories using carbon dioxide equivalent emissions and considering each greenhouse gas from each category separately

- either including or excluding LULUCF categories at the level shown in Good Practice Guidance for Land Use, Land-Use Change and Forestry table 5.4.1 (IPCC, 2003).

The level of aggregation used for the key category analysis is similar to the default aggregation used for the key category analysis within the CRF tables, with adjustments to better reflect New Zealand’s emissions profile. Specifically, a large proportion of emissions from the Energy and Agriculture sectors are disaggregated further than the key category analysis generated in the CRF tables, to allow for a more evenly proportioned analysis of categories.

## A1.3 	Tables 4.2–4.3 of the IPCC 2006 Guidelines (General Guidance and Reporting)

The following tables specify the level analyses for 2017 and 1990, and trend analyses, each including and excluding LULUCF. The tables show the categories that comprise 99 per cent of emissions for each analysis. Only the categories that comprise the top 95 per cent of emissions for the 2017 level analysis and the trend analysis are key categories. The 1990 level analysis tables are included for information only.

Table A1.3.1(a)	Results of the key category level analysis for 99 per cent of the net emissions
and removals for New Zealand in 2017

| IPCC Tier 1 category level assessment – including LULUCF (net emissions): 2017 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CRF category code | IPCC category | Gas | 2017 estimate (kt CO2-e) | Level assessment (%) | Cumulative total (%) |
| 4.A.2 | Forest Land – Land Converted to Forest Land | CO2 | –16,252.4 | 14.3 | 14.3 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CO2 | 14,335.0 | 12.6 | 26.9 |
| 3.A.1 | Option A – Dairy Cattle | CH4 | 13,560.1 | 11.9 | 38.9 |
| 3.A.2 | Other (please specify) – Sheep | CH4 | 8,253.1 | 7.3 | 46.1 |
| 4.G | Land Use, Land-Use Change and Forestry – Harvested Wood Products | CO2 | –6,518.5 | 5.7 | 51.9 |
| 4.A.1 | Forest Land – Forest Land Remaining Forest Land | CO2 | –5,587.1 | 4.9 | 56.8 |
| 3.D.1.3 | Direct N2O Emissions from Managed Soils – Urine and Dung Deposited by Grazing Animals | N2O | 5,435.3 | 4.8 | 61.6 |
| 3.A.1 | Option A – Non-dairy Cattle | CH4 | 5,298.0 | 4.7 | 66.2 |
| 5.A | Waste – Solid Waste Disposal | CH4 | 3,724.5 | 3.3 | 69.5 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Gaseous Fuels | CO2 | 3,065.2 | 2.7 | 72.2 |
| 4.C.2 | Grassland – Land Converted to Grassland | CO2 | 2,437.1 | 2.1 | 74.3 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Solid Fuels | CO2 | 1,784.0 | 1.6 | 75.9 |
| 2.C.1 | Metal Industry – Iron and Steel Production | CO2 | 1,758.3 | 1.5 | 77.5 |
| 1.A.2.c | Manufacturing Industries and Construction – Chemicals Gaseous Fuels | CO2 | 1,587.8 | 1.4 | 78.9 |
| 2.F.1 | Product Uses as Substitutes for ODS – Refrigeration and Air Conditioning | HFCs | 1,397.3 | 1.2 | 80.1 |
| 3.D.1.1 | Direct N2O Emissions from Managed Soils – Inorganic N Fertilisers | N2O | 1,365.6 | 1.2 | 81.3 |
| 3.B.1.1 | Option A – Dairy Cattle | CH4 | 1,255.8 | 1.1 | 82.4 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | CO2 | 1,215.1 | 1.1 | 83.5 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Liquid Fuels | CO2 | 1,163.8 | 1.0 | 84.5 |
| 1.A.3.a | Domestic Aviation – Jet Kerosene | CO2 | 961.7 | 0.8 | 85.3 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Gaseous Fuels | CO2 | 954.9 | 0.8 | 86.2 |
| 3.D.2.1 | Indirect N2O Emissions from Managed Soils – Atmospheric Deposition | N2O | 906.8 | 0.8 | 87.0 |
| 1.A.1.b | Energy Industries – Petroleum Refining Liquid Fuels | CO2 | 674.5 | 0.6 | 87.6 |
| 1.B.2.d | Other (please specify) – Geothermal | CO2 | 643.3 | 0.6 | 88.1 |
| 3.H | Agriculture – Urea Application | CO2 | 588.3 | 0.5 | 88.6 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Liquid Fuels | CO2 | 584.4 | 0.5 | 89.2 |
| 2.C.3 | Metal Industry – Aluminium Production | CO2 | 546.2 | 0.5 | 89.6 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Liquid Fuels | CO2 | 530.8 | 0.5 | 90.1 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Solid Fuels | CO2 | 522.7 | 0.5 | 90.6 |
| 3.D.2.2 | Indirect N2O Emissions from Managed Soils – Nitrogen Leaching and Run-off | N2O | 500.5 | 0.4 | 91.0 |
| 3.A.4 | Other Livestock – Deer | CH4 | 486.0 | 0.4 | 91.4 |
| 3.G | Agriculture – Liming | CO2 | 459.6 | 0.4 | 91.8 |
| 2.A.1 | Mineral Industry – Cement Production | CO2 | 443.8 | 0.4 | 92.2 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Gaseous Fuels | CO2 | 429.8 | 0.4 | 92.6 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Gaseous Fuels | CO2 | 379.6 | 0.3 | 92.9 |
| 1.A.4.b | Other Sectors – Residential Gaseous Fuels | CO2 | 365.1 | 0.3 | 93.3 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Liquid Fuels | CO2 | 351.8 | 0.3 | 93.6 |
| 4.B.1 | Cropland – Cropland Remaining Cropland | CO2 | 332.0 | 0.3 | 93.9 |
| 1.A.3.d | Domestic Navigation – Residual Fuel Oil | CO2 | 319.7 | 0.3 | 94.2 |
| 1.A.1.c | Energy Industries – Manufacture of Solid Fuels and Other Energy Industries Gaseous Fuels | CO2 | 305.9 | 0.3 | 94.4 |
| 1.B.2.c.2.iii | Flaring – Combined | CO2 | 292.7 | 0.3 | 94.7 |
| 1.B.2.c.1.ii | Venting – Gas | CO2 | 258.5 | 0.2 | 94.9 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Solid Fuels | CO2 | 250.6 | 0.2 | 95.1 |
| 3.D.1.4 | Direct N2O Emissions from Managed Soils – Crop Residues | N2O | 249.4 | 0.2 | 95.3 |
| 5.D | Waste – Wastewater Treatment and Discharge | CH4 | 246.9 | 0.2 | 95.6 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Solid Fuels | CO2 | 244.8 | 0.2 | 95.8 |
| 1.A.4.b | Other Sectors – Residential Liquid Fuels | CO2 | 223.4 | 0.2 | 96.0 |
| 1.B.2.b.5 | Natural Gas – Distribution | CH4 | 208.4 | 0.2 | 96.2 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Liquid Fuels | CO2 | 205.8 | 0.2 | 96.3 |
| 2.B.10 | Chemical Industry – Other (please specify) | CO2 | 173.8 | 0.2 | 96.5 |
| 1.B.2.d | Other (please specify) – Geothermal | CH4 | 171.5 | 0.2 | 96.6 |
| 1.A.1.b | Energy Industries – Petroleum Refining Gaseous Fuels | CO2 | 169.9 | 0.1 | 96.8 |
| 2.A.2 | Mineral Industry – Lime Production | CO2 | 138.9 | 0.1 | 96.9 |
| 1.B.2.b.2 | Natural Gas – Production | CH4 | 133.2 | 0.1 | 97.0 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Solid Fuels | CO2 | 118.2 | 0.1 | 97.1 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Gaseous Fuels | CO2 | 117.1 | 0.1 | 97.2 |
| 5.D | Waste – Wastewater Treatment and Discharge | N2O | 114.6 | 0.1 | 97.3 |
| 2.B.8 | Chemical Industry – Petrochemical and Carbon Black Production | CH4 | 112.0 | 0.1 | 97.4 |
| 1.A.3.c | Transport – Railways Liquid Fuels | CO2 | 111.3 | 0.1 | 97.5 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Liquid Fuels | CO2 | 109.6 | 0.1 | 97.6 |
| 3.B.2.5 | N2O and NMVOC Emissions – Indirect N2O Emissions | N2O | 106.1 | 0.1 | 97.7 |
| 1.A.2.a | Manufacturing Industries and Construction – Iron and Steel Gaseous Fuels | CO2 | 102.0 | 0.1 | 97.8 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | N2O | 99.7 | 0.1 | 97.9 |
| 2.F.4 | Product Uses as Substitutes for ODS – Aerosols | HFCs | 96.5 | 0.1 | 98.0 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Solid Fuels | CO2 | 92.4 | 0.1 | 98.1 |
| 2.A.4 | Mineral Industry – Other Process Uses of Carbonates | CO2 | 87.0 | 0.1 | 98.1 |
| 3.B.1.2 | CH4 Emissions – Sheep | CH4 | 85.5 | 0.1 | 98.2 |
| 3.D.1.2 | Direct N2O Emissions from Managed Soils – Organic N Fertilisers | N2O | 78.5 | 0.1 | 98.3 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Gaseous Fuels | CO2 | 78.4 | 0.1 | 98.4 |
| 1.B.1.a.1 | Coal Mining and Handling – Underground Mines | CH4 | 71.9 | 0.1 | 98.4 |
| 3.B.1.1 | Option A – Non-dairy Cattle | CH4 | 71.2 | 0.1 | 98.5 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | CH4 | 68.1 | 0.1 | 98.5 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Liquid Fuels | CO2 | 67.0 | 0.1 | 98.6 |
| 4.E.1 | Settlements – Settlements Remaining Settlements | CO2 | 65.1 | 0.1 | 98.7 |
| 2.G.3 | Other Product Manufacture and Use – N2O from Product Uses | N2O | 61.8 | 0.1 | 98.7 |
| 2.C.3 | Metal Industry – Aluminium Production | PFCs | 60.5 | 0.1 | 98.8 |
| 1.B.1.a.2 | Coal Mining and Handling – Surface Mines | CH4 | 60.1 | 0.1 | 98.8 |
| 1.B.2.c.2.iii | Flaring – Combined | CH4 | 58.6 | 0.1 | 98.9 |
| 4.A.2 | Forest Land – Land Converted to Forest Land | N2O | 57.3 | 0.1 | 98.9 |
| 4.B.2 | Cropland – Land Converted to Cropland | CO2 | 55.4 | 0.0 | 99.0 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Solid Fuels | CO2 | 53.0 | 0.0 | 99.0 |

Note:	Key categories are those that comprise 95 per cent of the total. Removals from the LULUCF sector are shown as negatives in this table. The absolute values for those removals were used for the calculations.

Table A1.3.1(b)	Results of the key category level analysis for 99 per cent of the gross emissions
and removals for New Zealand in 2017

| IPCC Tier 1 category level assessment – gross emissions (excluding LULUCF): 2017 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CRF category code | IPCC Category | Gas | 2017 estimate (kt CO2-e) | Level assessment (%) | Cumulative total (%) |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CO2 | 14,335.0 | 17.7 | 17.7 |
| 3.A.1 | Option A – Dairy Cattle | CH4 | 13,560.1 | 16.8 | 34.5 |
| 3.A.2 | Other (please specify) – Sheep | CH4 | 8,253.1 | 10.2 | 44.7 |
| 3.D.1.3 | Direct N2O Emissions from Managed Soils – Urine and Dung Deposited by Grazing Animals | N2O | 5,435.3 | 6.7 | 51.4 |
| 3.A.1 | Option A – Non-dairy Cattle | CH4 | 5,298.0 | 6.6 | 58.0 |
| 5.A | Waste – Solid Waste Disposal | CH4 | 3,724.5 | 4.6 | 62.6 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Gaseous Fuels | CO2 | 3,065.2 | 3.8 | 66.4 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Solid Fuels | CO2 | 1,784.0 | 2.2 | 68.6 |
| 2.C.1 | Metal Industry – Iron and Steel Production | CO2 | 1,758.3 | 2.2 | 70.8 |
| 1.A.2.c | Manufacturing Industries and Construction – Chemicals Gaseous Fuels | CO2 | 1,587.8 | 2.0 | 72.7 |
| 2.F.1 | Product Uses as Substitutes for ODS – Refrigeration and Air Conditioning | HFCs | 1,397.3 | 1.7 | 74.5 |
| 3.D.1.1 | Direct N2O Emissions from Managed Soils – Inorganic N Fertilisers | N2O | 1,365.6 | 1.7 | 76.1 |
| 3.B.1.1 | Option A – Dairy Cattle | CH4 | 1,255.8 | 1.6 | 77.7 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Liquid Fuels | CO2 | 1,163.8 | 1.4 | 79.1 |
| 1.A.3.a | Domestic Aviation – Jet Kerosene | CO2 | 961.7 | 1.2 | 80.3 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Gaseous Fuels | CO2 | 954.9 | 1.2 | 81.5 |
| 3.D.2.1 | Indirect N2O Emissions from Managed Soils – Atmospheric Deposition | N2O | 906.8 | 1.1 | 82.6 |
| 1.A.1.b | Energy Industries – Petroleum Refining Liquid Fuels | CO2 | 674.5 | 0.8 | 83.5 |
| 1.B.2.d | Other (please specify) – Geothermal | CO2 | 643.3 | 0.8 | 84.3 |
| 3.H | Agriculture – Urea Application | CO2 | 588.3 | 0.7 | 85.0 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Liquid Fuels | CO2 | 584.4 | 0.7 | 85.7 |
| 2.C.3 | Metal Industry – Aluminium Production | CO2 | 546.2 | 0.7 | 86.4 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Liquid Fuels | CO2 | 530.8 | 0.7 | 87.0 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Solid Fuels | CO2 | 522.7 | 0.6 | 87.7 |
| 3.D.2.2 | Indirect N2O Emissions from Managed Soils – Nitrogen Leaching and Run-off | N2O | 500.5 | 0.6 | 88.3 |
| 3.A.4 | Other Livestock – Deer | CH4 | 486.0 | 0.6 | 88.9 |
| 3.G | Agriculture – Liming | CO2 | 459.6 | 0.6 | 89.5 |
| 2.A.1 | Mineral Industry – Cement Production | CO2 | 443.8 | 0.5 | 90.0 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Gaseous Fuels | CO2 | 429.8 | 0.5 | 90.6 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Gaseous Fuels | CO2 | 379.6 | 0.5 | 91.0 |
| 1.A.4.b | Other Sectors – Residential Gaseous Fuels | CO2 | 365.1 | 0.5 | 91.5 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Liquid Fuels | CO2 | 351.8 | 0.4 | 91.9 |
| 1.A.3.d | Domestic Navigation – Residual Fuel Oil | CO2 | 319.7 | 0.4 | 92.3 |
| 1.A.1.c | Energy Industries – Manufacture of Solid Fuels and Other Energy Industries Gaseous Fuels | CO2 | 305.9 | 0.4 | 92.7 |
| 1.B.2.c.2.iii | Flaring – Combined | CO2 | 292.7 | 0.4 | 93.0 |
| 1.B.2.c.1.ii | Venting – Gas | CO2 | 258.5 | 0.3 | 93.4 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Solid Fuels | CO2 | 250.6 | 0.3 | 93.7 |
| 3.D.1.4 | Direct N2O Emissions from Managed Soils – Crop Residues | N2O | 249.4 | 0.3 | 94.0 |
| 5.D | Waste – Wastewater Treatment and Discharge | CH4 | 246.9 | 0.3 | 94.3 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Solid Fuels | CO2 | 244.8 | 0.3 | 94.6 |
| 1.A.4.b | Other Sectors – Residential Liquid Fuels | CO2 | 223.4 | 0.3 | 94.9 |
| 1.B.2.b.5 | Natural Gas – Distribution | CH4 | 208.4 | 0.3 | 95.1 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Liquid Fuels | CO2 | 205.8 | 0.3 | 95.4 |
| 2.B.10 | Chemical Industry – Other (please specify) | CO2 | 173.8 | 0.2 | 95.6 |
| 1.B.2.d | Other (please specify) – Geothermal | CH4 | 171.5 | 0.2 | 95.8 |
| 1.A.1.b | Energy Industries – Petroleum Refining Gaseous Fuels | CO2 | 169.9 | 0.2 | 96.0 |
| 2.A.2 | Mineral Industry – Lime Production | CO2 | 138.9 | 0.2 | 96.2 |
| 1.B.2.b.2 | Natural Gas – Production | CH4 | 133.2 | 0.2 | 96.4 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Solid Fuels | CO2 | 118.2 | 0.1 | 96.5 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Gaseous Fuels | CO2 | 117.1 | 0.1 | 96.6 |
| 5.D | Waste – Wastewater Treatment and Discharge | N2O | 114.6 | 0.1 | 96.8 |
| 2.B.8 | Chemical Industry – Petrochemical and Carbon Black Production | CH4 | 112.0 | 0.1 | 96.9 |
| 1.A.3.c | Transport – Railways Liquid Fuels | CO2 | 111.3 | 0.1 | 97.1 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Liquid Fuels | CO2 | 109.6 | 0.1 | 97.2 |
| 3.B.2.5 | N2O and NMVOC Emissions – Indirect N2O Emissions | N2O | 106.1 | 0.1 | 97.3 |
| 1.A.2.a | Manufacturing Industries and Construction – Iron and Steel Gaseous Fuels | CO2 | 102.0 | 0.1 | 97.5 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | N2O | 99.7 | 0.1 | 97.6 |
| 2.F.4 | Product Uses as Substitutes for ODS – Aerosols | HFCs | 96.5 | 0.1 | 97.7 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Solid Fuels | CO2 | 92.4 | 0.1 | 97.8 |
| 2.A.4 | Mineral Industry – Other Process Uses of Carbonates | CO2 | 87.0 | 0.1 | 97.9 |
| 3.B.1.2 | CH4 Emissions – Sheep | CH4 | 85.5 | 0.1 | 98.0 |
| 3.D.1.2 | Direct N2O Emissions from Managed Soils – Organic N Fertilisers | N2O | 78.5 | 0.1 | 98.1 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Gaseous Fuels | CO2 | 78.4 | 0.1 | 98.2 |
| 1.B.1.a.1 | Coal Mining and Handling – Underground Mines | CH4 | 71.9 | 0.1 | 98.3 |
| 3.B.1.1 | Option A – Non-Dairy Cattle | CH4 | 71.2 | 0.1 | 98.4 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Liquid Fuels | CO2 | 67.0 | 0.1 | 98.5 |
| 2.G.3 | Other Product Manufacture and Use – N2O from Product Uses | N2O | 61.8 | 0.1 | 98.6 |
| 2.C.3 | Metal Industry – Aluminium Production | PFCs | 60.5 | 0.1 | 98.6 |
| 1.B.1.a.2 | Coal Mining and Handling – Surface Mines | CH4 | 60.1 | 0.1 | 98.7 |
| 1.B.2.c.2.iii | Flaring – Combined | CH4 | 58.6 | 0.1 | 98.8 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Solid Fuels | CO2 | 53.0 | 0.1 | 98.8 |
| 1.A.4.b | Other Sectors – Residential Biomass | CH4 | 52.2 | 0.1 | 98.9 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Biomass | N2O | 50.6 | 0.1 | 99.0 |
| 2.D | Industrial Processes and Product Use – Non-energy Products from Fuels and Solvent Use | CO2 | 46.7 | 0.1 | 99.0 |

Note:	Key categories are those that comprise 95 per cent of the total.

Table A1.3.2(a)	Results of the level analysis for 99 per cent of the net emissions and removals
for New Zealand in 1990 included for reference only

| IPCC Tier 1 category level assessment – including LULUCF (net emissions): 1990 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CRF category code | IPCC category | Gas | 1990 estimate (kt CO2-e) | Level assessment (%) | Cumulative total (%) |
| 4.A.2 | Forest Land – Land Converted to Forest Land | CO2 | –19,433.5 | 19.6 | 19.6 |
| 3.A.2 | Other (please specify) – Sheep | CH4 | 14,085.7 | 14.2 | 33.8 |
| 4.A.1 | Forest Land – Forest Land Remaining Forest Land | CO2 | –10,614.5 | 10.7 | 44.5 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CO2 | 7,164.6 | 7.2 | 51.8 |
| 3.A.1 | Option A – Dairy Cattle | CH4 | 5,928.0 | 6.0 | 57.7 |
| 3.A.1 | Option A – Non-dairy Cattle | CH4 | 5,723.4 | 5.8 | 63.5 |
| 3.D.1.3 | Direct N2O Emissions from Managed Soils – Urine and Dung Deposited by Grazing Animals | N2O | 5,138.4 | 5.2 | 68.7 |
| 5.A | Waste – Solid Waste Disposal | CH4 | 3,711.3 | 3.7 | 72.5 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Gaseous Fuels | CO2 | 3,011.8 | 3.0 | 75.5 |
| 4.G | Land Use, Land-Use Change and Forestry – Harvested Wood Products | CO2 | –2,072.9 | 2.1 | 77.6 |
| 1.A.1.c | Energy Industries – Manufacture of Solid Fuels and Other Energy Industries Gaseous Fuels | CO2 | 1,720.1 | 1.7 | 79.3 |
| 2.C.1 | Metal Industry – Iron and Steel Production | CO2 | 1,306.7 | 1.3 | 80.6 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Liquid Fuels | CO2 | 1,072.3 | 1.1 | 81.7 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Solid Fuels | CO2 | 938.6 | 0.9 | 82.7 |
| 2.C.3 | Metal Industry – Aluminium Production | PFCs | 909.9 | 0.9 | 83.6 |
| 1.A.3.a | Domestic Aviation – Jet Kerosene | CO2 | 892.6 | 0.9 | 84.5 |
| 1.A.1.b | Energy Industries – Petroleum Refining Liquid Fuels | CO2 | 778.9 | 0.8 | 85.3 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Solid Fuels | CO2 | 731.1 | 0.7 | 86.0 |
| 3.D.2.1 | Indirect N2O Emissions from Managed Soils – Atmospheric Deposition | N2O | 700.9 | 0.7 | 86.7 |
| 1.A.2.c | Manufacturing Industries and Construction – Chemicals Gaseous Fuels | CO2 | 528.7 | 0.5 | 87.3 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Liquid Fuels | CO2 | 500.6 | 0.5 | 87.8 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Solid Fuels | CO2 | 474.8 | 0.5 | 88.2 |
| 2.C.3 | Metal Industry – Aluminium Production | CO2 | 449.0 | 0.5 | 88.7 |
| 2.A.1 | Mineral Industry – Cement Production | CO2 | 448.7 | 0.5 | 89.1 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Gaseous Fuels | CO2 | 445.1 | 0.4 | 89.6 |
| 3.A.4 | Other Livestock – Deer | CH4 | 434.1 | 0.4 | 90.0 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Solid Fuels | CO2 | 382.9 | 0.4 | 90.4 |
| 3.D.2.2 | Indirect N2O Emissions from Managed Soils – Nitrogen Leaching and Run-off | N2O | 377.2 | 0.4 | 90.8 |
| 3.B.1.1 | Option A – Dairy Cattle | CH4 | 376.4 | 0.4 | 91.2 |
| 3.G | Agriculture – Liming | CO2 | 360.1 | 0.4 | 91.5 |
| 4.B.1 | Cropland – Cropland Remaining Cropland | CO2 | 358.7 | 0.4 | 91.9 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Gaseous Fuels | CO2 | 348.9 | 0.4 | 92.3 |
| 1.A.4.b | Other Sectors – Residential Solid Fuels | CO2 | 344.9 | 0.3 | 92.6 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Liquid Fuels | CO2 | 331.5 | 0.3 | 92.9 |
| 1.B.1.a.1 | Coal Mining and Handling – Underground Mines | CH4 | 289.6 | 0.3 | 93.2 |
| 1.B.2.b.5 | Natural Gas – Distribution | CH4 | 277.5 | 0.3 | 93.5 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Liquid Fuels | CO2 | 265.1 | 0.3 | 93.8 |
| 4.C.2 | Grassland – Land Converted to Grassland | CO2 | 262.9 | 0.3 | 94.0 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Gaseous Fuels | CO2 | 236.1 | 0.2 | 94.3 |
| 1.A.3.d | Domestic Navigation – Residual Fuel Oil | CO2 | 232.9 | 0.2 | 94.5 |
| 3.D.1.1 | Direct N2O Emissions from Managed Soils – Inorganic N Fertilisers | N2O | 230.3 | 0.2 | 94.7 |
| 1.B.2.d | Other (please specify) – Geothermal | CO2 | 228.6 | 0.2 | 95.0 |
| 5.D | Waste – Wastewater Treatment and Discharge | CH4 | 218.8 | 0.2 | 95.2 |
| 3.A.4 | Other Livestock – Goats | CH4 | 196.6 | 0.2 | 95.4 |
| 1.A.4.b | Other Sectors – Residential Gaseous Fuels | CO2 | 185.6 | 0.2 | 95.6 |
| 3.D.1.4 | Direct N2O Emissions from Managed Soils – Crop Residues | N2O | 175.5 | 0.2 | 95.8 |
| 1.A.4.b | Other Sectors – Residential Liquid Fuels | CO2 | 167.4 | 0.2 | 95.9 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | CO2 | –154.5 | 0.2 | 96.1 |
| 2.B.10 | Chemical Industry – Other (please specify) | CO2 | 152.3 | 0.2 | 96.2 |
| 1.B.2.b.2 | Natural Gas – Production | CH4 | 143.5 | 0.1 | 96.4 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Solid Fuels | CO2 | 142.2 | 0.1 | 96.5 |
| 1.A.3.b | Transport – Road Transportation Gaseous Fuels | CO2 | 140.8 | 0.1 | 96.7 |
| 3.B.1.2 | CH4 Emissions – Sheep | CH4 | 136.0 | 0.1 | 96.8 |
| 4.A.2 | Forest Land – Land Converted to Forest Land | N2O | 124.0 | 0.1 | 96.9 |
| 1.A.2.a | Manufacturing Industries and Construction – Iron and Steel Gaseous Fuels | CO2 | 116.6 | 0.1 | 97.1 |
| 1.B.2.c.2.iii | Flaring – Combined | CO2 | 114.4 | 0.1 | 97.2 |
| 4.B.2 | Cropland – Land Converted to Cropland | CO2 | 110.7 | 0.1 | 97.3 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Solid Fuels | CO2 | 109.5 | 0.1 | 97.4 |
| 1.B.2.c.1.ii | Venting – Gas | CO2 | 109.3 | 0.1 | 97.5 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Gaseous Fuels | CO2 | 106.2 | 0.1 | 97.6 |
| 2.G.3 | Other Product Manufacture and Use – N2O from Product Uses | N2O | 102.4 | 0.1 | 97.7 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | N2O | 89.3 | 0.1 | 97.8 |
| 2.A.2 | Mineral Industry – Lime Production | CO2 | 82.6 | 0.1 | 97.9 |
| 5.D | Waste – Wastewater Treatment and Discharge | N2O | 82.4 | 0.1 | 98.0 |
| 1.A.3.c | Transport – Railways Liquid Fuels | CO2 | 78.4 | 0.1 | 98.0 |
| 3.B.1.1 | Option A – Non-dairy Cattle | CH4 | 76.5 | 0.1 | 98.1 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | CH4 | 73.3 | 0.1 | 98.2 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CH4 | 72.9 | 0.1 | 98.3 |
| 4.E.1 | Settlements – Settlements Remaining Settlements | CO2 | 65.1 | 0.1 | 98.3 |
| 1.B.2.c.2.iii | Flaring – Combined | CH4 | 64.6 | 0.1 | 98.4 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Gaseous Fuels | CO2 | 64.3 | 0.1 | 98.5 |
| 1.A.2.g.vi | Other (please specify) – Textile and Leather Gaseous Fuels | CO2 | 59.2 | 0.1 | 98.5 |
| 3.B.1.3 | CH4 Emissions – Swine | CH4 | 58.6 | 0.1 | 98.6 |
| 1.B.2.d | Other (please specify) – Geothermal | CH4 | 54.8 | 0.1 | 98.6 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Liquid Fuels | CO2 | 51.5 | 0.1 | 98.7 |
| 1.A.4.b | Other Sectors – Residential Biomass | CH4 | 48.4 | 0.0 | 98.7 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Liquid Fuels | CO2 | 48.1 | 0.0 | 98.8 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Liquid Fuels | CO2 | 47.7 | 0.0 | 98.8 |
| 1.A.3.a | Domestic Aviation – Aviation Gasoline | CO2 | 47.7 | 0.0 | 98.9 |
| 3.A.4 | Other Livestock – Horses | CH4 | 42.3 | 0.0 | 98.9 |
| 1.A.2.g.i | Other (please specify) – Manufacturing of Machinery Gaseous Fuels | CO2 | 41.9 | 0.0 | 99.0 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | N2O | 41.8 | 0.0 | 99.0 |

Note:	Removals from the LULUCF sector are shown as negatives in this table. The absolute values for those removals were used for the calculations.

Table A1.3.2(b)	Results of the level analysis for 99 per cent of the gross emissions 
for New Zealand in 1990 included for reference only

| IPCC Tier 1 category level assessment – gross emissions (excluding LULUCF): 1990 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CRF category code | IPCC category | Gas | 1990 estimate (kt CO2-e) | Level assessment (%) | Cumulative total (%) |
| 3.A.2 | Other (please specify) – Sheep | CH4 | 14,085.7 | 21.4 | 21.4 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CO2 | 7,164.6 | 10.9 | 32.4 |
| 3.A.1 | Option A – Dairy Cattle | CH4 | 5,928.0 | 9.0 | 41.4 |
| 3.A.1 | Option A – Non-dairy Cattle | CH4 | 5,723.4 | 8.7 | 50.1 |
| 3.D.1.3 | Direct N2O Emissions from Managed Soils – Urine and Dung Deposited by Grazing Animals | N2O | 5,138.4 | 7.8 | 57.9 |
| 5.A | Waste – Solid Waste Disposal | CH4 | 3,711.3 | 5.7 | 63.6 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Gaseous Fuels | CO2 | 3,011.8 | 4.6 | 68.2 |
| 1.A.1.c | Energy Industries – Manufacture of Solid Fuels and Other Energy Industries Gaseous Fuels | CO2 | 1,720.1 | 2.6 | 70.8 |
| 2.C.1 | Metal Industry – Iron and Steel Production | CO2 | 1,306.7 | 2.0 | 72.8 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Liquid Fuels | CO2 | 1,072.3 | 1.6 | 74.4 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Solid Fuels | CO2 | 938.6 | 1.4 | 75.8 |
| 2.C.3 | Metal Industry – Aluminium Production | PFCs | 909.9 | 1.4 | 77.2 |
| 1.A.3.a | Domestic Aviation – Jet Kerosene | CO2 | 892.6 | 1.4 | 78.6 |
| 1.A.1.b | Energy Industries – Petroleum Refining Liquid Fuels | CO2 | 778.9 | 1.2 | 79.8 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Solid Fuels | CO2 | 731.1 | 1.1 | 80.9 |
| 3.D.2.1 | Indirect N2O Emissions from Managed Soils – Atmospheric Deposition | N2O | 700.9 | 1.1 | 81.9 |
| 1.A.2.c | Manufacturing Industries and Construction – Chemicals Gaseous Fuels | CO2 | 528.7 | 0.8 | 82.8 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Liquid Fuels | CO2 | 500.6 | 0.8 | 83.5 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Solid Fuels | CO2 | 474.8 | 0.7 | 84.2 |
| 2.C.3 | Metal Industry – Aluminium Production | CO2 | 449.0 | 0.7 | 84.9 |
| 2.A.1 | Mineral Industry – Cement Production | CO2 | 448.7 | 0.7 | 85.6 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Gaseous Fuels | CO2 | 445.1 | 0.7 | 86.3 |
| 3.A.4 | Other Livestock – Deer | CH4 | 434.1 | 0.7 | 86.9 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Solid Fuels | CO2 | 382.9 | 0.6 | 87.5 |
| 3.D.2.2 | Indirect N2O Emissions from Managed Soils – Nitrogen Leaching and Run-off | N2O | 377.2 | 0.6 | 88.1 |
| 3.B.1.1 | Option A – Dairy Cattle | CH4 | 376.4 | 0.6 | 88.7 |
| 3.G | Agriculture – Liming | CO2 | 360.1 | 0.5 | 89.2 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Gaseous Fuels | CO2 | 348.9 | 0.5 | 89.8 |
| 1.A.4.b | Other Sectors – Residential Solid Fuels | CO2 | 344.9 | 0.5 | 90.3 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Liquid Fuels | CO2 | 331.5 | 0.5 | 90.8 |
| 1.B.1.a.1 | Coal Mining and Handling – Underground Mines | CH4 | 289.6 | 0.4 | 91.2 |
| 1.B.2.b.5 | Natural Gas – Distribution | CH4 | 277.5 | 0.4 | 91.6 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Liquid Fuels | CO2 | 265.1 | 0.4 | 92.1 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Gaseous Fuels | CO2 | 236.1 | 0.4 | 92.4 |
| 1.A.3.d | Domestic Navigation – Residual Fuel Oil | CO2 | 232.9 | 0.4 | 92.8 |
| 3.D.1.1 | Direct N2O Emissions from Managed Soils – Inorganic N Fertilisers | N2O | 230.3 | 0.4 | 93.1 |
| 1.B.2.d | Other (please specify) – Geothermal | CO2 | 228.6 | 0.3 | 93.5 |
| 5.D | Waste – Wastewater Treatment and Discharge | CH4 | 218.8 | 0.3 | 93.8 |
| 3.A.4 | Other Livestock – Goats | CH4 | 196.6 | 0.3 | 94.1 |
| 1.A.4.b | Other Sectors – Residential Gaseous Fuels | CO2 | 185.6 | 0.3 | 94.4 |
| 3.D.1.4 | Direct N2O Emissions from Managed Soils – Crop Residues | N2O | 175.5 | 0.3 | 94.6 |
| 1.A.4.b | Other Sectors – Residential Liquid Fuels | CO2 | 167.4 | 0.3 | 94.9 |
| 2.B.10 | Chemical Industry – Other (please specify) | CO2 | 152.3 | 0.2 | 95.1 |
| 1.B.2.b.2 | Natural Gas – Production | CH4 | 143.5 | 0.2 | 95.4 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Solid Fuels | CO2 | 142.2 | 0.2 | 95.6 |
| 1.A.3.b | Transport – Road Transportation Gaseous Fuels | CO2 | 140.8 | 0.2 | 95.8 |
| 3.B.1.2 | CH4 Emissions – Sheep | CH4 | 136.0 | 0.2 | 96.0 |
| 1.A.2.a | Manufacturing Industries and Construction – Iron and Steel Gaseous Fuels | CO2 | 116.6 | 0.2 | 96.2 |
| 1.B.2.c.2.iii | Flaring – Combined | CO2 | 114.4 | 0.2 | 96.3 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Solid Fuels | CO2 | 109.5 | 0.2 | 96.5 |
| 1.B.2.c.1.ii | Venting – Gas | CO2 | 109.3 | 0.2 | 96.7 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Gaseous Fuels | CO2 | 106.2 | 0.2 | 96.8 |
| 2.G.3 | Other Product Manufacture and Use – N2O from Product Uses | N2O | 102.4 | 0.2 | 97.0 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | N2O | 89.3 | 0.1 | 97.1 |
| 2.A.2 | Mineral Industry – Lime Production | CO2 | 82.6 | 0.1 | 97.3 |
| 5.D | Waste – Wastewater Treatment and Discharge | N2O | 82.4 | 0.1 | 97.4 |
| 1.A.3.c | Transport – Railways Liquid Fuels | CO2 | 78.4 | 0.1 | 97.5 |
| 3.B.1.1 | Option A – Non-dairy Cattle | CH4 | 76.5 | 0.1 | 97.6 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CH4 | 72.9 | 0.1 | 97.7 |
| 1.B.2.c.2.iii | Flaring – Combined | CH4 | 64.6 | 0.1 | 97.8 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Gaseous Fuels | CO2 | 64.3 | 0.1 | 97.9 |
| 1.A.2.g.vi | Other (please specify) – Textile and Leather Gaseous Fuels | CO2 | 59.2 | 0.1 | 98.0 |
| 3.B.1.3 | CH4 Emissions – Swine | CH4 | 58.6 | 0.1 | 98.1 |
| 1.B.2.d | Other (please specify) – Geothermal | CH4 | 54.8 | 0.1 | 98.2 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Liquid Fuels | CO2 | 51.5 | 0.1 | 98.3 |
| 1.A.4.b | Other Sectors – Residential Biomass | CH4 | 48.4 | 0.1 | 98.3 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Liquid Fuels | CO2 | 48.1 | 0.1 | 98.4 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Liquid Fuels | CO2 | 47.7 | 0.1 | 98.5 |
| 1.A.3.a | Domestic Aviation – Aviation Gasoline | CO2 | 47.7 | 0.1 | 98.6 |
| 3.A.4 | Other Livestock – Horses | CH4 | 42.3 | 0.1 | 98.6 |
| 1.A.2.g.i | Other (please specify) – Manufacturing of Machinery Gaseous Fuels | CO2 | 41.9 | 0.1 | 98.7 |
| 3.H | Agriculture – Urea Application | CO2 | 39.2 | 0.1 | 98.7 |
| 1.B.1.a.2 | Coal Mining and Handling – Surface Mines | CH4 | 38.7 | 0.1 | 98.8 |
| 3.D.1.2 | Direct N2O Emissions from Managed Soils – Organic N Fertilisers | N2O | 36.8 | 0.1 | 98.9 |
| 3.B.2.5 | N2O and NMVOC Emissions – Indirect N2O Emissions | N2O | 35.9 | 0.1 | 98.9 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Solid Fuels | CO2 | 35.1 | 0.1 | 99.0 |
| 1.A.2.b | Manufacturing Industries and Construction – Non-Ferrous Metals Liquid Fuels | CO2 | 30.8 | 0.0 | 99.0 |

Table A1.3.3(a)	Results of the key category trend analysis for 99 per cent of the net emissions
and removals for New Zealand in 1990–2017

| IPCC Tier 1 category trend assessment – including LULUCF (net emissions) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CRF category code | IPCC category | Gas | 1990 estimate (kt CO2-e) | 2017 estimate (kt CO2-e) | Trend assessment | Contribution to trend (%) | Cumulative total (%) |
| 3.A.2 | Other (please specify) – Sheep | CH4 | 14,085.7 | 8,253.1 | 0.061 | 13.4 | 13.4 |
| 3.A.1 | Option A – Dairy Cattle | CH4 | 5,928.0 | 13,560.1 | 0.052 | 11.4 | 24.8 |
| 4.A.1 | Forest Land – Forest Land Remaining Forest Land | CO2 | –10,614.5 | –5,587.1 | 0.051 | 11.1 | 35.9 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CO2 | 7,164.6 | 14,335.0 | 0.047 | 10.4 | 46.3 |
| 4.A.2 | Forest Land – Land Converted to Forest Land | CO2 | –19,433.5 | –16,252.4 | 0.046 | 10.2 | 56.5 |
| 4.G | Land Use, Land-Use Change and Forestry – Harvested Wood Products | CO2 | –2,072.9 | –6,518.5 | 0.032 | 7.0 | 63.5 |
| 4.C.2 | Grassland – Land Converted to Grassland | CO2 | 262.9 | 2,437.1 | 0.016 | 3.6 | 67.1 |
| 1.A.1.c | Energy Industries – Manufacture of Solid Fuels and Other Energy Industries Gaseous Fuels | CO2 | 1,720.1 | 305.9 | 0.013 | 2.8 | 69.9 |
| 2.F.1 | Product Uses as Substitutes for ODS – Refrigeration and Air Conditioning | HFCs | 0.0 | 1,397.3 | 0.011 | 2.4 | 72.3 |
| 3.A.1 | Option A – Non-dairy Cattle | CH4 | 5,723.4 | 5,298.0 | 0.010 | 2.1 | 74.4 |
| 3.D.1.1 | Direct N2O Emissions from Managed Soils – Inorganic N Fertilisers | N2O | 230.3 | 1,365.6 | 0.008 | 1.9 | 76.3 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | CO2 | –154.5 | 1,215.1 | 0.008 | 1.8 | 78.0 |
| 2.C.3 | Metal Industry – Aluminium Production | PFCs | 909.9 | 60.5 | 0.008 | 1.7 | 79.7 |
| 1.A.2.c | Manufacturing Industries and Construction – Chemicals Gaseous Fuels | CO2 | 528.7 | 1,587.8 | 0.008 | 1.7 | 81.4 |
| 3.B.1.1 | Option A – Dairy Cattle | CH4 | 376.4 | 1,255.8 | 0.006 | 1.4 | 82.7 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Solid Fuels | CO2 | 731.1 | 118.2 | 0.006 | 1.2 | 84.0 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Solid Fuels | CO2 | 938.6 | 1,784.0 | 0.005 | 1.2 | 85.2 |
| 3.H | Agriculture – Urea Application | CO2 | 39.2 | 588.3 | 0.004 | 0.9 | 86.1 |
| 5.A | Waste – Solid Waste Disposal | CH4 | 3,711.3 | 3,724.5 | 0.004 | 0.9 | 87.0 |
| 3.D.1.3 | Direct N2O Emissions from Managed Soils – Urine and Dung Deposited by Grazing Animals | N2O | 5,138.4 | 5,435.3 | 0.004 | 0.8 | 87.7 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Gaseous Fuels | CO2 | 445.1 | 954.9 | 0.003 | 0.8 | 88.5 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Gaseous Fuels | CO2 | 3,011.8 | 3,065.2 | 0.003 | 0.7 | 89.2 |
| 1.B.2.d | Other (please specify) – Geothermal | CO2 | 228.6 | 643.3 | 0.003 | 0.6 | 89.8 |
| 1.A.4.b | Other Sectors – Residential Solid Fuels | CO2 | 344.9 | 25.4 | 0.003 | 0.6 | 90.4 |
| 1.B.1.a.1 | Coal Mining and Handling – Underground Mines | CH4 | 289.6 | 71.9 | 0.002 | 0.4 | 90.9 |
| 2.C.1 | Metal Industry – Iron and Steel Production | CO2 | 1,306.7 | 1,758.3 | 0.002 | 0.4 | 91.3 |
| 1.A.1.b | Energy Industries – Petroleum Refining Liquid Fuels | CO2 | 778.9 | 674.5 | 0.002 | 0.4 | 91.7 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Solid Fuels | CO2 | 35.1 | 250.6 | 0.002 | 0.4 | 92.0 |
| 3.A.4 | Other Livestock – Goats | CH4 | 196.6 | 20.6 | 0.002 | 0.3 | 92.4 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Liquid Fuels | CO2 | 331.5 | 584.4 | 0.002 | 0.3 | 92.7 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Solid Fuels | CO2 | 382.9 | 244.8 | 0.001 | 0.3 | 93.1 |
| 1.A.1.b | Energy Industries – Petroleum Refining Gaseous Fuels | CO2 | 0.0 | 169.9 | 0.001 | 0.3 | 93.3 |
| 1.B.2.c.2.iii | Flaring – Combined | CO2 | 114.4 | 292.7 | 0.001 | 0.3 | 93.6 |
| 1.A.3.b | Transport – Road Transportation Gaseous Fuels | CO2 | 140.8 | 0.5 | 0.001 | 0.3 | 93.9 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Gaseous Fuels | CO2 | 236.1 | 429.8 | 0.001 | 0.3 | 94.2 |
| 1.A.4.b | Other Sectors – Residential Gaseous Fuels | CO2 | 185.6 | 365.1 | 0.001 | 0.3 | 94.4 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Liquid Fuels | CO2 | 51.5 | 205.8 | 0.001 | 0.2 | 94.7 |
| 1.B.2.c.1.ii | Venting – Gas | CO2 | 109.3 | 258.5 | 0.001 | 0.2 | 94.9 |
| 1.B.2.b.5 | Natural Gas – Distribution | CH4 | 277.5 | 208.4 | 0.001 | 0.2 | 95.1 |
| 1.B.2.d | Other (please specify) – Geothermal | CH4 | 54.8 | 171.5 | 0.001 | 0.2 | 95.3 |
| 3.D.2.1 | Indirect N2O Emissions from Managed Soils – Atmospheric Deposition | N2O | 700.9 | 906.8 | 0.001 | 0.2 | 95.4 |
| 2.F.4 | Product Uses as Substitutes for ODS – Aerosols | HFCs | 0.0 | 96.5 | 0.001 | 0.2 | 95.6 |
| 4.A.2 | Forest Land – Land Converted to Forest Land | N2O | 124.0 | 57.3 | 0.001 | 0.1 | 95.7 |
| 2.B.8 | Chemical Industry – Petrochemical and Carbon Black Production | CH4 | 27.6 | 112.0 | 0.001 | 0.1 | 95.9 |
| 4.B.1 | Cropland – Cropland Remaining Cropland | CO2 | 358.7 | 332.0 | 0.001 | 0.1 | 96.0 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Solid Fuels | CO2 | 109.5 | 53.0 | 0.001 | 0.1 | 96.1 |
| 4.B.2 | Cropland – Land Converted to Cropland | CO2 | 110.7 | 55.4 | 0.001 | 0.1 | 96.2 |
| 2.A.1 | Mineral Industry – Cement Production | CO2 | 448.7 | 443.8 | 0.001 | 0.1 | 96.4 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Solid Fuels | CO2 | 142.2 | 92.4 | 0.001 | 0.1 | 96.5 |
| 3.B.1.2 | CH4 Emissions – Sheep | CH4 | 136.0 | 85.5 | 0.001 | 0.1 | 96.6 |
| 3.D.2.2 | Indirect N2O Emissions from Managed Soils – Nitrogen Leaching and Run-off | N2O | 377.2 | 500.5 | 0.001 | 0.1 | 96.7 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Liquid Fuels | CO2 | 1,072.3 | 1,163.8 | 0.001 | 0.1 | 96.8 |
| 3.B.2.5 | N2O and NMVOC Emissions – Indirect N2O Emissions | N2O | 35.9 | 106.1 | 0.000 | 0.1 | 96.9 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CH4 | 72.9 | 21.4 | 0.000 | 0.1 | 97.0 |
| 1.A.3.a | Domestic Aviation – Jet Kerosene | CO2 | 892.6 | 961.7 | 0.000 | 0.1 | 97.2 |
| 2.G.3 | Other Product Manufacture and Use – N2O from Product Uses | N2O | 102.4 | 61.8 | 0.000 | 0.1 | 97.2 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Liquid Fuels | CO2 | 48.1 | 109.6 | 0.000 | 0.1 | 97.3 |
| 1.A.3.d | Domestic Navigation – Residual Fuel Oil | CO2 | 232.9 | 319.7 | 0.000 | 0.1 | 97.4 |
| 2.A.4 | Mineral Industry – Other Process Uses of Carbonates | CO2 | 30.5 | 87.0 | 0.000 | 0.1 | 97.5 |
| 3.D.1.4 | Direct N2O Emissions from Managed Soils – Crop Residues | N2O | 175.5 | 249.4 | 0.000 | 0.1 | 97.6 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Liquid Fuels | CO2 | 265.1 | 351.8 | 0.000 | 0.1 | 97.7 |
| 3.G | Agriculture – Liming | CO2 | 360.1 | 459.6 | 0.000 | 0.1 | 97.8 |
| 2.A.2 | Mineral Industry – Lime Production | CO2 | 82.6 | 138.9 | 0.000 | 0.1 | 97.8 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Gaseous Fuels | CO2 | 106.2 | 78.4 | 0.000 | 0.1 | 97.9 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Gaseous Fuels | CO2 | 64.3 | 117.1 | 0.000 | 0.1 | 98.0 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Liquid Fuels | CO2 | 500.6 | 530.8 | 0.000 | 0.1 | 98.1 |
| 1.A.2.g.vi | Other (please specify) – Textile and Leather Gaseous Fuels | CO2 | 59.2 | 25.8 | 0.000 | 0.1 | 98.1 |
| 3.D.1.2 | Direct N2O Emissions from Managed Soils – Organic N Fertilisers | N2O | 36.8 | 78.5 | 0.000 | 0.1 | 98.2 |
| 1.A.2.g.i | Other (please specify) – Manufacturing of Machinery Gaseous Fuels | CO2 | 41.9 | 14.0 | 0.000 | 0.1 | 98.2 |
| 1.A.2.a | Manufacturing Industries and Construction – Iron and Steel Gaseous Fuels | CO2 | 116.6 | 102.0 | 0.000 | 0.1 | 98.3 |
| 1.A.4.b | Other Sectors – Residential Liquid Fuels | CO2 | 167.4 | 223.4 | 0.000 | 0.1 | 98.3 |
| 2.C.3 | Metal Industry – Aluminium Production | CO2 | 449.0 | 546.2 | 0.000 | 0.1 | 98.4 |
| 1.B.2.b.2 | Natural Gas – Production | CH4 | 143.5 | 133.2 | 0.000 | 0.1 | 98.5 |
| 4.F.2 | Other Land – Land Converted to Other Land | CO2 | 13.5 | 45.6 | 0.000 | 0.1 | 98.5 |
| 1.A.4.b | Other Sectors – Residential Solid Fuels | CH4 | 27.3 | 2.0 | 0.000 | 0.0 | 98.6 |
| 3.A.4 | Other Livestock – Horses | CH4 | 42.3 | 19.6 | 0.000 | 0.0 | 98.6 |
| 1.A.3.e | Transport – Other Transportation (please specify) Gaseous Fuels | CO2 | 5.5 | 34.9 | 0.000 | 0.0 | 98.7 |
| 1.A.3.a | Domestic Aviation – Aviation Gasoline | CO2 | 47.7 | 26.7 | 0.000 | 0.0 | 98.7 |
| 1.B.2.c.1.iii | Venting – Combined | CH4 | 1.4 | 29.2 | 0.000 | 0.0 | 98.7 |
| 3.B.1.3 | CH4 Emissions – Swine | CH4 | 58.6 | 39.7 | 0.000 | 0.0 | 98.8 |
| 4.E.2 | Settlements – Land Converted to Settlements | CO2 | 7.6 | 35.6 | 0.000 | 0.0 | 98.8 |
| 4.C.1 | Grassland – Grassland Remaining Grassland | N2O | 41.8 | 23.1 | 0.000 | 0.0 | 98.9 |
| 1.A.2.g.vi | Other (please specify) – Textile and Leather Solid Fuels | CO2 | 23.0 | 1.6 | 0.000 | 0.0 | 98.9 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Gaseous Fuels | CO2 | 10.7 | 35.3 | 0.000 | 0.0 | 99.0 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Solid Fuels | CO2 | 19.9 | 0.0 | 0.000 | 0.0 | 99.0 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Solid Fuels | CO2 | 474.8 | 522.7 | 0.000 | 0.0 | 99.0 |

Note:	Key categories are those that comprise 95 per cent of the total. Removals from the LULUCF sector are shown as negatives in this table. The absolute values for those removals were used for the calculations.

Table A1.3.3(b)	Results of the key category trend analysis for 99 per cent of the gross emissions
for New Zealand in 1990–2017

| IPCC Tier 1 category trend assessment – gross emissions (excluding LULUCF) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CRF Category code | IPCC Category | Gas | 1990 estimate (kt CO2-e) | 2017 estimate (kt CO2-e) | Trend assessment | Contribution to trend (%) | Cumulative total (%) |
| 3.A.2 | Other (please specify) – Sheep | CH4 | 14,085.7 | 8,253.1 | 0.091 | 22.3 | 22.3 |
| 3.A.1 | Option A – Dairy Cattle | CH4 | 5,928.0 | 13,560.1 | 0.063 | 15.4 | 37.7 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CO2 | 7,164.6 | 14,335.0 | 0.055 | 13.6 | 51.3 |
| 1.A.1.c | Energy Industries – Manufacture of Solid Fuels and Other Energy Industries Gaseous Fuels | CO2 | 1,720.1 | 305.9 | 0.018 | 4.5 | 55.8 |
| 3.A.1 | Option A – Non-dairy Cattle | CH4 | 5,723.4 | 5,298.0 | 0.018 | 4.3 | 60.1 |
| 2.F.1 | Product Uses as Substitutes for ODS – Refrigeration and Air Conditioning | HFCs | 0.0 | 1,397.3 | 0.014 | 3.4 | 63.5 |
| 3.D.1.1 | Direct N2O Emissions from Managed Soils – Inorganic N Fertilisers | N2O | 230.3 | 1,365.6 | 0.011 | 2.7 | 66.2 |
| 2.C.3 | Metal Industry – Aluminium Production | PFCs | 909.9 | 60.5 | 0.011 | 2.6 | 68.8 |
| 1.A.2.c | Manufacturing Industries and Construction – Chemicals Gaseous Fuels | CO2 | 528.7 | 1,587.8 | 0.009 | 2.3 | 71.1 |
| 3.D.1.3 | Direct N2O Emissions From Managed Soils – Urine and Dung Deposited by Grazing Animals | N2O | 5,138.4 | 5,435.3 | 0.009 | 2.2 | 73.3 |
| 5.A | Waste – Solid Waste Disposal | CH4 | 3,711.3 | 3,724.5 | 0.008 | 2.1 | 75.3 |
| 3.B.1.1 | Option A – Dairy Cattle | CH4 | 376.4 | 1,255.8 | 0.008 | 1.9 | 77.3 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Solid Fuels | CO2 | 731.1 | 118.2 | 0.008 | 1.9 | 79.2 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Gaseous Fuels | CO2 | 3,011.8 | 3,065.2 | 0.006 | 1.6 | 80.8 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Solid Fuels | CO2 | 938.6 | 1,784.0 | 0.006 | 1.5 | 82.3 |
| 3.H | Agriculture – Urea Application | CO2 | 39.2 | 588.3 | 0.005 | 1.3 | 83.7 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Gaseous Fuels | CO2 | 445.1 | 954.9 | 0.004 | 1.0 | 84.7 |
| 1.A.4.b | Other Sectors – Residential Solid Fuels | CO2 | 344.9 | 25.4 | 0.004 | 1.0 | 85.6 |
| 1.B.2.d | Other (please specify) – Geothermal | CO2 | 228.6 | 643.3 | 0.004 | 0.9 | 86.5 |
| 1.B.1.a.1 | Coal Mining and Handling – Underground Mines | CH4 | 289.6 | 71.9 | 0.003 | 0.7 | 87.2 |
| 1.A.1.b | Energy Industries – Petroleum Refining Liquid Fuels | CO2 | 778.9 | 674.5 | 0.003 | 0.7 | 87.9 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Solid Fuels | CO2 | 382.9 | 244.8 | 0.002 | 0.6 | 88.5 |
| 3.A.4 | Other Livestock – Goats | CH4 | 196.6 | 20.6 | 0.002 | 0.5 | 89.0 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Solid Fuels | CO2 | 35.1 | 250.6 | 0.002 | 0.5 | 89.5 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Liquid Fuels | CO2 | 331.5 | 584.4 | 0.002 | 0.4 | 90.0 |
| 1.A.3.b | Transport – Road Transportation Gaseous Fuels | CO2 | 140.8 | 0.5 | 0.002 | 0.4 | 90.4 |
| 1.A.1.b | Energy Industries – Petroleum Refining Gaseous Fuels | CO2 | 0.0 | 169.9 | 0.002 | 0.4 | 90.8 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Liquid Fuels | CO2 | 1,072.3 | 1,163.8 | 0.002 | 0.4 | 91.2 |
| 1.B.2.c.2.iii | Flaring – Combined | CO2 | 114.4 | 292.7 | 0.002 | 0.4 | 91.6 |
| 2.C.1 | Metal Industry – Iron and Steel Production | CO2 | 1,306.7 | 1,758.3 | 0.002 | 0.4 | 91.9 |
| 1.A.2.g.viii | Other (please specify) – Other (please specify) Liquid Fuels | CO2 | 51.5 | 205.8 | 0.001 | 0.3 | 92.3 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Gaseous Fuels | CO2 | 236.1 | 429.8 | 0.001 | 0.3 | 92.6 |
| 1.A.3.a | Domestic Aviation – Jet Kerosene | CO2 | 892.6 | 961.7 | 0.001 | 0.3 | 93.0 |
| 1.A.4.b | Other Sectors – Residential Gaseous Fuels | CO2 | 185.6 | 365.1 | 0.001 | 0.3 | 93.3 |
| 1.B.2.b.5 | Natural Gas – Distribution | CH4 | 277.5 | 208.4 | 0.001 | 0.3 | 93.6 |
| 1.B.2.c.1.ii | Venting – Gas | CO2 | 109.3 | 258.5 | 0.001 | 0.3 | 93.9 |
| 2.A.1 | Mineral Industry – Cement Production | CO2 | 448.7 | 443.8 | 0.001 | 0.3 | 94.2 |
| 1.B.2.d | Other (please specify) – Geothermal | CH4 | 54.8 | 171.5 | 0.001 | 0.3 | 94.5 |
| 2.F.4 | Product Uses as Substitutes for ODS – Aerosols | HFCs | 0.0 | 96.5 | 0.001 | 0.2 | 94.7 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Liquid Fuels | CO2 | 500.6 | 530.8 | 0.001 | 0.2 | 94.9 |
| 1.A.4.a | Other Sectors – Commercial/Institutional Solid Fuels | CO2 | 142.2 | 92.4 | 0.001 | 0.2 | 95.1 |
| 3.B.1.2 | CH4 Emissions – Sheep | CH4 | 136.0 | 85.5 | 0.001 | 0.2 | 95.3 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Solid Fuels | CO2 | 109.5 | 53.0 | 0.001 | 0.2 | 95.5 |
| 2.B.8 | Chemical Industry – Petrochemical and Carbon Black Production | CH4 | 27.6 | 112.0 | 0.001 | 0.2 | 95.7 |
| 1.A.3.b | Transport – Road Transportation Liquid Fuels | CH4 | 72.9 | 21.4 | 0.001 | 0.2 | 95.9 |
| 2.G.3 | Other Product Manufacture and Use – N2O from Product Uses | N2O | 102.4 | 61.8 | 0.001 | 0.2 | 96.0 |
| 3.B.2.5 | N2O and NMVOC Emissions – Indirect N2O Emissions | N2O | 35.9 | 106.1 | 0.001 | 0.2 | 96.2 |
| 1.A.1.a | Energy Industries – Public Electricity and Heat Production Solid Fuels | CO2 | 474.8 | 522.7 | 0.001 | 0.2 | 96.3 |
| 1.A.4.c | Other Sectors – Agriculture/Forestry/Fishing Gaseous Fuels | CO2 | 106.2 | 78.4 | 0.001 | 0.1 | 96.5 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Liquid Fuels | CO2 | 48.1 | 109.6 | 0.001 | 0.1 | 96.6 |
| 1.A.2.d | Manufacturing Industries and Construction – Pulp, Paper and Print Gaseous Fuels | CO2 | 348.9 | 379.6 | 0.001 | 0.1 | 96.7 |
| 2.A.4 | Mineral Industry – Other Process Uses of Carbonates | CO2 | 30.5 | 87.0 | 0.000 | 0.1 | 96.8 |
| 3.A.4 | Other Livestock – Deer | CH4 | 434.1 | 486.0 | 0.000 | 0.1 | 97.0 |
| 1.A.2.g.vi | Other (please specify) – Textile and Leather Gaseous Fuels | CO2 | 59.2 | 25.8 | 0.000 | 0.1 | 97.1 |
| 3.D.2.1 | Indirect N2O Emissions from Managed Soils – Atmospheric Deposition | N2O | 700.9 | 906.8 | 0.000 | 0.1 | 97.2 |
| 1.B.2.b.2 | Natural Gas – Production | CH4 | 143.5 | 133.2 | 0.000 | 0.1 | 97.3 |
| 1.A.2.a | Manufacturing Industries and Construction – Iron and Steel Gaseous Fuels | CO2 | 116.6 | 102.0 | 0.000 | 0.1 | 97.4 |
| 1.A.2.f | Manufacturing Industries and Construction – Non-metallic Minerals Gaseous Fuels | CO2 | 64.3 | 117.1 | 0.000 | 0.1 | 97.5 |
| 1.A.2.g.i | Other (please specify) – Manufacturing of Machinery Gaseous Fuels | CO2 | 41.9 | 14.0 | 0.000 | 0.1 | 97.6 |
| 2.A.2 | Mineral Industry – Lime Production | CO2 | 82.6 | 138.9 | 0.000 | 0.1 | 97.7 |
| 3.D.2.2 | Indirect N2O Emissions from Managed Soils – Nitrogen Leaching and Run-off | N2O | 377.2 | 500.5 | 0.000 | 0.1 | 97.7 |
| 3.D.1.4 | Direct N2O Emissions from Managed Soils – Crop Residues | N2O | 175.5 | 249.4 | 0.000 | 0.1 | 97.8 |
| 3.D.1.2 | Direct N2O Emissions from Managed Soils – Organic N Fertilisers | N2O | 36.8 | 78.5 | 0.000 | 0.1 | 97.9 |
| 1.A.3.d | Domestic Navigation – Residual Fuel Oil | CO2 | 232.9 | 319.7 | 0.000 | 0.1 | 98.0 |
| 3.A.4 | Other Livestock – Horses | CH4 | 42.3 | 19.6 | 0.000 | 0.1 | 98.1 |
| 3.B.1.3 | CH4 Emissions – Swine | CH4 | 58.6 | 39.7 | 0.000 | 0.1 | 98.2 |
| 1.A.3.a | Domestic Aviation – Aviation Gasoline | CO2 | 47.7 | 26.7 | 0.000 | 0.1 | 98.2 |
| 1.A.4.b | Other Sectors – Residential Solid Fuels | CH4 | 27.3 | 2.0 | 0.000 | 0.1 | 98.3 |
| 1.A.3.e | Transport – Other Transportation (please specify) Gaseous Fuels | CO2 | 5.5 | 34.9 | 0.000 | 0.1 | 98.4 |
| 1.B.2.c.1.iii | Venting – Combined | CH4 | 1.4 | 29.2 | 0.000 | 0.1 | 98.4 |
| 1.A.2.g.vi | Other (please specify) – Textile and Leather Solid Fuels | CO2 | 23.0 | 1.6 | 0.000 | 0.1 | 98.5 |
| 1.A.2.e | Manufacturing Industries and Construction – Food Processing, Beverages and Tobacco Liquid Fuels | CO2 | 265.1 | 351.8 | 0.000 | 0.1 | 98.6 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Solid Fuels | CO2 | 19.9 | 0.0 | 0.000 | 0.1 | 98.6 |
| 3.B.1.1 | Option A – Non-dairy Cattle | CH4 | 76.5 | 71.2 | 0.000 | 0.1 | 98.7 |
| 5.D | Waste – Wastewater Treatment and Discharge | CH4 | 218.8 | 246.9 | 0.000 | 0.1 | 98.7 |
| 1.A.2.g.iii | Other (please specify) – Mining (excluding fuels) and Quarrying Gaseous Fuels | CO2 | 10.7 | 35.3 | 0.000 | 0.1 | 98.8 |
| 1.B.2.c.2.iii | Flaring – Combined | CH4 | 64.6 | 58.6 | 0.000 | 0.1 | 98.9 |
| 1.A.2.g.vi | Other (please specify) – Textile and Leather Liquid Fuels | CO2 | 20.1 | 5.5 | 0.000 | 0.0 | 98.9 |
| 1.A.2.b | Manufacturing Industries and Construction – Non-ferrous Metals Solid Fuels | CO2 | 0.0 | 18.4 | 0.000 | 0.0 | 98.9 |
| 5.C | Waste – Incineration and Open Burning of Waste | CO2 | 17.1 | 3.8 | 0.000 | 0.0 | 99.0 |
| 1.A.4.b | Other Sectors – Residential Liquid Fuels | CO2 | 167.4 | 223.4 | 0.000 | 0.0 | 99.0 |

Note:	Key categories are those that comprise 95 per cent of the total.

## Annex 1: References

IPCC. 2003. Penman J, Gytarsky M, Hiraishi T, Krug T, Kruger D, Pipatti R, Buendia L, Miwa K, Ngara T, Tanabe K, Wagner F (eds). Good Practice Guidance for Land-Use, Land-Use Change and Forestry. IPCC National Greenhouse Gas Inventories Programme. Japan: Published for the IPCC by the Institute for Global Environmental Strategies.

IPCC. 2006. Eggleston HS, Buendia L, Miwa K, Ngara T, Tanabe K (eds). 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 1. General Guidance and Reporting. IPCC National Greenhouse Gas Inventories Programme. Japan: Published for the IPCC by the Institute for Global Environmental Strategies.

# Annex 2: Uncertainty analysis

Uncertainty estimates are an essential element of a complete emissions inventory. The purpose of uncertainty information is to help prioritise efforts to improve the accuracy of inventories in the future and guide decisions on methodological choice (IPCC, 2006).

New Zealand has followed Approach 1 for uncertainty analysis, as required by the inventory reporting guidelines under United Nations Framework Convention on Climate Change (UNFCCC, 2013) and Intergovernmental Panel on Climate Change (IPCC) methodological guidelines (IPCC, 2006). Uncertainties in the categories are combined to provide uncertainty estimates for the entire inventory in any year and the uncertainty in the overall inventory trend over time. Land Use, Land-Use Change and Forestry (LULUCF) sector categories have been included using the absolute value of any removals of carbon dioxide (table A2.1.1). Table A2.1.2 calculates the uncertainty only in emissions, that is, excluding LULUCF removals.

## A2.1 	Approach 1 uncertainty calculation

The uncertainty in activity data and emission and/or removal factors shown in tables A2.1.1 and A2.1.2 are equal to half the 95 per cent confidence interval divided by the mean and expressed as a percentage. The reason for halving the 95 per cent confidence interval is that the value corresponds to the familiar plus or minus value when uncertainties are loosely quoted as ‘plus or minus x per cent’.

Where uncertainty is highly asymmetrical, the larger percentage difference between the mean and the confidence limit is entered. Where only the total uncertainty is known for a category, then:

- if uncertainty is correlated across years, the uncertainty is entered as the emission or the removal factor uncertainty and as zero in the activity data uncertainty

- if uncertainty is not correlated across years, the uncertainty is entered as the uncertainty in the activity data and as zero in the emission or the removal factor uncertainty.

In Approach 1, uncertainties in the trend are estimated using two sensitivities.

- Type A sensitivity is the change in the difference of total emissions between the base year and the current year, expressed as a percentage. Further, this change results from a 1 per cent increase in emissions of a given source category and a greenhouse gas in both the base year and the current year.

- Type B sensitivity is the change in the difference of total emissions between the base year and the current year, expressed as a percentage. Further, this change results from a 1 per cent increase in emissions of a given source category and gas in the current year only.

Uncertainties that are fully correlated between years are associated with Type A sensitivities, and uncertainties that are not correlated between years are associated with Type B sensitivities.

In tables A2.1.1 and A2.1.2, the figure labelled ‘Uncertainty in the trend’ is an estimate of the total uncertainty in the trend in emissions since the base year. This is expressed as the number of percentage points in the 95 per cent confidence interval in the per cent change in emissions since the base year. The total uncertainty in the trend is calculated by combining the contribution of emissions factor uncertainty and activity data uncertainty to the trend across all categories using equation 3.1 (IPCC, 2006).

The values for individual categories are an estimate of the uncertainty introduced into the trend by the category in question.

Table A2.1.1 	Uncertainty calculation (including LULUCF) for New Zealand’s Greenhouse Gas Inventory 1990–2017 (IPCC, 2006, Approach 1)

| IPCC source category | Gas | 1990 emissions or absolute value of removals (kt CO2-e) | 2017 emissions or absolute value of removals (kt CO2-e) | Activity data uncertainty (%) | Emission or removal factor uncertainty (%) | Combined uncertainty (%) | Combined uncertainty as a per cent of the national total in 2017 (%) | Type A sensitivity (%) | Type B sensitivity (%) | Uncertainty in the trend in national total introduced by emission or removal factor uncertainty (%) | Uncertainty in trend in national total introduced by activity data uncertainty (%) | Uncertainty introduced into the trend in the national total (%) | Emission/ removal factor quality indicator | Activity data quality indicator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Energy – Liquid Fuels | CO2 | 11,789.99 | 19,743.54 | 1.2 | 0.5 | 1.3 | 0.2 | 0.1 | 0.2 | 0.0 | 0.3 | 0.3 | R | M |
| Energy – Solid Fuels | CO2 | 3,211.03 | 3,112.86 | –4.0 | 2.2 | 4.6 | 0.1 | 0.0 | 0.0 | 0.0 | –0.2 | 0.2 | M | M |
| Energy – Gaseous Fuels | CO2 | 7,054.90 | 7,700.48 | 0.3 | 2.4 | 2.4 | 0.2 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | M | M |
| Energy – Fugitive – Geothermal | CO2 | 228.58 | 643.26 | 5.0 | 5.0 | 7.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | D |
| Energy – Fugitive – Venting/flaring | CO2 | 229.84 | 554.60 | 0.3 | 2.4 | 2.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| Energy – Fugitive – Oil and Gas Activities | CO2 | 0.21 | 0.20 | 5.0 | 100.0 | 100.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Energy – Fugitive – Transmission and Distribution | CO2 | 1.46 | 1.30 | 0.3 | 100.0 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| IPPU – Mineral Industry | CO2 | 561.87 | 669.67 | 2.0 | 7.0 | 7.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| IPPU – Chemical Industry | CO2 | 175.40 | 193.09 | 2.0 | 6.0 | 6.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| IPPU – Metal Industry | CO2 | 1,757.51 | 2,304.45 | 5.0 | 7.0 | 8.6 | 0.2 | 0.0 | 0.0 | 0.0 | 0.2 | 0.2 | D | D |
| IPPU – Non-energy Products from Fuels and Solvent Use | CO2 | 25.12 | 46.67 | 20.0 | 50.0 | 53.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Agriculture – Liming | CO2 | 360.06 | 459.60 | 3.4 | 50.0 | 50.1 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| Agriculture – Urea Application | CO2 | 39.19 | 588.26 | 10.0 | 50.0 | 51.0 | 0.3 | 0.0 | 0.0 | 0.3 | 0.1 | 0.3 | D | R |
| LULUCF – Forest Land | CO2 | 30,048.01 | 21,839.53 | 6.1 | 36.7 | 37.2 | 7.2 | –0.1 | 0.2 | –4.7 | 1.9 | 5.1 | M | M |
| LULUCF – Cropland | CO2 | 469.42 | 387.40 | 5.7 | 72.5 | 72.8 | 0.2 | 0.0 | 0.0 | –0.1 | 0.0 | 0.1 | M | M |
| LULUCF – Grassland | CO2 | 108.49 | 3,652.15 | 5.7 | 51.9 | 52.2 | 1.7 | 0.0 | 0.0 | 1.9 | 0.3 | 1.9 | M | M |
| LULUCF – Wetlands | CO2 | 9.96 | 13.89 | 43.1 | 1.2 | 43.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| LULUCF – Settlements | CO2 | 72.65 | 100.69 | 16.2 | 84.7 | 86.2 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| LULUCF – Other Land | CO2 | 13.50 | 45.64 | 22.0 | 65.2 | 68.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| LULUCF – Harvested Wood Products | CO2 | 2,072.91 | 6,518.52 | 15.0 | 67.4 | 69.0 | 4.0 | 0.0 | 0.1 | 2.8 | 1.4 | 3.1 | M | M |
| Waste – Incineration and Open Burning of Waste | CO2 | 17.12 | 3.75 | 50.0 | 40.0 | 64.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Energy Industries – Sectoral Approach – Liquid | CO2 | 1.15 | 0.20 | 10.0 | 7.0 | 12.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Gas Diesel Oil – Sectoral Approach – Liquid | CO2 | 1.60 | 1.60 | 50.0 | 1.5 | 50.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Other/Residential – Sectoral Approach – Liquid | CO2 | 0.08 | 0.09 | 20.0 | 7.0 | 21.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Waste – Incineration and Open Burning of Waste | CO2 | 0.05 | 0.04 | 50.0 | 40.0 | 64.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Energy – Liquid Fuels | CH4 | 82.70 | 29.52 | 1.2 | 50.0 | 50.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Solid Fuels | CH4 | 36.40 | 27.50 | –4.0 | 50.0 | 50.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Gaseous Fuels | CH4 | 9.05 | 4.55 | 0.3 | 50.0 | 50.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Biomass | CH4 | 66.60 | 84.14 | 50.0 | 50.0 | 70.7 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | D | D |
| Energy – Fugitive – Geothermal | CH4 | 54.79 | 171.50 | 5.0 | 5.0 | 7.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Energy – Fugitive – Venting/processing | CH4 | 66.06 | 87.81 | 0.3 | 50.0 | 50.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Fugitive – Coal Mining | CH4 | 328.23 | 132.05 | –4.0 | 50.0 | 50.2 | 0.1 | 0.0 | 0.0 | –0.1 | 0.0 | 0.1 | D | M |
| Energy – Fugitive – Transmission and Distribution | CH4 | 279.97 | 211.20 | 0.3 | 100.0 | 100.0 | 0.2 | 0.0 | 0.0 | –0.1 | 0.0 | 0.1 | D | M |
| Energy – Fugitive – Oil and Gas Activities | CH4 | 143.52 | 133.23 | 0.3 | 100.0 | 100.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Energy – Fugitive – Oil Transportation and Storage | CH4 | 4.40 | 4.97 | 5.0 | 50.0 | 50.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| IPPU – Chemical Industry | CH4 | 27.60 | 112.02 | 2.0 | 80.0 | 80.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | D | D |
| Agriculture – Enteric Fermentation | CH4 | 26,420.72 | 27,646.56 | 3.9 | 15.5 | 16.0 | 3.9 | 0.0 | 0.3 | –0.4 | 1.5 | 1.6 | M | M |
| Agriculture – Manure Management | CH4 | 668.85 | 1,475.15 | 5.0 | 20.0 | 20.6 | 0.3 | 0.0 | 0.0 | 0.1 | 0.1 | 0.2 | M | M |
| Agriculture – Burning of Residues | CH4 | 22.62 | 19.35 | 6.0 | 20.0 | 20.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| LULUCF | CH4 | 97.91 | 92.93 | 30.0 | 41.6 | 51.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| Waste – Solid Waste Disposal | CH4 | 3,711.34 | 3,724.45 | 92.0 | 40.0 | 100.3 | 3.3 | 0.0 | 0.0 | –0.2 | 4.9 | 4.9 | M | R |
| Waste – Wastewater Treatment and Discharge | CH4 | 218.81 | 246.89 | 10.0 | 40.0 | 41.2 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| Waste – Biological Treatment of Solid Waste | CH4 | 2.74 | 17.47 | 100.0 | 100.0 | 141.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Waste – Incineration and Open Burning of Waste | CH4 | 4.89 | 3.15 | 50.0 | 100.0 | 111.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Energy Industries – Sectoral Approach – Liquid | CH4 | 0.00 | 0.00 | 10.0 | 50.0 | 51.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Gas Diesel Oil – Sectoral Approach – Liquid | CH4 | 0.00 | 0.00 | 50.0 | 50.0 | 70.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Other/Residential – Sectoral Approach – Liquid | CH4 | 0.00 | 0.00 | 20.0 | 50.0 | 53.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Agriculture – Enteric Fermentation | CH4 | 0.00 | 0.00 | 20.0 | 50.0 | 53.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | D |
| Tokelau Agriculture – Manure Management | CH4 | 0.00 | 0.00 | 20.0 | 30.0 | 36.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | D |
| Tokelau Waste – Solid Waste Disposal | CH4 | 0.40 | 0.35 | 97.0 | 40.0 | 104.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | R |
| Tokelau Waste – Wastewater Treatment and Discharge | CH4 | 0.16 | 0.14 | 10.0 | 40.0 | 41.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| Tokelau Waste – Incineration and Open Burning of Waste | CH4 | 0.10 | 0.08 | 50.0 | 100.0 | 111.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Energy – Liquid Fuels | N2O | 140.77 | 156.34 | 1.2 | 50.0 | 50.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Solid Fuels | N2O | 14.99 | 14.39 | –4.0 | 50.0 | 50.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Gaseous Fuels | N2O | 5.53 | 3.84 | 0.3 | 50.0 | 50.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Biomass | N2O | 36.59 | 59.10 | 50.0 | 50.0 | 70.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Energy – Fugitive – Venting/flaring | N2O | 0.06 | 0.15 | 5.0 | 100.0 | 100.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| IPPU – Other Product Manufacture and Use | N2O | 102.45 | 61.79 | 15.0 | 0.0 | 15.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | R | R |
| Agriculture – Agricultural Soils | N2O | 6,689.12 | 8,566.17 | 11.3 | 54.1 | 55.3 | 4.2 | 0.0 | 0.1 | 0.5 | 1.4 | 1.5 | M | M |
| Agriculture – Manure Management | N2O | 51.89 | 121.63 | 5.0 | 100.0 | 100.1 | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | R | R |
| Agriculture – Burning of Residues | N2O | 4.77 | 4.00 | 6.0 | 20.0 | 20.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| LULUCF | N2O | 207.14 | 106.91 | 30.0 | 41.6 | 51.3 | 0.0 | 0.0 | 0.0 | –0.1 | 0.0 | 0.1 | M | M |
| Waste – Wastewater Treatment and Discharge | N2O | 82.40 | 114.64 | 10.0 | 90.0 | 90.6 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| Waste – Incineration and Open Burning of Waste | N2O | 2.60 | 1.89 | 50.0 | 100.0 | 111.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Waste – Biological Treatment of Solid Waste | N2O | 1.96 | 12.50 | 100.0 | 150.0 | 180.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Energy Industries – Sectoral Approach – Liquid | N2O | 0.00 | 0.00 | 10.0 | 50.0 | 51.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Gas Diesel Oil – Sectoral Approach – Liquid | N2O | 0.01 | 0.01 | 50.0 | 50.0 | 70.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Other/Residential – Sectoral Approach – Liquid | N2O | 0.00 | 0.00 | 20.0 | 50.0 | 53.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau IPPU – Other Product Manufacture and Use | N2O | 0.05 | 0.02 | 15.0 | 0.0 | 15.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | R | R |
| Tokelau Waste – Wastewater Treatment and Discharge | N2O | 0.02 | 0.02 | 10.0 | 90.0 | 90.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| Tokelau Waste – Incineration and open Burning of Waste | N2O | 0.01 | 0.01 | 50.0 | 100.0 | 111.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| IPPU – Product Uses as Substitutes for ODS | HFCs | 0.00 | 1,505.42 | 32.0 | 0.0 | 32.0 | 0.4 | 0.0 | 0.0 | 0.0 | 0.7 | 0.7 | R | R |
| IPPU – Aluminium Production | PFCs | 909.95 | 60.45 | 5.0 | 30.0 | 30.4 | 0.0 | 0.0 | 0.0 | –0.3 | 0.0 | 0.3 | M | M |
| IPPU – Product Uses as Substitutes for ODS | PFCs | 0.00 | 0.00 | 35.0 | 0.0 | 35.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | R | R |
| IPPU – Metal Industry | SF6 | 2.74 | 0.00 | 100.0 | 0.0 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | R | R |
| IPPU – Other Product Manufacture and Use | SF6 | 17.24 | 15.00 | 36.0 | 45.0 | 57.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | R | R |
| Tokelau IPPU – Product Uses as Substitutes for ODS | HFCs | 0.00 | 0.28 | 44.0 | 0.0 | 44.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | R | R |
| Total emissions/ removals |  | 98,768.3 | 113,611.1 |  |  | Uncertainty in the year | 10.7% |  | Uncertainty in the trend |  | 8.311% |  |  |  |

Note:	D = default; M = measurements; R = national referenced information.

Table A2.1.2 	Uncertainty calculation (excluding LULUCF) for New Zealand’s Greenhouse Gas Inventory 1990–2017 (2006 IPCC, Approach 1)

| IPCC source category | Gas | 1990 emissions or absolute value of removals, Kt CO2-e | 2017 emissions or absolute value of removals, kt CO2-e | Activity data uncertainty (%) | Emission or removal factor uncertainty (%) | Combined uncertainty (%) | Combined uncertainty as a per cent of the national total in 2017 (%) | Type A sensitivity (%) | Type B sensitivity (%) | Uncertainty in the trend in national total introduced by emission or removal factor uncertainty (%) | Uncertainty in trend in national total introduced by activity data uncertainty (%) | Uncertainty introduced into the trend in the national total (%) | Emission/ removal factor quality indicator | Activity data quality indicator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Energy – Liquid Fuels | CO2 | 11,789.99 | 19,743.54 | 1.2 | 0.5 | 1.3 | 0.3 | 0.1 | 0.3 | 0.0 | 0.5 | 0.5 | R | M |
| Energy – Solid Fuels | CO2 | 3,211.03 | 3,112.86 | –4.0 | 2.2 | 4.6 | 0.2 | 0.0 | 0.0 | 0.0 | –0.3 | 0.3 | M | M |
| Energy – Gaseous Fuels | CO2 | 7,054.90 | 7,700.48 | 0.3 | 2.4 | 2.4 | 0.2 | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | M | M |
| Energy – Fugitive – Geothermal | CO2 | 228.58 | 643.26 | 5.0 | 5.0 | 7.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | M | D |
| Energy – Fugitive – Venting/flaring | CO2 | 229.84 | 554.60 | 0.3 | 2.4 | 2.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| Energy – Fugitive – Oil and Gas Activities | CO2 | 0.21 | 0.20 | 5.0 | 100.0 | 100.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Energy – Fugitive – Transmission and Distribution | CO2 | 1.46 | 1.30 | 0.3 | 100.0 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| IPPU – Mineral Industry | CO2 | 561.87 | 669.67 | 2.0 | 7.0 | 7.3 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| IPPU – Chemical Industry | CO2 | 175.40 | 193.09 | 2.0 | 6.0 | 6.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| IPPU – Metal Industry | CO2 | 1,757.51 | 2,304.45 | 5.0 | 7.0 | 8.6 | 0.2 | 0.0 | 0.0 | 0.0 | 0.2 | 0.2 | D | D |
| IPPU – Non-energy Products from Fuels and Solvent Use | CO2 | 25.12 | 46.67 | 20.0 | 50.0 | 53.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Agriculture – Liming | CO2 | 360.06 | 459.60 | 3.4 | 50.0 | 50.1 | 0.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| Agriculture – Urea Application | CO2 | 39.19 | 588.26 | 10.0 | 50.0 | 51.0 | 0.4 | 0.0 | 0.0 | 0.4 | 0.1 | 0.4 | D | R |
| Waste – Incineration and Open Burning of Waste | CO2 | 17.12 | 3.75 | 50.0 | 40.0 | 64.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Energy Industries – Sectoral Approach – Liquid | CO2 | 1.15 | 0.20 | 10.0 | 7.0 | 12.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Gas Diesel Oil – Sectoral Approach – Liquid | CO2 | 1.60 | 1.60 | 50.0 | 1.5 | 50.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Other/residential – Sectoral Approach – Liquid | CO2 | 0.08 | 0.09 | 20.0 | 7.0 | 21.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Waste – Incineration and Open Burning of Waste | CO2 | 0.05 | 0.04 | 50.0 | 40.0 | 64.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Energy – Liquid Fuels | CH4 | 82.70 | 29.52 | 1.2 | 50.0 | 50.0 | 0.0 | 0.0 | 0.0 | –0.1 | 0.0 | 0.1 | D | M |
| Energy – Solid Fuels | CH4 | 36.40 | 27.50 | –4.0 | 50.0 | 50.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Gaseous Fuels | CH4 | 9.05 | 4.55 | 0.3 | 50.0 | 50.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Biomass | CH4 | 66.60 | 84.14 | 50.0 | 50.0 | 70.7 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | D | D |
| Energy – Fugitive – Geothermal | CH4 | 54.79 | 171.50 | 5.0 | 5.0 | 7.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Energy – Fugitive – Venting/processing | CH4 | 66.06 | 87.81 | 0.3 | 50.0 | 50.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Fugitive – Coal Mining | CH4 | 328.23 | 132.05 | –4.0 | 50.0 | 50.2 | 0.1 | 0.0 | 0.0 | –0.2 | 0.0 | 0.2 | D | M |
| Energy – Fugitive – Transmission and Distribution | CH4 | 279.97 | 211.20 | 0.3 | 100.0 | 100.0 | 0.3 | 0.0 | 0.0 | –0.2 | 0.0 | 0.2 | D | M |
| Energy – Fugitive – Oil and Gas Activities | CH4 | 143.52 | 133.23 | 0.3 | 100.0 | 100.0 | 0.2 | 0.0 | 0.0 | –0.1 | 0.0 | 0.1 | D | D |
| Energy – Fugitive – Oil Transportation and Storage | CH4 | 4.40 | 4.97 | 5.0 | 50.0 | 50.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| IPPU – Chemical Industry | CH4 | 27.60 | 112.02 | 2.0 | 80.0 | 80.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | D | D |
| Agriculture – Enteric Fermentation | CH4 | 26,420.72 | 27,646.56 | 3.9 | 15.5 | 16.0 | 5.5 | -0.1 | 0.4 | –1.1 | 2.3 | 2.6 | M | M |
| Agriculture – Manure Management | CH4 | 668.85 | 1,475.15 | 5.0 | 20.0 | 20.6 | 0.4 | 0.0 | 0.0 | 0.2 | 0.2 | 0.3 | M | M |
| Agriculture – Burning of Residues | CH4 | 22.62 | 19.35 | 6.0 | 20.0 | 20.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| Waste – Solid Waste Disposal | CH4 | 3,711.34 | 3,724.45 | 92.0 | 40.0 | 100.3 | 4.6 | 0.0 | 0.1 | –0.5 | 7.4 | 7.4 | M | R |
| Waste – Wastewater Treatment and Discharge | CH4 | 218.81 | 246.89 | 10.0 | 40.0 | 41.2 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | D | R |
| Waste – Biological Treatment of Solid Waste | CH4 | 2.74 | 17.47 | 100.0 | 100.0 | 141.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Waste – Incineration and Open Burning of Waste | CH4 | 4.89 | 3.15 | 50.0 | 100.0 | 111.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Energy Industries – Sectoral Approach – Liquid | CH4 | 0.00 | 0.00 | 10.0 | 50.0 | 51.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Gas Diesel Oil – Sectoral Approach – Liquid | CH4 | 0.00 | 0.00 | 50.0 | 50.0 | 70.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Other/residential – Sectoral Approach – Liquid | CH4 | 0.00 | 0.00 | 20.0 | 50.0 | 53.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Agriculture – Enteric Fermentation | CH4 | 0.00 | 0.00 | 20.0 | 50.0 | 53.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | D |
| Tokelau Agriculture – Manure Management | CH4 | 0.00 | 0.00 | 20.0 | 30.0 | 36.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | D |
| Tokelau Waste – Solid Waste Disposal | CH4 | 0.40 | 0.35 | 97.0 | 40.0 | 104.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | R |
| Tokelau Waste – Wastewater Treatment and Discharge | CH4 | 0.16 | 0.14 | 10.0 | 40.0 | 41.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| Tokelau Waste – Incineration and Open Burning of Waste | CH4 | 0.10 | 0.08 | 50.0 | 100.0 | 111.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Energy – Liquid Fuels | N2O | 140.77 | 156.34 | 1.2 | 50.0 | 50.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Solid Fuels | N2O | 14.99 | 14.39 | –4.0 | 50.0 | 50.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Gaseous Fuels | N2O | 5.53 | 3.84 | 0.3 | 50.0 | 50.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | M |
| Energy – Biomass | N2O | 36.59 | 59.10 | 50.0 | 50.0 | 70.7 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | D | D |
| Energy – Fugitive – Venting/flaring | N2O | 0.06 | 0.15 | 5.0 | 100.0 | 100.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| IPPU – Other Product Manufacture and Use | N2O | 102.45 | 61.79 | 15.0 | 0.0 | 15.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | R | R |
| Agriculture – Agricultural Soils | N2O | 6,689.12 | 8,566.17 | 11.3 | 54.1 | 55.3 | 5.9 | 0.0 | 0.1 | 0.3 | 2.1 | 2.1 | M | M |
| Agriculture – Manure Management | N2O | 51.89 | 121.63 | 5.0 | 100.0 | 100.1 | 0.2 | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | R | R |
| Agriculture – Burning of Residues | N2O | 4.77 | 4.00 | 6.0 | 20.0 | 20.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| Waste – Wastewater Treatment and Discharge | N2O | 82.40 | 114.64 | 10.0 | 90.0 | 90.6 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| Waste – Incineration and Open Burning of Waste | N2O | 2.60 | 1.89 | 50.0 | 100.0 | 111.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Waste – Biological Treatment of Solid Waste | N2O | 1.96 | 12.50 | 100.0 | 150.0 | 180.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Energy Industries – Sectoral Approach – Liquid | N2O | 0.00 | 0.00 | 10.0 | 50.0 | 51.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Gas Diesel Oil – Sectoral Approach – Liquid | N2O | 0.01 | 0.01 | 50.0 | 50.0 | 70.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau Other/residential – Sectoral Approach – Liquid | N2O | 0.00 | 0.00 | 20.0 | 50.0 | 53.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| Tokelau IPPU – Other Product Manufacture and Use | N2O | 0.05 | 0.02 | 15.0 | 0.0 | 15.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | R | R |
| Tokelau Waste – Wastewater Treatment and Discharge | N2O | 0.02 | 0.02 | 10.0 | 90.0 | 90.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | R |
| Tokelau Waste – Incineration and Open Burning of Waste | N2O | 0.01 | 0.01 | 50.0 | 100.0 | 111.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | D | D |
| IPPU – Product Uses as Substitutes for ODS | HFCs | 0.00 | 1,505.42 | 32.0 | 0.0 | 32.0 | 0.6 | 0.0 | 0.0 | 0.0 | 1.0 | 1.0 | R | R |
| IPPU – Aluminium Production | PFCs | 909.95 | 60.45 | 5.0 | 30.0 | 30.4 | 0.0 | 0.0 | 0.0 | –0.5 | 0.0 | 0.5 | M | M |
| IPPU – Product Uses as Substitutes for ODS | PFCs | 0.00 | 0.00 | 35.0 | 0.0 | 35.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | R | R |
| IPPU – Metal Industry | SF6 | 2.74 | 0.00 | 100.0 | 0.0 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | R | R |
| IPPU – Other Product Manufacture and Use | SF6 | 17.24 | 15.00 | 36.0 | 45.0 | 57.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | R | R |
| Tokelau IPPU – Product Uses as Substitutes for ODS | HFCs | 0.00 | 0.28 | 44.0 | 0.0 | 44.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | R | R |
| Total emissions |  | 65,668.3 | 80,853.4 |  |  |  | Uncertainty in the year | 9.3% |  |  | Uncertainty in the trend |  | 8.2% |  |

Note:	D = default; M = measurements; R = national referenced information.

## Annex 2: References

IPCC. 2006. Eggleston HS, Buendia L, Miwa K, Ngara T, Tanabe K (eds). 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 1. General Guidance and Reporting. IPCC National Greenhouse Gas Inventories Programme. Japan: Published for the IPCC by the Institute for Global Environmental Strategies.

UNFCCC. 2013. FCCC/CP/2013/Add.3. Guidelines for the preparation of national communications by Parties included in Annex I to the Convention, Part I: UNFCCC reporting guidelines on annual greenhouse gas inventories (addendum to Decision 24/CP.19).

# Annex 3: Detailed methodological information for other sectors

## A3.1	Supplementary information for the Agriculture sector

### A3.1.1	Livestock population data

#### 2017 Agricultural Production census and survey

Details of the Agricultural Production census and survey are included to provide an understanding of the livestock statistics process and uncertainty values. The information here is provided by Statistics New Zealand, with full details available from the Statistics New Zealand website (www.stats.govt.nz/information-releases/agricultural-production-statistics-june-2017-final).

Statistics New Zealand holds an Agricultural Production census every five years, with the most recent census held in 2017. In all other years, Statistics New Zealand holds an Agricultural Production survey. The national inventory report is compiled with data from the Agricultural Production census and survey.

The target population for the 2017 Agricultural Production census was all businesses that were engaged in agricultural production activity (including livestock, cropping, horticulture and forestry) or that owned land that was intended for agricultural activity during the year ended 30 June 2017. The response rate, or the estimated proportion of eligible businesses that responded to the 2017 Agricultural Production census, was 85.5 per cent. These businesses represent 88.3 per cent of the total estimated value of agricultural operations. Statistics New Zealand imputes by randomly selecting data from the same population (i.e., ‘hot deck’ procedure) for values for farmers and growers who did not return a completed questionnaire. The imputation levels of the 2016 Agricultural Production survey and the 2017 Agricultural Production census are provided in table A3.1.1.

Sampling error arises in the Agricultural Production survey from selecting a sample of businesses and weighting the results rather than taking a complete enumeration (i.e., census). Non-sampling error arises from biases in the patterns of response and non-response, inaccuracies in reporting by respondents and errors in the recording and classification of data. Statistics New Zealand adopts procedures to detect and minimise these types of errors, but they may still occur and are not easy to quantify.

Table A3.1.1	Imputation levels and sampling errors for recent Agricultural Production surveys

| Statistic | Proportion of total estimate imputed (%) |  | Relative sampling errors at 95% confidence interval (%) |  |
| --- | --- | --- | --- | --- |
| Survey year | 2016 | 2017 | 2016 | 2017 |
| Ewe hoggets put to ram | 22 | 13 | 3 | N/A |
| Breeding ewes, two tooth and over | 21 | 13 | 2 | N/A |
| Total number of sheep | 21 | 13 | 2 | N/A |
| Lambs born to ewe hoggets | 22 | 12 | 3 | N/A |
| Lambs born to ewes | 21 | 13 | 2 | N/A |
| Total number of lambs | 21 | 13 | 2 | N/A |
| Dairy cows and heifers, in milk or calf | 29 | 19 | 5 | N/A |
| Total number of dairy cattle | 29 | 19 | 5 | N/A |
| Calves born alive to beef heifers and/or cows | 28 | 13 | 4 | N/A |
| Beef cows and heifers (in calf) one to two years | 22 | 12 | 7 | N/A |
| Beef cows and heifers (in calf) two years and over | 20 | 12 | 3 | N/A |
| Total number of beef cattle | 21 | 13 | 3 | N/A |
| Calves born alive to dairy heifers and/or cows | 22 | 20 | 4 | N/A |
| Female deer mated | 16 | 8 | 7 | N/A |
| Total number of deer | 16 | 9 | 6 | N/A |
| Fawns born on farm and alive at four months | 17 | 8 | 7 | N/A |
| Breeding sows (one year and over) | 9 | 4 | 5 | N/A |
| Mated gilts | 7 | 3 | 5 | N/A |
| Total pigs | 7 | 3 | 2 | N/A |
| Piglets weaned on the farm | 7 | 3 | 1 | N/A |
| Area of wheat harvested | 24 | 8 | 11 | N/A |
| Area of barley harvested | 26 | 10 | 9 | N/A |
| Area of maize grain harvested | 18 | 12 | 10 | N/A |

#### Livestock characterisation in New Zealand’s Tier 2 modelling

The delineation of the major livestock categories in New Zealand’s Tier 2 livestock nutritional and energy requirements modelling (see table A3.1.2) are taken from population data collected by the Agricultural Production census and survey and the Ministry for Primary Industries slaughter statistics.

Table A3.1.2	Characterisation of major livestock subcategories (dairy cattle, non-dairy cattle,
sheep and deer) in New Zealand’s Tier 2 livestock modelling

| Livestock category |  | Subcategory |
| --- | --- | --- |
| Dairy cattle | Dairy cattle categories | Milking cows and heifers |
|  |  | Growing females less than one year |
|  |  | Growing females one to two years |
|  |  | Breeding bulls |
|  | Dairy data: geographical regions based on regional council areas | Northland |
|  |  | Auckland |
|  |  | Waikato |
|  |  | Bay of Plenty |
|  |  | Gisborne |
|  |  | Hawke's Bay |
|  |  | Taranaki |
|  |  | Manawatu–Wanganui |
|  |  | Wellington |
|  |  | Tasman |
|  |  | Nelson |
|  |  | Marlborough |
|  |  | West Coast |
|  |  | Canterbury |
|  |  | Otago |
|  |  | Southland |
| Non-dairy (beef) cattle categories |  | Breeding growing cows 0 to one year |
|  |  | Breeding growing cows one to two years |
|  |  | Breeding growing cows two to three years |
|  |  | Breeding mature cows |
|  |  | Breeding bulls – mixed age |
|  |  | Slaughter heifers 0 to one year |
|  |  | Slaughter heifers one to two years |
|  |  | Slaughter steers 0 to one year |
|  |  | Slaughter steers one to two years |
|  |  | Slaughter bulls 0 to one year |
|  |  | Slaughter bulls one to two years |
| Sheep categories |  | Dry ewes |
|  |  | Mature breeding ewes |
|  |  | Growing breeding sheep |
|  |  | Growing non-breeding sheep |
|  |  | Wethers |
|  |  | Lambs |
|  |  | Rams |
| Deer categories |  | Breeding hinds |
|  |  | Hinds less than one year |
|  |  | Hinds one to two years |
|  |  | Stags less than one year |
|  |  | Stags one to two years |
|  |  | Stags two to three years |
|  |  | Mixed age and breeding stags |

### A3.1.2	Key parameters and emission factors used in the
Agriculture sector

For the major livestock categories, milk yield varies over the course of a year, which affects energy requirements, intake and emissions. Table A3.1.3 shows the proportions that are used to calculate milk yield for different months over the course of a year. Table A3.1.4 shows the emission factors used to calculate methane emissions from livestock, while table A3.1.5 shows the emission factors used to calculate nitrous oxide emissions from agriculture. Table A3.1.6 shows some of the parameter values used to calculate nitrous oxide emissions.

Table A3.1.3	Proportion of annual milk yield each month for major livestock categories

| Month | Dairy cattle | Non-dairy cattle | Sheep | Deer |
| --- | --- | --- | --- | --- |
| July | 0.0088 | 0.0000 | 0.0000 | 0.0000 |
| August | 0.0578 | 0.0000 | 0.0000 | 0.0000 |
| September | 0.1213 | 0.1670 | 0.1639 | 0.0000 |
| October | 0.1503 | 0.1670 | 0.2541 | 0.0000 |
| November | 0.1425 | 0.1670 | 0.2459 | 0.1000 |
| December | 0.1282 | 0.1670 | 0.2541 | 0.2583 |
| January | 0.1109 | 0.1670 | 0.0820 | 0.2583 |
| February | 0.0900 | 0.1670 | 0.0000 | 0.2333 |
| March | 0.0851 | 0.0000 | 0.0000 | 0.1500 |
| April | 0.0654 | 0.0000 | 0.0000 | 0.0000 |
| May | 0.0335 | 0.0000 | 0.0000 | 0.0000 |
| June | 0.0061 | 0.0000 | 0.0000 | 0.0000 |

Source: Suttie (2012) and Pickering and Fick (2015)

Note:	All values presented in the table are rounded to four decimal places for presentation purposes and precise values are available upon request.

Table A3.1.4	Emission factors for Tier 1 enteric fermentation livestock and manure management

| Emission factor | Emission type | Source | Parameter value (kg CH4/head/yr) |
| --- | --- | --- | --- |
| EFGOATS | Enteric fermentation – goats | Lassey (2011) | 8.1 |
| EFHORSES | Enteric fermentation – horses | IPCC (2006), table 10.10 | 18 |
| EFMULES | Enteric fermentation – mules and asses | IPCC (2006), table 10.10 | 10 |
| EFSWINE | Enteric fermentation – swine | Hill (2012) | 1.06 |
| EFALPACA | Enteric fermentation – alpaca | IPCC (2006), table 10.10 | 8 |
| MMGOATS | Manure management – goats | IPCC (2006), table 10.15 | 0.20 |
| MMHORSES | Manure management – horses | IPCC (2006), table 10.15 | 2.34 |
| MMMULES | Manure management – mules and asses | IPCC (2006), table 10.15 | 1.1 |
| MMSWINE | Manure management – swine | Hill (2012); IPCC (2000) | 5.94 |
| MMBROILERS | Manure management – broilers | Fick et al. (2011) | 0.022 |
| MMLAYERS | Manure management – layer hens | Fick et al. (2011) | 0.016 |
| MMOTHER POULTRY | Manure management – other poultry | IPCC (1996), table 4.5 | 0.117 |
| MMALPACA | Manure management – alpaca | New Zealand 1990 sheep value | 0.097 |

Table A3.1.5	Emission factors for New Zealand’s agriculture nitrous oxide emissions

| Emission factor | Emissions | Source | Parameter value |
| --- | --- | --- | --- |
| EF1 (kg N2O-N/kg N) | Direct emissions from nitrogen input to soil | Kelliher and de Klein (unpublished) | 0.0100 |
| EF1-UREA (kg N2O-N/kg N) | Direct emissions from nitrogen input to soil from urea fertiliser | van der Weerden et al. (2016) | 0.0059 |
| EF1-DAIRY (kg N2O-N/kg N) | Direct emissions from nitrogen input to soil from dairy cattle manure | van der Weerden et al. (2016) | 0.0025 |
| EF2 (kg N2O-N/ha-yr) | Direct emissions from organic soil mineralisation due to cultivation | IPCC (2006), table 11.1 | 8.0000 |
| EF3SSD (kg N2O-N/kg N excreted) | Direct emissions from waste in the solid waste and dry lot animal waste management systems | IPCC (2000), table 4.12 | 0.0200 |
| EF3 (PRP URINE) (kg N2O-N/kg N excreted) | Direct emissions from urine in the pasture, range and paddock animal waste management systems for cattle, sheep and deer, and direct emissions from manure waste in the pasture, range and paddock animal waste management systems for all other species | Carran et al. (1995); Muller et al. (1995); de Klein et al. (2003) | 0.0100 |
| EF3(PRP DUNG) (kg N2O-N/kg N excreted) | Direct emissions from dung in the pasture, range and paddock animal waste management systems for cattle, sheep and deer | Luo et al. (2009) | 0.0025 |
| EF3OTHER (kg N2O-N/kg N excreted) | Direct emissions from waste in other animal waste management systems | IPCC (2000), table 4.13 | 0.0050 |
| EF3POULTRY (kg N2O-N/kg N excreted) | Direct emissions from waste in other animal waste management systems – poultry specific | Fick et al. (2011) | 0.0010 |
| EF4 (kg N2O-N/kg NHx-N) | Indirect emissions from volatising nitrogen | IPCC (2006), table 11.3 | 0.0100 |
| EF5 (kg N2O-N/kg N leached and run-off) | Indirect emissions from leaching nitrogen | IPCC (2006), table 11.3 | 0.0075 |

Table A3.1.6	Parameter values for New Zealand’s agriculture nitrous oxide emissions

| Parameter (fraction) | Fraction of the parameter | Source | Parameter value |
| --- | --- | --- | --- |
| FracGASF (kg NH3-N + NOx-N/kg of synthetic fertiliser N applied) | Total synthetic fertiliser emitted as NOx or NH3 | IPCC (2006) verified by Sherlock et al. (2008) | 0.1 |
| FracGASM (kg NH3-N + NOx-N/kg of N excreted by livestock) | Total nitrogen emitted as NOx or NH3 | Sherlock et al. (2008) | 0.1 |
| FracLEACH(-H) (kg N/kg fertiliser or manure N) | Nitrogen input to soils that is lost through leaching and run-off | Thomas et al. (unpublished, 2005) | 0.07 |
| FracBURN (kg N/kg crop-N) | Crop residue burned in fields | Thomas et al. (2008), table 14 | Crop specific survey data |
| FracBURNL (kg N/kg legume-N) | Legume crop residue burned in fields | Thomas et al. (2008) Practice does not occur in New Zealand | 0 |
| FracRENEW | Fraction of land undergoing pasture renewal | Thomas et al. (2014) | Year specific |
| FracREMOVE | Fraction of nitrogen in above-ground residues removed for bedding, feed or construction | Thomas et al. (2014) Practice does not occur in New Zealand | 0 |
| FracFUEL (N/kg N excreted) | Livestock nitrogen excretion in excrements burned for fuel | Practice does not occur in New Zealand | 0 |

Some of the parameters used to calculate Nitrous oxide emissions from crop residue returned to soil and emissions from Field burning of agricultural residues are summarised in table A3.1.7. These values are taken from research conducted by Thomas et al. (2008, 2011).

Table A3.1.7	Parameter values for New Zealand’s cropping emissions

| Crop | HI | dmf | AGN | Root Shoot ratio RBG | BGN |
| --- | --- | --- | --- | --- | --- |
| Wheat | 0.41 | 0.86 | 0.005 | 0.1 | 0.009 |
| Barley | 0.46 | 0.86 | 0.005 | 0.1 | 0.009 |
| Oats | 0.3 | 0.86 | 0.005 | 0.1 | 0.009 |
| Maize grain | 0.5 | 0.86 | 0.007 | 0.1 | 0.007 |
| Field seed peas | 0.5 | 0.21 | 0.02 | 0.1 | 0.015 |
| Lentils | 0.5 | 0.86 | 0.02 | 0.1 | 0.015 |
| Peas fresh and process | 0.45 | 0.86 | 0.03 | 0.1 | 0.015 |
| Potatoes | 0.9 | 0.22 | 0.02 | 0.1 | 0.01 |
| Onions | 0.8 | 0.11 | 0.02 | 0.1 | 0.01 |
| Sweet corn | 0.55 | 0.24 | 0.009 | 0.1 | 0.007 |
| Squash | 0.8 | 0.2 | 0.02 | 0.1 | 0.01 |
| Herbage seeds | 0.11 | 0.85 | 0.015 | 0.1 | 0.01 |
| Legume seeds | 0.09 | 0.85 | 0.04 | 0.1 | 0.01 |
| Brassica seeds | 0.2 | 0.85 | 0.01 | 0.1 | 0.008 |

Source: Thomas et al. (2008, 2011)

Note: 	AGN = above-ground nitrogen residue; BGN = below-ground nitrogen residue; dmf = dry-matter conversion factor; HI = harvest index; RBG = ratio of below-ground residues to the harvest yield.

## A3.2	Supplementary information for the LULUCF sector

### A3.2.1	Annual land-use change summary

This section contains a summary of the annual land-use change from 1990 to 2017 (see table A3.2.1).

Table A3.2.1 	Annual land-use changes (units in 000s hectares)

### A3.2.2 	Natural forest yield table

This section contains the natural forest yield table used for this submission.

Table A3.2.2	Pre-1990 natural forest – regenerating forest yield table (tonnes C ha–1)

| Year | Above-ground biomass | Below-ground biomass | Dead wood | Litter | Total biomass |
| --- | --- | --- | --- | --- | --- |
| 1990 | 37.47 | 9.37 | 5.35 | 6.58 | 58.77 |
| 1991 | 38.53 | 9.63 | 5.43 | 6.58 | 60.17 |
| 1992 | 39.59 | 9.9 | 5.5 | 6.58 | 61.57 |
| 1993 | 40.65 | 10.16 | 5.57 | 6.58 | 62.96 |
| 1994 | 41.7 | 10.43 | 5.64 | 6.58 | 64.35 |
| 1995 | 42.76 | 10.69 | 5.71 | 6.58 | 65.74 |
| 1996 | 43.82 | 10.96 | 5.78 | 6.58 | 67.14 |
| 1997 | 44.88 | 11.22 | 5.85 | 6.58 | 68.53 |
| 1998 | 45.94 | 11.48 | 5.92 | 6.58 | 69.92 |
| 1999 | 46.99 | 11.75 | 5.99 | 6.58 | 71.31 |
| 2000 | 48.05 | 12.01 | 6.06 | 6.58 | 72.7 |
| 2001 | 49.11 | 12.28 | 6.13 | 6.58 | 74.1 |
| 2002 | 50.17 | 12.54 | 6.2 | 6.58 | 75.49 |
| 2003 | 51.22 | 12.81 | 6.27 | 6.58 | 76.88 |
| 2004 | 52.28 | 13.07 | 6.34 | 6.58 | 78.27 |
| 2005 | 53.34 | 13.34 | 6.41 | 6.58 | 79.67 |
| 2006 | 54.4 | 13.6 | 6.48 | 6.58 | 81.06 |
| 2007 | 55.46 | 13.86 | 6.55 | 6.58 | 82.45 |
| 2008 | 56.51 | 14.13 | 6.62 | 6.58 | 83.84 |
| 2009 | 57.57 | 14.39 | 6.69 | 6.58 | 85.23 |
| 2010 | 58.63 | 14.66 | 6.76 | 6.58 | 86.63 |
| 2011 | 59.69 | 14.92 | 6.83 | 6.58 | 88.02 |
| 2012 | 60.74 | 15.19 | 6.9 | 6.58 | 89.41 |
| 2013 | 61.8 | 15.45 | 6.97 | 6.58 | 90.8 |
| 2014 | 62.86 | 15.71 | 7.04 | 6.58 | 92.19 |
| 2015 | 63.92 | 15.97 | 7.11 | 6.58 | 93.58 |
| 2016 | 64.98 | 16.23 | 7.18 | 6.58 | 94.97 |
| 2017 | 66.04 | 16.49 | 7.25 | 6.58 | 96.36 |

### A3.2.3	Uncertainty analysis for the LULUCF sector

This section contains the disaggregated uncertainty analysis for the Land Use, Land-Use Change and Forestry (LULUCF) sector. This additional information has been provided as a result of the review of New Zealand’s 2010 inventory (2012 submission). One of the recommendations of the review was that New Zealand provides “a detailed disaggregated assessment of uncertainty, as well as the aggregated uncertainty associated with the LULUCF sector, consistent with the Intergovernmental Panel on Climate Change (IPCC) good practice guidance for LULUCF”. This information is now provided in table A3.2.3.

Table A3.2.3	Uncertainty analysis for the LULUCF sector

| IPCC category | Gas | 1990 emissions or removals (kt CO2-e) | 2017 emissions or removals (kt CO2-e) | Activity data uncertainty (%) | Emission factor / estimation parameter uncertainty (biomass) (%) | Emission factor / estimation parameter uncertainty (mineral soil) (%) | Combined uncertainty (%) | Contribution to variance by category in 2017 (%) | Type A sensitivity (%) | Type B sensitivity (%) | Uncertainty in trend in LULUCF emissions introduced by emission factor / estimation parameter uncertainty (%) | Uncertainty in trend in LULUCF emissions introduced by activity data uncertainty (%) | Uncertainty introduced into the trend in total LULUCF emissions (%) | Emission factor quality indicator | Activity data quality indicator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-1990 natural forest remaining pre-1990 natural forest | CO2 | –6,154.6 | –6,125.5 | 5.0 | 126.6 | 7.9 | 126.7 | 32.4 | 4.5 | 19.7 | 5.7 | 0.3 | 5.7 | M | M |
| Land converted to pre-1990 natural forest | CO2 | –124.8 | 159.8 | 5.0 | 126.6 | 7.9 | 241.5 | 1.6 | –0.8 | –0.5 | –1.0 | –0.1 | 1.0 | M | M |
| Pre-1990 planted forest remaining pre-1990 planted forest | CO2 | 4,277.6 | 1,186.1 | 5.0 | 11.4 | 12.3 | 693.3 | 34.3 | 6.8 | –3.8 | 0.8 | 0.5 | 0.9 | M | M |
| Land converted to pre-1990 planted forest | CO2 | –28,293.3 | –819.4 | 5.0 | 11.4 | 12.3 | 10.3 | 0.4 | –66.6 | 2.6 | –7.6 | –4.7 | 8.9 | M | M |
| Post-1989 forest remaining post-1989 forest | CO2 | 0.0 | 0.0 | 8.0 | 15.9 | 10.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| Land converted to post-1989 planted forest | CO2 | 247.1 | –16,240.4 | 8.0 | 15.9 | 10.4 | 16.2 | 11.0 | 52.7 | 52.1 | 8.4 | 6.0 | 10.3 | M | M |
| G-WB remaining G-WB | CO2 | 61.8 | 48.5 | 83.0 | 75.0 | 7.3 | 86.9 | 0.2 | 0.0 | –0.2 | 0.0 | 0.0 | 0.0 | M | M |
| Land converted to G-WB | CO2 | –459.2 | –129.6 | 83.0 | 75.0 | 7.3 | 83.1 | 0.4 | –0.7 | 0.4 | –0.5 | –0.8 | 1.0 | M | M |
| G-HP remaining G-HP | CO2 | 1,116.9 | 1,098.5 | 8.0 | 75.0 | 5.8 | 90.4 | 4.1 | –0.8 | –3.5 | –0.6 | –0.1 | 0.6 | M | M |
| Land converted to G-HP | CO2 | –787.7 | 1,631.4 | 8.0 | 75.0 | 5.8 | 21.1 | 1.4 | –7.2 | –5.2 | –5.4 | –0.8 | 5.4 | M | M |
| G-LP remaining G-LP | CO2 | 220.7 | 67.1 | 8.0 | 75.0 | 7.3 | 90.4 | 0.3 | 0.3 | –0.2 | 0.2 | 0.0 | 0.2 | M | M |
| Land converted to G-LP | CO2 | –44.1 | 936.2 | 8.0 | 75.0 | 7.3 | 10.4 | 0.4 | –3.1 | –3.0 | –2.3 | –0.4 | 2.4 | M | M |
| Cropland – perennial remaining cropland – perennial | CO2 | 81.8 | 76.4 | 8.0 | 75.0 | 14.1 | 90.4 | 0.3 | 0.0 | –0.2 | 0.0 | 0.0 | 0.0 | M | M |
| Land converted to cropland – perennial | CO2 | 45.0 | –9.3 | 8.0 | 75.0 | 14.1 | 690.4 | 0.3 | 0.1 | 0.0 | 0.1 | 0.0 | 0.1 | M | M |
| Cropland – annual remaining cropland – annual | CO2 | 269.1 | 260.2 | 8.0 | 75.0 | 9.7 | 90.4 | 1.0 | –0.2 | –0.8 | –0.1 | 0.0 | 0.1 | M | M |
| Land converted to cropland – annual | CO2 | 73.5 | 60.1 | 8.0 | 75.0 | 9.7 | 33.3 | 0.1 | 0.0 | –0.2 | 0.0 | 0.0 | 0.0 | M | M |
| Wetland – open water remaining wetlands – open water | CO2 | 0.0 | 0.0 | 33.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| Land converted to wetland – open water | CO2 | –20.0 | –2.9 | 33.0 | 0.0 | 0.0 | 74.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| Wetland – vegetative non-forest remaining wetland – vegetative non-forest | CO2 | 9.4 | 17.9 | 33.0 | 75.0 | 12.3 | 95.9 | 0.1 | 0.0 | –0.1 | 0.0 | 0.0 | 0.0 | M | M |
| Land converted to wetland – vegetative non-forest | CO2 | 0.6 | –1.1 | 33.0 | 75.0 | 12.3 | 17.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| Settlements remaining settlements | CO2 | 65.1 | 65.1 | 22.0 | 75.0 | 95.0 | 92.6 | 0.3 | 0.0 | –0.2 | 0.0 | 0.0 | 0.0 | M | M |
| Land converted to settlements | CO2 | 7.6 | 35.6 | 22.0 | 75.0 | 95.0 | 75.6 | 0.1 | –0.1 | –0.1 | –0.1 | 0.0 | 0.1 | M | M |
| Other land remaining other land | CO2 | 0.0 | 0.0 | 22.0 | 75.0 | 70.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | M | M |
| Land converted to other land | CO2 | 13.5 | 45.6 | 22.0 | 75.0 | 70.7 | 30.1 | 0.1 | –0.1 | –0.1 | –0.1 | 0.0 | 0.1 | M | M |
| Harvested wood products | CO2 | –2,072.9 | –6,518.5 | 15.0 | 67.4 | – | 68.2 | 18.6 | 15.8 | 20.9 | 10.6 | 3.4 | 11.2 | M | M |
| LULUCF CH4 (CO2-e) | CO2-e | 97.9 | 92.9 | 30.0 | 41.6 | – | 51.3 | 0.2 | –0.1 | –0.3 | 0.0 | 0.0 | 0.0 | R | R |
| LULUCF N2O (CO2-e) | CO2-e | 207.1 | 106.9 | 30.0 | 41.6 | – | 12.2 | 0.1 | 0.2 | –0.3 | 0.1 | 0.1 | 0.1 | R | R |

Note:	G-HP = high producing grassland; G-LP = low producing grassland; G-WB = grassland with woody biomass; M = measurements; R = national referenced information.

### A3.2.4	LUCAS data management system

The Land Use Carbon Analysis System (LUCAS) data management system stores, manages and archives data for international greenhouse gas reporting for the LULUCF sector. This system is used for managing the land use spatial databases, plot and reference data, and for combining the two sets of data to calculate the numbers required for reporting under the United Nations Framework Convention on Climate Change (the Convention) and the Kyoto Protocol (see figure A3.2.1).

The data collected is stored and manipulated within three systems: the Geospatial System, the Gateway, and the Calculation and Reporting Application (CRA).

The main objectives of these systems are to:

- provide a transparent system for data storage and carbon calculations

- provide a repository for the versioning and validation of plot measurements and land use data

- calculate carbon stocks, emissions and removals per hectare for land uses and carbon pools based on the plot and spatial data collected

- calculate biomass burning emissions by land use based on area and emission factors stored in the Gateway

- produce the outputs required for the LULUCF sector reporting under the Convention and the Kyoto Protocol

- archive all inputs and outputs used in reporting.

The module ‘joint calculations’ refers to the process New Zealand uses to estimate national average carbon values by carbon pool for each land use category and subcategory.

The joint calculation process is performed within the CRA. Within the joint calculations interface, the user selects the appropriate area data and emission factors. The results of the calculations are carbon gains, losses and net change for all land use subcategories (whether in a conversion state or land remaining land), by year, by carbon pool, and stratified by North Island or South Island.

Figure A3.2.1	New Zealand’s LUCAS data management system

Note:	LUM = land use map; QA/QC = quality assurance/quality control. Joint calculations are described below.

#### Geospatial System

The Geospatial System consists of hardware and specific applications designed to meet LULUCF reporting requirements. The hardware largely comprises servers for spatial database storage, management, versioning and running web-mapping applications. The core components of the Geospatial System are outlined in figure A3.2.2.

Figure A3.2.2	New Zealand’s Geospatial System components

Note:	Blue indicates land use mapping data flow. LUCAS = Land Use Carbon Analysis System; 
LUM = land use map.

#### Land use mapping functionality

The land use mapping (LUM) functionality of the Geospatial System largely involves the editing and maintenance of time-stamped land use mapping data. The five main components within the LUM functionality are:

- LUM Import/Export Tool – provides functionality for managing the importing and exporting of LUM data in to and out of the database

- LUM Attribute Tool – an extension to the standard ArcGIS Desktop software that facilitates maintenance and updates to the LUM data by external contractors

- LUM Database – a non-versioned geographic information system (GIS) database for interim LUM data

- Spatial Gateway Tool – used to validate and version data from the LUM database prior to loading into the LUCAS GIS database. Validation business rules are stored in the Spatial Gateway database

- LUCAS Database – stores versions of LUM used to derive land-use change reporting.

#### LUCAS Management Studio

The LUCAS Management Studio (see figure A3.2.3) is the package of applications used to store activity data and calculate and report New Zealand’s emissions and removals for LULUCF. The LUCAS Gateway is a data warehouse with the purpose of storing, versioning and validating activity data and emission factors. The CRA sources all data from the Gateway. It then calculates and outputs New Zealand’s emissions and removals for LULUCF for land remaining land and land converted to another land use by pool and year.

Figure A3.2.3 	LUCAS Management Studio

#### LUCAS Gateway

The LUCAS Gateway enables the storage of activity data such as field plot data, land use area, biomass burning and other data needed by the CRA, such as IPCC defaults.

The LUCAS Gateway provides a viewing, querying and editing interface to the source (plot, land use area, carbon and non-carbon) data. It also stores any published or saved results from running the CRA.

All activity data and emission factors are stored within the Gateway database (see figure A3.2.4). It contains the following main components.

- A data and results layer containing all activity data (natural, planted forest, soils, default carbon, non-carbon, land use areas, land-use change and reference tables). The user has the ability to create a ‘snapshot’ in time (a data set archiving system) of the data held in the Gateway. This enables users of the CRA to select from a range of data snapshots and ensures past results can be replicated over time.

- A validation layer allows users to judge the suitability of data for use in the CRA calculations, subsequent to passing primary validation. Where records are deemed not acceptable for use within published reports, they are tagged as ‘invalid’ in the LUCAS Gateway database.

- An audit trail provides a history of any changes to the database tables within the Gateway.

- Versioning at a number of levels ensures any changes to data, schema or the database itself are logged and versioned, while providing the user with the ability to track what changes have been applied and roll back to a previous version if required. The results of saved or published reports within the CRA are also stored within the Gateway for repeatability and reference.

- Primary data validation, both during data capture and during import of the data into the Gateway, ensures only data that has passed acceptability criteria is available for a publishable CRA run.

- Hosting and application support provides hosting services, system security, backup and restore, daily maintenance and monitoring for the Gateway and CRA.

Figure A3.2.4	LUCAS Gateway database

#### Calculation and Reporting Application

The CRA enables users to import carbon and non-carbon data from the Gateway and, by running the various modules, determine emissions and removals by New Zealand’s forests, cropland, grassland and other land use types. This information, combined with land area data, enables New Zealand to meet its reporting requirements under the Convention and the Kyoto Protocol.

The CRA allows for the inclusion of other data sets, models and calculations without the complete redesign of the applications. All models, data and results are versioned, and the CRA allows the user to alter specific key values within a model or calculation (parameters) without the intervention of a programmer or technical support officer. The CRA is deployed as a client-based application that sources the required data from the Gateway.

The CRA comprises four modules: natural forest, soils, non-carbon and joint calculations. Any of these modules can be run independently or as a group. The results are provided as ‘views’ to the user at the completion of the run.

To activate a module, the user selects the module to run within the CRA, the version of the data set to be used, the model version and other calculation parameters. The natural forest and soil carbon modules use R statistical language as the base program language, while the non-carbon module and joint calculations module are developed in the programming language C Sharp (C#).

Within the joint calculations module, the user has the option of using the carbon results from running the modules or using default carbon estimates (based on published reports) stored within the Gateway. The joint calculations module combines the carbon estimates with the land use area to calculate carbon stock and change following the methodology set out in section 2.3 of volume 4 of the 2006 IPCC Guidelines for National Greenhouse Gas Inventories (IPCC, 2006). The results represent carbon stock and change for every ‘from’ and ‘to’ land use combination outlined by the IPCC since 1990.

On completion of running a module, the results can be saved or published back to the Gateway. This provides a versioned and auditable record of the results used for reporting. If the results are saved or published, other information such as the time created, the user’s identification and the module-particular parameters that were used are also saved for tracking and audit control.

The CRA is maintained and supported by Interpine Innovation, a New Zealand-based company that specialises in forestry inventories and related information technology development. Interpine Innovation also provides support services, such as database and application backups, day-to-day issue resolution and enhancement projects to the Gateway or CRA as required.

Any changes to the data or table structure within the Gateway, or to the people accessing the Gateway or CRA, are tracked via audit logs. For any changes to the data within the Gateway, the person making the change, the date, the reason for change and the version are logged and reports are made available to users for review.

#### Document management

All reference material, including scientific reports containing information on methodologies or emission factors used in the production of the LULUCF and Kyoto Protocol estimates, is archived on the Ministry for the Environment’s document management store, Te Puna.

The emission factors and area estimates for published runs are also archived within the Gateway and can be accessed via the Gateway or the CRA.

## Annex 3: References

Some references may be downloaded directly from the following webpage: www.mpi.govt.nz/news-and-resources/statistics-and-forecasting/greenhouse-gas-reporting/agriculture-greenhouse-gas-inventory-reports.

The Ministry for Primary Industries is progressively making reports used for the inventory available on this page, provided copyright permits.

Carran RA, Theobold PW, Evans JP. 1995. Emissions of nitrous oxide from some grazed pasture soils. New Zealand and Australian Journal of Soil Research 33: 341–352.

de Klein CAM, Barton L, Sherlock RR, Li Z, Littlejohn RP. 2003. Estimating a nitrous oxide emission factor for animal urine from some New Zealand pastoral soils. Australian Journal of Soil Research 41: 381–399.

Fick J, Saggar S, Hill J, Giltrap D. 2011. Poultry Management in New Zealand: Production, Manure Management and Emissions Estimations for the Commercial Chicken, Turkey, Duck and Layer Industries within New Zealand. Report prepared for the Ministry of Agriculture and Forestry by Poultry Industry Association, Egg Producers Association, Landcare Research and Massey University. Wellington: Ministry of Agriculture and Forestry.

Hill J. 2012. Recalculate Pork Industry Emissions Inventory. Report prepared for the Ministry of Agriculture and Forestry by Massey University and the New Zealand Pork Industry Board. Wellington: Ministry of Agriculture and Forestry.

IPCC. 1996. Houghton JT, Meira Filho LG, Lim B, Treanton K, Mamaty I, Bonduki Y, Griggs DJ, Callender BA (eds). IPCC/OECD/IEA. Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories. Bracknell: United Kingdom Meteorological Office.

IPCC. 2000. Penman J, Kruger D, Galbally I, Hiraishi T, Nyenzi B, Emmanuel S, Buendia L, Hoppaus R, Martinsen T, Meijer J, Miwa K, Tanabe K (eds). Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. IPCC National Greenhouse Gas Inventories Programme. Japan: Published for the IPCC by the Institute for Global Environmental Strategies.

IPCC. 2006. Eggleston HS, Buendia L, Miwa K, Ngara T, Tanabe K (eds). 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 4. Agriculture, Forestry and Other Land Use. IPCC National Greenhouse Gas Inventories Programme. Japan: Published for the IPCC by the Institute for Global Environmental Strategies.

Kelliher FM, de Klein CAM. Unpublished. Review of New Zealand’s Fertiliser Nitrous Oxide Emission Factor (EF1) Data. Report prepared for the Ministry for the Environment by Landcare Research and AgResearch in 2006.

Lassey K. 2011. Methane Emissions and Nitrogen Excretion Rates for New Zealand Goats. Report for the Ministry of Agriculture and Forestry, National Institute of Water and Atmospheric Research. Wellington: National Institute of Water and Atmospheric Research.

Luo J, van der Weerden T, Hoogendoorn C, de Klein C. 2009. Determination of the N2O Emission Factor for Animal Dung Applied in Spring in Three Regions of New Zealand. Report prepared for the Ministry of Agriculture and Forestry by AgResearch. Wellington: Ministry of Agriculture and Forestry.

Ministry for the Environment. 2010. New Zealand’s Greenhouse Gas Inventory 1990–2008. Wellington: Ministry for the Environment.

Muller C, Sherlock RR, Williams PH. 1995. Direct field measurements of nitrous oxide emissions from urine-affected and urine-unaffected pasture in Canterbury. In: Proceedings of the Workshop on Fertilizer Requirements of Grazed Pasture and Field Crops: Macro and Micronutrients. Currie LD, Loganathan P (eds). Occasional Report No. 8. Palmerston North: Massey University. pp 243–34.

Pickering A and Fick JM. 2015. Detailed methodologies for agricultural greenhouse gas emission calculation. Version 3. Wellington: Ministry for Primary Industries. (Refer to: http://mpi.govt.nz/news-and-resources/statistics-and-forecasting/greenhouse-gas-reporting.)

Sherlock RR, Jewell P, Clough T. 2008. Review of New Zealand Specific FracGASM and FracGASF Emissions Factors. Report prepared for the Ministry of Agriculture and Forestry by Landcare Research and AgResearch. Wellington: Ministry of Agriculture and Forestry.

Suttie J. 2012. Report to the Deer Industry New Zealand: Estimation of Deer Population and Productivity Data 1990 to 2012. Dunedin: Suttie Consulting Limited.

Thomas S, Hume E, Fraser T, Curtin, D. 2011. Factors and Activity Data to Estimate Nitrous Oxide Emissions from Cropping Systems, and Stubble and Tussock Burning. Report prepared for Ministry of Agriculture and Forestry by Plant and Food Research. Wellington: Ministry of Agriculture and Forestry.

Thomas S, Wallace D, Beare M. 2014. Pasture Renewal Activity Data and Factors for New Zealand. Report prepared for the Ministry for Primary Industries by Plant and Food Research. Wellington: Ministry for Primary Industries.

Thomas SM, Fraser T, Curtin D, Brown H, Lawrence E. 2008. Review of Nitrous Oxide Emission Factors and Activity Data for Crops. Report prepared for the Ministry of Agriculture and Forestry by Plant and Food Research. Wellington: Ministry of Agriculture and Forestry.

Thomas SM, Ledgard SF, Francis GS. Unpublished. Appropriateness of IPCC Default Values for Estimating New Zealand’s Indirect Nitrous Oxide Emissions. Report prepared for the Ministry of Agriculture and Forestry in 2003.

Thomas SM, Ledgard SF, Francis GS. 2005. Improving estimates of nitrate leaching for quantifying New Zealand’s indirect nitrous oxide emissions. Nutrient Cycling in Agroecosystems 73: 213–226.

van der Weerden T, Cox N, Luo J, Di HJ, Podolyan A, Phillips RL, Saggar S, de Klein CAM, Ettema P, Rys G. 2016. Refining the New Zealand nitrous oxide emission factor for urea fertiliser and farm dairy effluent. Agriculture Ecosystems & Environment 222: 133–137.

# Annex 4: Methodology and data collection for estimating emissions from fossil fuel combustion

New Zealand emission factors are based on gross calorific value. Energy activity data and emission factors in New Zealand are conventionally reported in gross (higher heating value) terms, with some minor exceptions. The convention adopted by New Zealand to convert gross calorific value to net calorific value follows the Organisation for Economic Co-operation and Development and International Energy Agency assumptions:

Net calorific value = 0.95 × gross calorific value for coal and liquid fuels

Net calorific value = 0.90 × gross calorific value for gas

Net calorific value = 0.80 × gross calorific value for wood

Emission factors for gas, coal, biomass and liquid fuels used by New Zealand are shown in tables A4.1–A4.4. Where Intergovernmental Panel on Climate Change (IPCC) default emission factors are used, a net-to-gross factor as above is used to account for New Zealand activity data representing gross energy figures:

Gross EF = Net EF × Factor

Table A4.1 	Gross carbon dioxide emission factors used for New Zealand’s energy sector in 2017

|  | Emission factor (t CO2/TJ) | Source |
| --- | --- | --- |
| Gas |  |  |
| Maui | 52.26 | 1 |
| Kapuni | 53.57 | 1 |
| McKee | 54.40 | 2 |
| Kaimiro | 63.51 | 2 |
| Ngatoro | 63.51 | 2 |
| TAWN | 52.72 | 2 |
| Mangahewa | 54.67 | 2 |
| Turangi | 55.15 | 2 |
| Pohokura | 54.81 | 1 |
| Rimu/Kauri | 51.97 | 2 |
| Maari | 54.71 | 2 |
| Weighted Average | 54.07 |  |
| Kapuni LTS | 85.13 | 1 |
| Methanol – Mixed Feed – to 94 | 62.44 | 2 |
| Methanol – LTS – to 94 | 83.97 | 2 |
| Liquid fuels |  |  |
| Crude oil | 69.67 | 4 |
| Regular petrol | 66.70 | 3 |
| Petrol – premium | 66.94 | 3 |
| Diesel (10 parts (sulphur) per million) | 69.31 | 3 |
| Jet kerosene | 68.22 | 3 |
| Av gas | 65.89 | 3 |
| LPG | 60.43 | 6 |
| Heavy fuel oil | 73.59 | 3 |
| Light fuel oil | 72.30 | 3 |
| Bitumen (asphalt) | 76.90 | 3 |
| Biomass |  |  |
| Biogas | 49.17 | 4 |
| Wood (industrial) | 89.47 | 4 |
| Bioethanol | 64.20 | 5 |
| Biodiesel | 67.26 | 4 |
| Wood (residential) | 89.47 | 4 |
| Coal |  |  |
| All sectors excl. electricity (sub-bituminous) | 91.99 | 6 |
| All sectors (bituminous) | 89.13 | 6 |
| All sectors (lignite) | 93.11 | 6 |

1.	New Zealand Emissions Trading Scheme data.

2.	Specific gas field operator.

3.	Refining NZ.

4.	IPCC Guidelines (2006).

5.	New Zealand Energy Information Handbook (Eng et al., 2008).

6.	Review of Default Emissions Factors in Draft Stationary Energy and Industrial Processes Regulations: Coal (CRL Energy, 2009).

Table A4.2 	Consumption-weighted average emission factors used for New Zealand’s
sub-bituminous coal-fired electricity generation for 1990 to 2017

| Year | Emission factor (t CO2/TJ) |
| --- | --- |
| 1990 | 91.20 |
| 1991 | 91.24 |
| 1992 | 91.29 |
| 1993 | 91.33 |
| 1994 | 91.38 |
| 1995 | 91.42 |
| 1996 | 91.47 |
| 1997 | 91.51 |
| 1998 | 91.56 |
| 1999 | 91.60 |
| 2000 | 91.64 |
| 2001 | 91.69 |
| 2002 | 91.73 |
| 2003 | 91.78 |
| 2004 | 91.82 |
| 2005 | 91.87 |
| 2006 | 91.91 |
| 2007 | 92.43 |
| 2008 | 92.31 |
| 2009 | 92.39 |
| 2010 onwards | 92.20 |

Table A4.3 	Methane emission factors used for New Zealand’s energy sector for 1990 to 2017

|  | Emission factor (t CH4/PJ) | Source |
| --- | --- | --- |
| Natural gas |  |  |
| Electricity industries | 0.9 | IPCC 2006 (table 2.2) |
| Commercial | 4.50 | IPCC 2006 (table 2.4) |
| Residential | 4.50 | IPCC 2006 (table 2.5) |
| Domestic transport (CNG) | 82.80 | IPCC 2006 (table 3.2.2) |
| Other stationary (mainly industrial) | 0.9 | IPCC 2006 (table 2.3) |
| Liquid fuels |  |  |
| Stationary sources |  |  |
| Electricity – residual oil | 2.85 | IPCC 2006 (table 2.2) |
| Industrial (including refining) – residual oil | 2.85 | IPCC 2006 (table 2.3) |
| Industrial – LPG | 0.95 | IPCC 2006 (table 2.3) |
| Commercial – residual oil | 9.50 | IPCC 2006 (table 2.4) |
| Commercial – distillate oil | 9.50 | IPCC 2006 (table 2.4) |
| Commercial – LPG | 4.75 | IPCC 2006 (table 2.4) |
| Residential – distillate oil | 9.50 | IPCC 2006 (table 2.5) |
| Residential – LPG | 4.75 | IPCC 2006 (table 2.5) |
| Agriculture – stationary | 2.85 | IPCC 2006 (table 2.5) |
| Mobile sources |  |  |
| LPG | 58.9 | IPCC 2006 (table 3.2.2) |
| Petrol | 28.05 | IPCC 2006 (table 3.2.2) |
| Diesel | 3.71 | IPCC 2006 (table 3.2.2) |
| Navigation (fuel oil and diesel) | 6.65 | IPCC 2006 (table 3.5.3) |
| Aviation fuel/kerosene | 0.48 | IPCC 2006 (table 3.6.5) |
| Coal |  |  |
| Electricity generation | 0.95 | IPCC 2006 (table 2.2) |
| Industry | 9.50 | IPCC 2006 (table 2.3) |
| Commercial | 9.50 | IPCC 2006 (table 2.4) |
| Residential | 285.00 | IPCC 2006 (table 2.5) |
| Biomass |  |  |
| Wood/wood waste | 24 | IPCC 2006 (table 2.3) |
| Wood – fireplaces | 240.00 | IPCC 2006 (table 2.5) wood – residential |
| Bioethanol | 18.00 | IPCC 2006 (table 3.2.2) – ethanol, cars, Brazil |
| Biodiesel | 18.00 | IPCC 2006 (table 3.2.2) – ethanol, cars, Brazil |
| Gas biomass | 0.9 | IPCC 2006 (table 2.2) |

Table A4.4 	Nitrous oxide emission factors used for New Zealand’s energy sector for 1990 to 2017

|  | Emission factor (t N2O/PJ) | Source |
| --- | --- | --- |
| Natural gas |  |  |
| Electricity generation | 0.09 | IPCC 2006 (table 2.2) |
| Commercial | 0.09 | IPCC 2006 (table 2.4) |
| Residential | 0.09 | IPCC 2006 (table 2.5) |
| Domestic transport (CNG) | 2.70 | IPCC 2006 (table 3.2.2) |
| Other stationary (mainly industrial) | 0.09 | IPCC 2006 (table 2.3) |
| Liquid fuels |  |  |
| Stationary sources |  |  |
| Electricity – residual oil | 0.57 | IPCC 2006 (table 2.2) |
| Electricity – distillate oil | 0.57 | IPCC 2006 (table 2.2) |
| Industrial (including refining) – residual oil | 0.57 | IPCC 2006 (table 2.2) |
| Industrial – distillate oil | 0.57 | IPCC 2006 (table 2.3) |
| Commercial – residual oil | 0.57 | IPCC 2006 (table 2.4) |
| Commercial – distillate oil | 0.57 | IPCC 2006 (table 2.4) |
| Residential (all oil) | 0.57 | IPCC 2006 (table 2.5) |
| LPG (all uses) | 0.095 | IPCC 2006 (tables 2.2 – 2.5) |
| Agriculture – stationary | 0.38 | Tier 2, diesel engines – agriculture |
| Mobile sources |  |  |
| LPG | 0.19 | IPCC 2006 (table 3.22) |
| Petrol | 7.6 | IPCC 2006 (table 3.2.2) |
| Diesel | 3.71 | IPCC 2006 (table 3.2.2) |
| Fuel oil (ships) | 1.90 | IPCC 2006 (table 3.5.3) |
| Aviation fuel/kerosene | 1.90 | IPCC 2006 (table 3.6.5) |
| Coal |  |  |
| Electricity generation | 1.43 | IPCC 2006 (table 2.2) |
| Industry | 1.43 | IPCC 2006 (table 2.3) |
| Commercial | 1.43 | IPCC 2006 (table 2.4) |
| Residential | 1.43 | IPCC 2006 (table 2.5) |
| Biomass |  |  |
| Wood (all uses) | 3.20 | IPCC 2006 (table 2.5) wood/wood waste |
| Gas biomass | 0.09 | IPCC 2006 (table 2.5) |

## A4.1	Emissions from liquid fuels

### A4.1.1	Activity data and uncertainties

The Delivery of Petroleum Fuels by Industry Survey is conducted by the Ministry of Business, Innovation and Employment (MBIE). Because it is a census, there is no sampling error. The only possible sources of error are non-sample errors (such as respondent error and processing error). The 2017 statistical difference for liquid fuels in the balance table of the publication Energy in New Zealand (MBIE, 2018) was 1.2 per cent. This is used as the activity data uncertainty for liquid fuels in 2017.

### A4.1.2	Emission factors and uncertainties

The carbon dioxide (CO2) emission factors are described in table A4.1. Table A4.5 shows a complete time series of gross calorific values, while table A4.6 shows a complete time series of carbon content of liquid fuels. This information is supplied by Refining New Zealand Ltd and is used in the calculation of annual emission factors for liquid fuels.

A 2009 consultant report (Hale and Twomey, unpublished) to the Ministry for the Environment estimates the uncertainty of CO2 emission factors for liquid fuels at ±0.5 per cent. The uncertainty for methane and nitrous oxide emission factors is ±50.0 per cent because almost all emission factors are IPCC defaults.

Table A4.5 	Gross calorific values (MJ/kg) for liquid fuels for 1990 to 2017

|  | Premium petrol | Regular petrol | Diesel | Jet kerosene | Heavy fuel oil | Light fuel oil | Bitumen (asphalt) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | 47.24 | 47.22 | 45.76 | 46.37 | 43.07 | 44.12 | 41.30 |
| 1991 | 47.17 | 47.17 | 45.73 | 46.38 | 43.02 | 44.07 | 41.30 |
| 1992 | 47.18 | 47.14 | 45.75 | 46.41 | 43.03 | 44.14 | 41.30 |
| 1993 | 47.09 | 47.14 | 45.74 | 46.36 | 43.01 | 44.13 | 41.31 |
| 1994 | 47.10 | 47.11 | 45.75 | 46.34 | 43.03 | 44.16 | 41.30 |
| 1995 | 47.07 | 47.14 | 45.59 | 46.31 | 43.03 | 44.01 | 41.30 |
| 1996 | 46.91 | 47.14 | 45.54 | 46.26 | 43.00 | 43.98 | 41.30 |
| 1997 | 46.93 | 47.17 | 45.58 | 46.32 | 42.92 | 43.92 | 41.30 |
| 1998 | 46.89 | 47.12 | 45.64 | 46.27 | 43.06 | 44.02 | 41.27 |
| 1999 | 46.92 | 47.13 | 45.56 | 46.29 | 43.09 | 43.93 | 41.28 |
| 2000 | 46.91 | 47.12 | 45.58 | 46.22 | 43.07 | 43.90 | 41.27 |
| 2001 | 46.92 | 47.15 | 45.64 | 46.25 | 43.08 | 43.96 | 41.27 |
| 2002 | 46.90 | 47.16 | 45.62 | 46.29 | 43.03 | 43.84 | 41.26 |
| 2003 | 46.87 | 47.11 | 45.61 | 46.23 | 43.06 | 43.79 | 41.27 |
| 2004 | 46.91 | 47.10 | 45.59 | 46.25 | 43.04 | 43.90 | 41.30 |
| 2005 | 46.95 | 47.10 | 45.73 | 46.28 | 43.11 | 43.94 | 41.30 |
| 2006 | 46.97 | 47.09 | 45.79 | 46.23 | 42.93 | 43.68 | 41.30 |
| 2007 | 46.97 | 47.10 | 45.77 | 46.23 | 42.97 | 43.72 | 41.30 |
| 2008 | 46.93 | 47.06 | 45.72 | 46.19 | 42.86 | 43.72 | 41.30 |
| 2009 | 46.95 | 47.03 | 45.72 | 46.17 | 42.89 | 43.75 | 41.29 |
| 2010 | 46.96 | 47.03 | 45.69 | 46.17 | 42.95 | 43.70 | 41.29 |
| 2011 | 46.96 | 47.04 | 45.69 | 46.19 | 42.89 | 43.72 | 41.27 |
| 2012 | 46.98 | 47.03 | 45.66 | 46.18 | 43.03 | 43.71 | 41.27 |
| 2013 | 46.99 | 47.05 | 45.71 | 46.23 | 43.05 | 43.84 | 41.26 |
| 2014 | 46.95 | 47.02 | 45.71 | 46.23 | 42.94 | 43.73 | 41.26 |
| 2015 | 46.96 | 47.03 | 45.67 | 46.19 | 42.98 | 43.70 | 41.28 |
| 2016 | 46.92 | 46.99 | 45.79 | 46.29 | 42.99 | 43.91 | 41.29 |
| 2017 | 46.92 | 46.99 | 45.79 | 46.29 | 42.99 | 43.91 | 41.29 |

Table A4.6 	Carbon content (per cent mass) for liquid fuels for 1990 to 2017

|  | Premium petrol | Regular petrol | Diesel | Jet kerosene | Heavy fuel oil | Light fuel oil | Bitumen (asphalt) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | 84.87 | 84.92 | 86.28 | 85.92 | 86.22 | 86.67 | 86.57 |
| 1991 | 85.04 | 85.04 | 86.33 | 85.89 | 86.26 | 86.30 | 86.57 |
| 1992 | 85.03 | 85.13 | 86.29 | 85.84 | 86.25 | 86.18 | 86.57 |
| 1993 | 85.25 | 85.13 | 86.32 | 85.94 | 86.27 | 86.20 | 86.56 |
| 1994 | 85.21 | 85.19 | 86.30 | 85.99 | 86.25 | 86.13 | 86.57 |
| 1995 | 85.30 | 85.13 | 86.63 | 86.05 | 86.25 | 86.39 | 86.57 |
| 1996 | 85.66 | 85.13 | 86.73 | 86.16 | 86.28 | 86.45 | 86.57 |
| 1997 | 85.63 | 85.04 | 86.64 | 86.04 | 86.35 | 86.55 | 86.58 |
| 1998 | 85.72 | 85.17 | 86.52 | 86.14 | 86.22 | 86.39 | 86.63 |
| 1999 | 85.65 | 85.15 | 86.69 | 86.10 | 86.20 | 86.53 | 86.63 |
| 2000 | 85.67 | 85.16 | 86.64 | 86.25 | 86.22 | 86.58 | 86.63 |
| 2001 | 85.65 | 85.09 | 86.53 | 86.18 | 86.21 | 86.49 | 86.64 |
| 2002 | 85.68 | 85.06 | 86.57 | 86.10 | 86.25 | 86.68 | 86.66 |
| 2003 | 85.76 | 85.19 | 86.58 | 86.23 | 86.23 | 86.76 | 86.63 |
| 2004 | 85.66 | 85.22 | 86.62 | 86.20 | 86.24 | 86.58 | 86.58 |
| 2005 | 85.58 | 85.22 | 86.62 | 86.12 | 86.18 | 86.52 | 86.57 |
| 2006 | 85.54 | 85.25 | 86.57 | 86.24 | 86.34 | 86.93 | 86.57 |
| 2007 | 85.54 | 85.23 | 86.61 | 86.24 | 86.30 | 86.87 | 86.57 |
| 2008 | 85.63 | 85.32 | 86.70 | 86.32 | 86.39 | 86.87 | 86.57 |
| 2009 | 85.56 | 85.38 | 86.72 | 86.36 | 86.37 | 86.83 | 86.60 |
| 2010 | 85.54 | 85.40 | 86.77 | 86.35 | 86.31 | 86.90 | 86.59 |
| 2011 | 85.55 | 85.37 | 86.78 | 86.32 | 86.37 | 86.87 | 86.64 |
| 2012 | 85.51 | 85.38 | 86.84 | 86.34 | 86.25 | 86.89 | 86.63 |
| 2013 | 85.49 | 85.35 | 86.73 | 86.22 | 86.24 | 86.68 | 86.65 |
| 2014 | 85.57 | 85.42 | 86.74 | 86.23 | 86.33 | 86.87 | 86.65 |
| 2015 | 85.54 | 85.40 | 86.81 | 86.33 | 86.30 | 86.90 | 86.62 |
| 2016 | 85.66 | 85.48 | 86.56 | 86.11 | 86.28 | 86.58 | 86.60 |
| 2017 | 85.66 | 85.48 | 86.56 | 86.11 | 86.28 | 86.58 | 86.60 |

Table A4.7 	Emission factors for European gasoline and diesel vehicles – COPERT IV model (European Environment Agency, 2007)

|  | N2O emission factors (mg/km) |  |  |  | CH4 emission factors (mg/km) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Urban |  | Rural | Highway | Urban |  | Rural | Highway |
|  | Cold | Hot |  |  | Cold | Hot |  |  |
| Passenger car |  |  |  |  |  |  |  |  |
| Gasoline |  |  |  |  |  |  |  |  |
| pre-Euro | 10 | 10 | 6.5 | 6.5 | 201 | 131 | 86 | 41 |
| Euro 1 | 38 | 22 | 17 | 8 | 45 | 26 | 16 | 14 |
| Euro 2 | 24 | 11 | 4.5 | 2.5 | 94 | 17 | 13 | 11 |
| Euro 3 | 12 | 3 | 2 | 1.5 | 83 | 3 | 2 | 4 |
| Euro 4 | 6 | 2 | 0.8 | 0.7 | 57 | 2.87 | 2.69 | 5.08 |
| Diesel |  |  |  |  |  |  |  |  |
| pre-Euro | 0 | 0 | 0 | 0 | 22 | 28 | 12 | 8 |
| Euro 1 | 0 | 2 | 4 | 4 | 18 | 11 | 9 | 3 |
| Euro 2 | 3 | 4 | 6 | 6 | 6 | 7 | 3 | 2 |
| Euro 3 | 15 | 9 | 4 | 4 | 3 | 3 | 0 | 0 |
| Euro 4 | 15 | 9 | 4 | 4 | 1.1 | 1.1 | 0 | 0 |
| LPG |  |  |  |  |  |  |  |  |
| pre-ECE | 0 | 0 | 0 | 0 | 80 | 80 | 35 | 25 |
| Euro 1 | 38 | 21 | 13 | 8 | 80 | 80 | 35 | 25 |
| Euro 2 | 23 | 13 | 3 | 2 | 80 | 80 | 35 | 25 |
| Euro 3 and later | 9 | 5 | 2 | 1 | 80 | 80 | 35 | 25 |
| Light duty vehicles |  |  |  |  |  |  |  |  |
| Gasoline |  |  |  |  |  |  |  |  |
| pre-Euro | 10 | 10 | 6.5 | 6.5 | 201 | 131 | 86 | 41 |
| Euro 1 | 122 | 52 | 52 | 52 | 45 | 26 | 16 | 14 |
| Euro 2 | 62 | 22 | 22 | 22 | 94 | 17 | 13 | 11 |
| Euro 3 | 36 | 5 | 5 | 5 | 83 | 3 | 2 | 4 |
| Euro 4 | 16 | 2 | 2 | 2 | 57 | 2 | 2 | 0 |
| Diesel |  |  |  |  |  |  |  |  |
| pre-Euro | 0 | 0 | 0 | 0 | 22 | 28 | 12 | 8 |
| Euro 1 | 0 | 2 | 4 | 4 | 18 | 11 | 9 | 3 |
| Euro 2 | 3 | 4 | 6 | 6 | 6 | 7 | 3 | 2 |
| Euro 3 | 15 | 9 | 4 | 4 | 3 | 3 | 0 | 0 |
| Euro 4 | 15 | 9 | 4 | 4 | 1.1 | 1.1 | 0 | 0 |
| Heavy duty truck and bus |  |  |  |  |  |  |  |  |
| Gasoline – all technologies | 6 | 6 | 6 | 6 | 140 | 140 | 110 | 70 |
| Diesel |  |  |  |  |  |  |  |  |
| GVW<16t | 30 | 30 | 30 | 30 | 85 | 85 | 23 | 20 |
| GVW>16t | 30 | 30 | 30 | 30 | 175 | 175 | 80 | 70 |
| Urban busses and coaches | 30 | 30 | 30 | 30 | 175 | 175 | 80 | 70 |
| CNG |  |  |  |  |  |  |  |  |
| pre-Euro 4 |  |  |  |  | 5,400 | 5,400 | 5,400 | 5,400 |
| Euro 4 and later |  |  |  |  | 900 | 900 | 900 | 900 |
| Power two wheeler |  |  |  |  |  |  |  |  |
| Gasoline |  |  |  |  |  |  |  |  |
| <50 cm3 | 1 | 1 | 1 | 1 | 219 | 219 | 219 | 219 |
| >50 cm3 2 stroke | 2 | 2 | 2 | 2 | 150 | 150 | 150 | 150 |
| >50 cm3 4 stroke | 2 | 2 | 2 | 2 | 200 | 200 | 200 | 200 |

## A4.2	Emissions from solid fuels

### A4.2.1	Activity data and uncertainties

The New Zealand Quarterly Statistical Return of Coal Production and Sales conducted by MBIE has near coverage of the sector, meaning that sampling error is small. The only other possible sources of error are non-sample errors (such as respondent error and processing error). The 2017 statistical difference for solid fuels in the balance table of the publication Energy in New Zealand (MBIE, 2018) was 4.0 per cent. This is used as the activity data uncertainty for solid fuels in 2017.

### A4.2.2	Emission factors and uncertainties

The estimated uncertainty in CO2 emission factors for solid fuels is ±2.2 per cent. This is based on the difference between the range of updated emission factors for the three different ranks of coal used in New Zealand. The uncertainty for methane and nitrous oxide emission factors is ±50.0 per cent because almost all emission factors are IPCC defaults.

## A4.3	Emissions from gaseous fuels

### A4.3.1	Activity data

Through the various surveys and information collected by MBIE, it has full coverage of the natural gas sector. This means that there is no sampling error in natural gas statistics and the only possible sources of error include those such as respondent error and processing error. The 2017 statistical difference for gaseous fuels in the balance table of the publication Energy in New Zealand (MBIE, 2018) was 0.3 per cent. This is used as the activity data uncertainty for gaseous fuels in 2017.

### A4.3.2	Emission factors

The estimated uncertainty in CO2 emission factors for gaseous fuels is ±2.8 per cent. This is based on the difference between the range of emission factors for three large gas fields in New Zealand. Together, these gas fields made up over 55 per cent of New Zealand’s total gas supply in 2017. The uncertainty for methane and nitrous oxide emission factors is ±50.0 per cent because almost all emission factors are IPCC defaults.

## A4.4	Energy balance

Detailed and up-to-date energy balance tables for New Zealand are available online: www.mbie.govt.nz/info-services/sectors-industries/energy/energy-data-modelling/statistics/energy-balances.

Further information can be found within the publication Energy in New Zealand (MBIE, 2018), also available online: www.mbie.govt.nz/info-services/sectors-industries/energy/energy-data-modelling/publications/energy-in-new-zealand.

Table A.4.8 gives a time series of energy use versus non-energy use of natural gas.

Table A4.8 	Split of energy use and non-energy use of natural gas in petajoules

|  | Energy use | Non-energy use |
| --- | --- | --- |
| 1990 | 129.5 | 14.2 |
| 1991 | 143.9 | 22.1 |
| 1992 | 152.6 | 18.8 |
| 1993 | 148.0 | 21.1 |
| 1994 | 137.7 | 25.8 |
| 1995 | 127.4 | 36.2 |
| 1996 | 147.7 | 47.5 |
| 1997 | 170.4 | 48.9 |
| 1998 | 146.2 | 46.6 |
| 1999 | 168.5 | 54.2 |
| 2000 | 173.9 | 61.8 |
| 2001 | 190.6 | 55.4 |
| 2002 | 177.1 | 57.8 |
| 2003 | 151.9 | 26.1 |
| 2004 | 129.8 | 32.1 |
| 2005 | 136.4 | 13.0 |
| 2006 | 137.2 | 15.0 |
| 2007 | 148.6 | 15.4 |
| 2008 | 137.1 | 18.4 |
| 2009 | 133.6 | 25.5 |
| 2010 | 148.9 | 25.6 |
| 2011 | 132.2 | 24.5 |
| 2012 | 145.0 | 32.0 |
| 2013 | 146.1 | 40.3 |
| 2014 | 149.7 | 60.7 |
| 2015 | 141.5 | 51.4 |
| 2016 | 133.7 | 59.1 |
| 2017 | 142.9 | 53.8 |

## A4.5	Carbon dioxide reference approach for the Energy sector

### A4.5.1	Estimation of carbon dioxide using the IPCC reference approach

The reference approach uses a country’s energy supply data to calculate the CO2 emissions from the combustion of fossil fuels using the apparent consumption equation. The apparent consumption in the reference approach is derived from production, import and export data. This information is included as a check for combustion-related emissions calculated from the sectoral approach.

The apparent consumption for primary fuels in the reference approach is obtained from ‘calculated’ energy-use figures (see annex 2 and section A4.4). These are derived as a residual figure from an energy balance equation comprising production, imports, exports, stock change and international transport on the supply side according to the IPCC Guidelines (IPCC, 2006).

The majority of the CO2 emission factors for the reference approach are specific to New Zealand. Most emission factors for liquid fuels are based on annual carbon content and the gross calorific value data provided by New Zealand’s only oil refinery, Refining New Zealand Ltd. Where these data are not available, an IPCC default is used. The natural gas emission factor is based on a production-derived, weighted average of emission factors from all gas production fields. The CO2 emission factors for solid fuels were updated for the 2014 inventory submission following analysis to verify default emission factors used for the New Zealand Emissions Trading Scheme. For more information on this improvement, see chapter 3, section 3.3.2.

#### Solid fuels in iron and steel manufacture

As mentioned in chapter 3, section 3.2.3, some of the coal production activity data in the reference approach are used in steel production. The Industrial Processes and Product Use sector accounts for the CO2 emissions from this coal in the sectoral approach, as recommended by the IPCC Guidelines (IPCC, 2006); therefore they are not included in the common reporting format table 1.AA Fuel combustion – sectoral approach.

For simplicity, all feedstock carbon is excluded from the reference approach according to the IPCC Guidelines (IPCC, 2006). Without taking into account the use of by-product gases, this can create some discrepancies between the reference and sectoral approaches.

### A4.5.2	Comparison of the IPCC reference approach with the New Zealand sectoral methodology

For 2017, CO2 emissions estimated with the sectoral approach were 2.2 per cent lower than those estimated with the reference approach. Figure A4.1 shows the results for the two approaches for the period 1990–2017.

In some years, differences exist between the reference and sectoral approaches. Much of this is due to the statistical differences found in the energy balance tables (MBIE, 2018) that are used as the basis for the reference and sectoral approach. Since 2000, the standard of national energy data has improved significantly, due to increased resources and focus. In 2008, Statistics New Zealand delegated responsibility for the collection and analysis of national energy data to MBIE. Before 2008, various energy statistics were collected by Statistics New Zealand or MBIE. The change resulted in a more consistent and transparent approach to energy data collection because one agency collected data across the supply chain.

Figure A4.1 	Reference and sectoral approach carbon dioxide by fuel type (kt CO2)

###### Sources of differences

For gaseous fuels, the field-specific emission factors are used for natural gas supplied for industrial processes, while the reference approach uses an average emission factor.

For liquid fuels, the energy balance is mass balanced but not carbon balanced. The fuel category ‘other oil’ is an aggregation of several fuel types, and so it is difficult to quantify a reliable carbon emission factor for the reference approach.

In the sectoral approach, sector- or even plant-specific calorific values are used to calculate energy consumption, whereas in the reference approach, average (country-specific) calorific values are applied.

## Annex 4: References

CRL Energy Ltd. 2009. Reviewing Default Emission Factors in Draft Stationary Energy and Industrial Processes. Contract report prepared for the Ministry for the Environment. Wellington: Ministry for the Environment. Retrieved from www.climatechange.govt.nz/consultation/ energy/review-default-emissions-factors.pdf (March 2016).

Eng G, Bywater I, Hendtlass C. 2008. New Zealand Energy Information Handbook. Christchurch: New Zealand Centre for Advanced Engineering.

European Environment Agency. 2007. EMEP/CORINAIR Emission Inventory Guidebook – 2007. Copenhagen: European Environment Agency.

Hale R, Twomey I. Unpublished. Reviewing default emission factors for liquid fossil fuels adopted by the New Zealand Emissions Trading Scheme. Report by consultants Hale & Twomey Limited prepared for the Ministry for the Environment. Wellington: Ministry for the Environment.

IPCC. 2006. Eggleston HS, Buendia L, Miwa K, Ngara T, Tanabe K (eds). 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 2. Energy. IPCC National Greenhouse Gas Inventories Programme. Japan: Institute for Global Environmental Strategies for IPCC.

Ministry of Business, Innovation and Employment. 2018. Energy in New Zealand 2018. Wellington: Ministry of Business, Innovation and Employment.

# Annex 5: Supplementary information for the KP-LULUCF sector

## A5.1	Technical corrections to the FMRL

### A5.1.1	Introduction

For the second commitment period, reporting on Forest management under the Kyoto Protocol is mandatory. Accounting for Forest management during the second commitment period is relative to a forest management reference level (FMRL) (Decision 2/CMP.7, UNFCCC, 2012).

New Zealand’s FMRL, first submitted in 2009 and revised in 2011, is set at 11.15 million tonnes carbon dioxide equivalent (Mt CO2-e) per year for the period 2013 to 2020 (New Zealand Government, 2011). This value was constructed using a business-as-usual projection of pre-1990 planted forest growth and harvest for the period 2013 to 2020. It was based on yield tables and statistics on the area in each age class of pre-1990 planted forest from the National Exotic Forest Description (NEFD) as at 2009 (Ministry for Primary Industries, 2014).

The 2011 FMRL included the following assumptions:

pre-1990 natural forests were in steady state

no pre-1990 planted forest deforestation would occur between 2013 and 2020 (pre-1990 natural forests were excluded from the analyses; post-1989 forest deforestation is reported under Article 3.3 – Deforestation)

between 2013 and 2020, 2,000 hectares per year would be converted to non-forest land, and the equivalent forest would be planted elsewhere (i.e., 2,000 hectares per year would be reported as carbon equivalent forest (CEF) and be accounted for under Forest management)

while harvest of post-1989 planted forest will increase over the period, pre-1990 planted forests will still make up a substantial proportion of total forest harvest

all carbon was instantly emitted at the time of harvest (emissions and removals by the Harvested wood products pool were not considered)

no allowance was made for the impacts of potential natural disturbances beyond background levels captured in the carbon stock yield tables.

The FMRL also reflects the following New Zealand legislation (including amendments) and current policies:

the Forest Act 1949, which regulates the removal of timber from natural indigenous forests

the South Island Landless Natives Act 1906, which transferred 17,000 hectares of natural indigenous forest to South Island Māori. The harvesting of this forest is also subject to the Resource Management Act 1991

the Climate Change Response Act 2002, which makes owners of pre-1990 forest who deforest liable for the emissions associated with that activity

the New Zealand’s biofuels policy of the time (under which it was thought most feedstock for biofuel was likely to be derived from non-forest sources).

It was thought that this legislation and these policies would prevent any significant deforestation of pre-1990 forests, and that the New Zealand Emissions Trading Scheme would encourage harvest in pre-1990 planted forests over post-1989 forest.

The 2011 FMRL was determined by modelling the pre-1990 planted forest estate using a Forestry-Oriented Linear Programming Interpreter (FOLPI). As mentioned above, the model developed in FOLPI was based on an age-class distribution of pre-1990 planted forest as at 2009 from the NEFD, and simulated expected harvesting and replanting of this forest. Some additional modelling of decay of residues from harvest events was also carried out in MS Excel.

Since the 2011 FMRL was submitted, supplementary guidance has been prepared that describes the circumstances that would trigger a technical correction to the FMRL (IPCC, 2014). Changes to policies that affect harvest rate (as listed above) cannot be corrected for, but corrections can be made to reflect changes to the method for reporting against the FMRL and to address recommendations made by United Nations Framework Convention on Climate Change (UNFCCC) expert review teams (ERTs).

A technical assessment of New Zealand’s reference level submission was carried out by an ERT in 2011 (UNFCCC, 2011). The ERT noted a number of items for New Zealand to address through either the provision of additional data or through applying technical corrections. These included (UNFCCC, 2011, pp 6–10):

- maintaining consistency in the fraction of harvested biomass instantaneously oxidised when estimating emissions from harvest in the FMRL and in reporting against it (paragraph 21);

- ensuring consistency between the National Inventory Report (NIR) and the FMRL and, therefore, the updating of the current FMRL when new data/information becomes available (paragraph 22);

- making efforts to disaggregate gains and losses by biomass pool (paragraph 35);

- providing further information on how forest owners will be able to move from historic/current harvesting practice to the longer rotation length projected in the FOLPI model (paragraph 36);

- explaining in more detail how the difference in both harvested areas and harvesting age as calculated by FOLPI could be achieved (paragraph 36);

- comparing the results provided in its submission with a rerun of the FOLPI model in which the harvesting of over-mature forests (over 32 years of age) is constrained, and modify the reference level accordingly if necessary (paragraph 36);

- if estimates for natural forests are included in future NIR submissions, making a technical adjustment of the FMRL (paragraph 37);

- agreeing that in the future a technical correction should be made to incorporate the Harvested wood product (HWP) pool (paragraph 38).

### A5.1.2	Technical corrections required

For the 2016 submission, the following technical corrections were made to meet IPCC guidance and address recommendations by the UNFCCC ERT. These aimed to:

- ensure consistency between the method used for greenhouse gas reporting of Forest management and that used to calculate the FMRL (Kyoto Protocol Supplement, IPCC, 2014, sections 2.7.5.2 and 2.7.6). This involves making changes to:

  - align forest area estimates

  - align CEF emissions calculation methods

  - include over planting estimates (pre-1990 natural forest conversions to pre-1990 planted forest)

  - include non-carbon emissions

- include an estimate for pre-1990 natural forest emissions following completion of the re-measurement of the pre-1990 natural forest inventory and subsequent analysis

- address new elements of Decision 2/CMP.7 including:

  - accounting for Harvested wood products

  - the application of the natural disturbances provision.

An additional technical correction has been applied to the FMRL for the 2019 submission, to capture recent improvements to the Harvested wood products pool.

#### Technical correction 1 (2014 National Inventory Report, 2016 submission): Addressing methodological inconsistencies between the 2011 FMRL and Forest management reporting

The first step taken to calculate technical corrections to the FMRL was to replicate the FMRL as submitted in 2011, applying the same policy assumptions, but using the reporting system and historical data that are used to report on Forest management in the inventory.

This technical correction addresses two of the findings of the technical assessment (listed above) by:

- maintaining consistency in the fraction of harvested biomass instantaneously oxidised

- ensuring consistency between the emissions reported in the inventory for Forest management and the FMRL.

This is achieved by using the harvest and deforestation data from 1990 to 2008 from the 2013 inventory (Ministry for the Environment, 2015) as the starting point for the revised projections. Harvesting and deforestation areas for 2009 to 2020, sourced from the Ministry for Primary Industries, are the same as those used for the 2011 FMRL (2009 was the first year of projected data within New Zealand’s 2011 FMRL submission).

Minor adjustments were made to this data as outlined below:

- alignment of forest area estimate to match it to the area of forest that is included under the definition of forest used for UNFCCC reporting

- pre-1990 harvesting data (average harvest age) from 2009 to 2020 have been altered from that used in the 2011 FMRL. The average age at harvest has been adjusted down to 28 years, to address the issue raised by the ERT in its technical assessment of the 2011 FMRL. The area of harvest, however, has been kept the same

- the age-class distribution (as at 2013, based on the 2013 inventory) needed to be altered to ensure enough area was present to maintain the 2011 FMRL harvest rate assumptions. While the result of forcing the harvest profile to match the 2011 FMRL creates an improbable age class, it has limited impact on emissions because average age harvested each year is maintained. The creation of a more realistic age class is an issue New Zealand will look to correct in future technical corrections

- changes to CEF reporting to reflect updated guidance for this reporting released after the 2011 FMRL submission.

##### Aligning forest area estimates

The 2011 FMRL submission was based on data derived from the NEFD (Ministry for Primary Industries, 2014). The NEFD is an annual survey of forest owners that represents the ‘net stocked area’ of the planted production forest estate established with the primary intention of producing wood or fibre. The Land Use and Carbon Analysis System (LUCAS) that is used for reporting emissions for Forest management in the inventory uses complete wall-to-wall mapping to estimate forest area. This means LUCAS maps to a ‘gross stocked area’ where harvested areas, skid sites, forest roads and unstocked gullies are included in the mapped forest area. This gross stocked area is also the basis for the national sampling system used for deriving emission factors for the Forest land use classes. For modelling emissions for reporting under the UNFCCC, LUCAS has isolated the net stocked area from the mapped gross stocked area so the modelled area is compatible between the two data sources (LUCAS and NEFD). The LUCAS gross stocked area of pre-1990 planted forest area is 1.47 million hectares as at 2009. The LUCAS net stocked area is estimated to be 1.25 million hectares (a 12.4 per cent difference). This compares with 1.14 million hectares from the NEFD as at 2009. Because the 2011 FMRL did not take into account differences in the data sources due to the two purposes for which the data are collected, a technical correction is required to correct the original NEFD-based FMRL to the LUCAS mapped area estimates used for reporting for Forest management.

The need for this adjustment extends to estimates of the area of CEF and deforestation, meaning these original net stocked area values need to have an unstocked area component added to them (the same adjustment of 12.4 per cent is used). The harvest areas, however, remain unchanged because both approaches harvest a net stocked area.

The area reported under Forest management will change each year because emissions from deforestation will be reported under Article 3.3 – Deforestation instead. This means that, every year, a technical correction will be required to remove this area and the associated emissions from both Forest management reporting and the FMRL.

##### Harvest data

Pre-1990 planted forest harvesting uses projections from 2009 to 2020 at an average age of (approximately) 28 years (see table A5.1.1). This is to address the issue raised by the ERT that harvest ages in the projection were older than those observed historically and there were no policies in place that would influence rotation length or change the average harvest ages of planted forests. While the average harvest ages used for the technical correction do not match, the harvest areas match the 2011 FMRL harvest areas.

Table A5.1.1 	Pre-1990 planted forest data used to estimate emissions for the technically corrected FMRL

| Year | Pre-1990 planted forest deforestation (kha) | Pre-1990 planted forest harvested (kha) | Pre-1990 planted forest harvest average age (years) |
| --- | --- | --- | --- |
| 1990 | – | 19.369 | 28.0 |
| 1991 | – | 19.883 | 28.0 |
| 1992 | – | 22.639 | 28.0 |
| 1993 | – | 23.275 | 28.0 |
| 1994 | – | 25.000 | 28.0 |
| 1995 | – | 29.275 | 28.0 |
| 1996 | – | 31.250 | 28.0 |
| 1997 | – | 32.175 | 28.0 |
| 1998 | – | 31.575 | 28.0 |
| 1999 | – | 34.075 | 28.0 |
| 2000 | 2.305 | 35.551 | 28.0 |
| 2001 | 2.225 | 39.371 | 28.0 |
| 2002 | 1.616 | 46.149 | 28.0 |
| 2003 | 3.137 | 40.428 | 28.0 |
| 2004 | 6.777 | 33.867 | 28.0 |
| 2005 | 13.186 | 27.198 | 28.0 |
| 2006 | 16.596 | 27.036 | 28.0 |
| 2007 | 22.022 | 22.175 | 28.0 |
| 2008 | 4.103 | 37.243 | 28.0 |
| 2009 | 2.389 | 29.218 | 27.8 |
| 2010 | 2.383 | 33.086 | 28.4 |
| 2011 | 2.396 | 37.479 | 28.2 |
| 2012 | 2.378 | 41.354 | 27.8 |
| 2013 | 2.378 | 46.112 | 27.7 |
| 2014 | 2.247 | 50.021 | 27.8 |
| 2015 | 2.247 | 49.697 | 28.0 |
| 2016 | 2.247 | 49.724 | 28.1 |
| 2017 | 2.247 | 50.018 | 28.5 |
| 2018 | 2.247 | 49.967 | 28.9 |
| 2019 | 2.247 | 45.817 | 29.8 |
| 2020 | 2.247 | 43.817 | 28.9 |

#### Carbon equivalent forests

The method used to calculate the emissions from the application of CEF in the 2011 FMRL was inconsistent with the provisions of Decision 2/CMP.7 (UNFCCC, 2012) and the guidance for reporting (Kyoto Protocol Supplement, IPCC, 2014). The correct method for calculating emissions for CEF is to model the events by applying the same methods as would apply to deforestation and afforestation events but report all emissions (and removals) under Forest management.

##### Carbon equivalent forest harvested and converted

The estimate for carbon equivalent forests harvested and converted (CEFhc) included in the technical correction uses projections of land-use change from 2009 to 2020 at an average age of (approximately) 28 years and at the rate of 2,247 hectares per annum (the net stocked area from the 2011 FMRL of 2,000 hectares, plus an unstocked proportion of 247 hectares, which contains a much lower carbon stock as explained above under ‘Aligning forest area estimates’).

##### Carbon equivalent forest newly established

Carbon equivalent forest newly established (CEFne) land is replanted at an equivalent annual area (2,000 hectares net stocked area plus 247 hectares that is unstocked), and the post-1989 planted forest yield table is applied to the net stocked area. The post-1989 planted forest yield table is deemed appropriate because the new forest is established on Grassland and the history of this newly planted land is most similar to post-1989 planted forest land.

In the technical correction to the FMRL, CEF land is modelled as going to and coming from the three Grassland types (low producing grassland, high producing grassland and grassland with woody biomass) in equal amounts. Soil emissions resulting from conversion and establishment of CEF land are also now included in the FMRL.

#### Overplanting

The 2011 FMRL did not model emissions from overplanting that occurs on Forest management land. This activity occurs when pre-1990 natural forest is converted to planted forest. The system used for national greenhouse gas reporting for the sector reports the area and emissions associated with that practice within the Forest management category. To maintain consistency with Forest management reporting, a technical correction needed to be applied. This technical correction results in the addition of 0.039 kilotonnes carbon dioxide (kt CO2) emissions to the annual estimate of emissions in the FMRL.

#### Non-carbon emissions

Non-carbon emissions were not included in the 2011 FMRL submission, therefore, a technical correction is required to include these emissions. Non-carbon emissions are estimated based on the average controlled burning from 1990 to 2009 and the minimum historic level for wildfire.

##### Controlled burning

Emissions from the burning of pre-1990 planted forest harvest residues are now included. The harvest rate is as per the FMRL, and the proportion burned is that applied to the LULUCF Forest land remaining forest land category during the first commitment period of the Kyoto Protocol.

Burning of residues associated with conversions of pre-1990 natural forest to pre-1990 planted forest are included and are assumed to occur at the same rate as reported during the first commitment period.

##### Wildfire emissions

Wildfires are hard to predict and are influenced by inter-annual climatic conditions and regional drought. To estimate emissions from wildfire, the minimum annual historic level that occurred between 1990 and 2009, the calibration period, is applied. This approach is taken to be consistent with New Zealand’s background level of natural disturbance.

##### Nitrous oxide emissions

It is assumed that there are no nitrous oxide emissions from fertilisation of forests within the FMRL. These are minor and captured within the Agriculture sector.

#### Natural disturbance

Emissions from natural disturbance events were not originally considered in the calculation of the 2011 FMRL. New Zealand has reported its intention to apply the natural disturbance provision, and, for Forest management, the background level has been set at the minimum historic level. This is included in the estimate of the non-carbon emissions as described above.

However, emissions from, and associated with, salvage logging cannot be excluded from accounting during the second commitment period. This means that, when developing the natural disturbance background level, historical emissions from natural disturbances should exclude these emissions. New Zealand has not excluded these emissions from the historic data used to calculate its background level of natural disturbance emissions under its technically corrected FMRL. If New Zealand applies the provision to exclude emissions from natural disturbances from its accounting, the background level will then be adjusted to remove these salvage logging emissions.

#### Pre-1990 natural forest

Emissions and removals by pre-1990 natural forest were not included in the 2011 FMRL submission. Because pre-1990 natural forest is now included in New Zealand’s reporting of emissions for Forest management land, a technical correction is required. The rate of carbon change used for this technical correction is consistent with that reported from 1990 to 2013 in the 2015 inventory.

When projections of pre-1990 natural forest emissions are incorporated into the technically corrected FMRL, the area under Forest management is reduced to factor in the projected deforestation of these forests. This deforested land will be reported under Article 3.3 
– Deforestation. The business-as-usual projection of pre-1990 natural forest deforestation is based on the historical rate seen between 1990 and 2009.

#### Harvested wood products

Emissions and removals for the Harvested wood products pool were not included in the 2011 FMRL submission. The technical correction for this uses the same spreadsheet model used for New Zealand’s Forest management reporting with minor modifications, in order to enable reporting to 2020. The technical correction made reflects that there were no government policies either in place, or being planned, that would increase wood use and/or domestic production between 2013 and 2020.

To estimate emissions from harvested wood products from 2013 to 2020, the activity data time series was investigated for trends from 1990 to 2009. Production of products with relatively flat trends through the time series (i.e., pulp and paper) was held at 2009 rates between 2009 and 2020, and products whose production had been increasing over the period (i.e., panels and sawn wood) were increased at the projected rate of population increase (1 per cent; sourced from Statistics New Zealand). Changes in the harvesting rate between 2013 and 2020 have no impact on the production of domestic harvested wood products because wood that is not processed in New Zealand is assumed to be exported.

Exported raw materials are now included in the accounting as a result of research on the use and discard rate of harvested wood products produced from raw materials of New Zealand origin. Because harvested wood products produced from exported raw materials are now accounted for, a technical correction will be required. This is planned for the next submission.

Harvested wood products from pre-1990 natural forest are excluded. The volume produced from the harvesting of pre-1990 natural forests is less than 0.1 per cent of New Zealand’s total harvest volume (Ministry for Primary Industries, 2015).

#### Technical correction 2 (2019 submission): Addressing methodological inconsistencies between the 2011 FMRL and Forest management reporting

For the 2019 submission, the following technical correction is made to meet IPCC guidance. This aims to ensure consistency between the method used for greenhouse gas reporting of harvested wood products derived from Forest management activities and that used to calculate the FMRL (Kyoto Protocol Supplement, IPCC, 2014, sections 2.7.5.2 and 2.7.6).

In the 2016 National Inventory Report (2018 submission), New Zealand revised its harvested wood products model to include the contribution of export logs based on an export markets study. This improvement has had a major impact on emissions reported for Forest management activities. This technical correction has been carried out to address this methodological inconsistency between the 2011 FMRL and Forest management reporting.

### A5.1.3	Technical corrections and their impact

The impact of the technical corrections made in the 2016 and 2019 submissions to the FMRL are summarised in table A5.1.2.

Table A5.1.2	Summary of the technical corrections to the FMRL

|  | Emissions (Mt CO2-e yr-1) |
| --- | --- |
| FMRL | 11.15 |
| Technical corrections |  |
| Addressing methodological inconsistencies | –6.82 |
| Additional elements: |  |
| Non-carbon (including natural disturbance) | 0.01 |
| Pre-1990 natural forest | –6.08 |
| Harvested wood products | –7.68 |
| Sum of technical corrections | –20.57 |
| FMRLcorr | –9.42 |

Note: 	FMRL = forest management reference level; FMRLcorr = technically corrected forest management reference level. Annual changes are presented in table A5.1.3.

Figure A5.1.1 and table A5.1.3 provide a breakdown of the various components of the technical corrections over the time series.

Figure A5.1.1	Technical corrections to the FMRL by category

Note: 	Non-carbon emissions are 0.013 Mt CO2-e per year, which is too small to display at this scale. FM = forest management; FMRL = forest management reference level.

Figure A5.1.2 provides a comparison of recalculated estimates with previous estimates. This illustrates the time-series consistency of the estimates.

Figure A5.1.2	Comparison of the 2011 FMRL, technical corrections to ensure consistency with Forest management reporting, and total of technical corrections over the period to 2020

Note: 	FM = forest management; FMRL = forest management reference level; Corrected FMRL = technically corrected forest management reference level.

Table A5.1.3 	Contribution of each source to the FMRLcorr

|  | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 | 2020 | Annual average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addressing methodological inconsistencies between the 2011 FMRL and Forest management reporting | (Mt CO2-e) |  |  |  |  |  |  |  |  |
| Pre-1990 planted forest growth and harvesting – biomass | –2.96 | 0.30 | 2.29 | 1.96 | 5.01 | 6.00 | 5.38 | 3.54 | 2.69 |
| Pre-1990 planted forest – soil | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Carbon equivalent forests – biomass | 1.75 | 1.73 | 1.72 | 1.69 | 1.64 | 1.54 | 1.49 | 1.35 | 1.61 |
| Carbon equivalent forests – soil | –0.00 | –0.00 | –0.00 | –0.01 | –0.01 | –0.01 | –0.01 | –0.01 | –0.01 |
| Overplanting (conversion of pre-1990 natural forest to pre-1990 planted forest) – biomass | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.02 | 0.02 | 0.03 |
| Overplanting – soils | –0.00 | –0.00 | –0.00 | –0.00 | –0.00 | –0.00 | –0.00 | –0.00 | 0.00001 |
| Recalculated FMRL |  |  |  |  |  |  |  |  | 4.33 |
| Additional elements |  |  |  |  |  |  |  |  |  |
| Non-carbon emissions (including natural disturbance) | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Pre-1990 natural forest – biomass | –6.08 | –6.08 | –6.08 | –6.08 | –6.12 | –6.08 | –6.08 | –6.08 | –6.08 |
| Harvested wood products pool | –10.089 | –9.119 | –8.346 | –7.683 | –7.158 | –6.720 | –6.344 | –6.016 | –7.684 |
| FMRLcorr | –17.333 | –13.114 | –10.371 | –10.059 | –6.591 | –5.215 | –5.521 | –7.175 | –9.422 |

Note: 	FMRL = forest management reference level; FMRLcorr = technically corrected forest management reference level.

## A5.2 	Natural disturbance

New Zealand has chosen the minimum historical level approach for calculating its background level of natural disturbances for both Afforestation and reforestation and Forest management.

Types of natural disturbances New Zealand intends to exclude from the accounting are:

- wildfires

- invertebrate and vertebrate pests and diseases

- extreme weather events

- geological disturbances.

In all cases except fire, New Zealand assumes a zero baseline between 1990 and 2009. While other natural disturbance events occurred throughout the calibration period, assumptions were made for the purposes of calculating the background level.

For planted forests reported under Afforestation and reforestation and Forest management, salvage logging is considered to take place in all disturbed forests.

In the case of pre-1990 natural forests, the ground plot measurement programme captures emissions from natural disturbances implicitly, and the emissions from natural disturbance events, apart from wildfires, cannot be separated from other disturbance events. The stock change estimates reported for natural forests include background levels of small scale natural disturbance events.

Only direct oxidation of biomass in wildfires is considered for the purposes of calculating a background level of natural disturbance for both Afforestation and reforestation and Forest management lands, regardless of forest type. The data used are as reported under the UNFCCC for the period 1990–2009 (see chapter 6, section 6.11.5).

### A5.2.1 	Afforestation and reforestation

New Zealand may choose to apply the provision for the treatment of natural disturbance emissions to its Afforestation and reforestation accounting (Ministry for the Environment, 2015). Due to the nature of Afforestation and reforestation accounting and reporting methods, the background level of carbon dioxide emissions from natural disturbance is already captured implicitly within the reported estimates. New Zealand separately estimates and reports the non-carbon emissions from natural disturbances. The background level is set by calculating the minimum non-carbon emissions that occurred from natural disturbances during the calibration period (1990–2009) (figure A5.2.1). However, both the post-1989 forest area and the carbon stock increase during the calibration period. Therefore, the background level is selected as the year in the calibration period with the minimum emissions from natural disturbance in proportion to total carbon stock. The minimum proportion from the calibration period is then multiplied by the carbon stock in post-1989 forest for each year in the reporting period (2013–20). This approach provides the background level and corrects for the increasing area and age (and therefore carbon stock exposed to natural disturbance) in post-1989 forests.

The Afforestation and reforestation background level for 2017 was 0.268 kilotonnes carbon dioxide equivalent (kt CO2-e).

Figure A5.2.1 	Calculating the background level of natural disturbance during the calibration period for Afforestation and reforestation land

##### Avoiding the expectation of net credits or net debits for the application of the natural disturbance provision: Afforestation and reforestation

The background level is selected as the year in the calibration period with the minimum emissions from natural disturbance in proportion to total carbon stock. The minimum proportion from the calibration period is then multiplied by the carbon stock in post-1989 forest for each year in the reporting period (2013 to 2020). This approach is taken because:

- a trend is observed in natural disturbance emissions during the calibration period for Afforestation and reforestation. Emissions from natural disturbances have been increasing throughout the calibration period as the age of these forests and, therefore, biomass increase through time. It is expected that this trend will continue during the second commitment period. The calibration period was used to obtain an annual emissions value by proportion of carbon stocks and the minimum then used to calculate the background level for the 2013 year onwards, based on the carbon stocks of Afforestation and reforestation lands in each year

- gross:net accounting applies to Afforestation and reforestation activities. Emissions from natural disturbances occurring during any year of the commitment period, which fall below the background level, are not excluded from the accounting. Emissions from natural disturbances that are greater than the background level in any year of the commitment period are able to be excluded from the accounting if a Party chooses

- the background level has been set at the minimum historical level so there is an expectation that emissions will exceed the background level every year during the commitment period. If emissions from natural disturbances are greater than the background level, they can be excluded from the accounting and there is no expectation of net debits arising. If emissions are less than the background level in any year of the commitment period, all emissions from natural disturbance will still be accounted for. There is no expectation of net debits in this scenario. Under gross:net accounting for Afforestation and reforestation activities, it would not be possible to expect net credits when applying this approach to excluding the emissions from natural disturbances.

### A5.2.2	Forest management

The background level of natural disturbance for Forest management was calculated as 2.387 kt CO2-e (figure A5.2.2).

Figure A5.2.2 	Emissions from natural disturbance during the calibration period on
Forest management land

##### Avoiding the expectation of net credits or net debits for the application of the natural disturbance provision: Forest management

The background level has been set at the minimum annual emissions value of the historical time series (see figure A5.2.2) because:

- there is no observed trend in natural disturbance emissions during the calibration period for Forest management and therefore none can be expected during the second commitment period

- the background level of emissions for Forest management, to be included in the FMRL via a technical correction, is equal to the minimum annual emissions value estimated during the calibration period

- any emissions from natural disturbances during the commitment period that fall below the background level are not excluded from the accounting. During the commitment period, emissions from natural disturbances that are above the background level are, subject to New Zealand’s discretion, able to be excluded from the accounting

- the accounting for Forest management is against a projected business-as-usual FMRL. The background level is included implicitly within the FMRL, and any emissions greater than the background level can be excluded from the accounting. When applying this approach, there is, therefore, no expectation of net debits. In setting the background level to the minimum across the calibration period, emissions are expected to exceed this level every year. There is, therefore, no expectation that emissions will be less than the background level and also no expectation of net credits.

## A5.3	Carbon equivalent forests

Information on carbon equivalent forests is provided in aggregated form in CRF table 4(KP-I)B.1.2. Details of each application that make up the reported estimates are provided in table A5.3.1.

Table A5.3.1	Breakdown of carbon equivalent forests by domestic scheme application, 2014–17

| Scheme ID | Management type | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- |
| CEF - 2 | Newly Established (Ha) | – | – | – | 291.98 |
|  | Harvested and Converted (Ha) | – | 37.62 | 106.29 | 106.61 |
|  | Net Change (tC) | – | –7.97 | –22.49 | –24.29 |
| CEF - 3 | Newly Established (Ha) | – | – | 407.49 | – |
|  | Harvested and Converted (Ha) | 49.74 | 300.22 | 57.15 | – |
|  | Net Change (tC) | –11.63 | –70.16 | –14.09 | 0.10 |
| CEF - 4 | Newly Established (Ha) | – | – | 53.79 | – |
|  | Harvested and Converted (Ha) | – | – | 24.06 | – |
|  | Net Change (tC) | – | – | –4.94 | –0.01 |
| CEF - 8 | Newly Established (Ha) | – | – | 52.57 | – |
|  | Harvested and Converted (Ha) | 0.38 | – | 52.19 | – |
|  | Net Change (tC) | –0.08 | 0.00 | –11.73 | 0.01 |
| CEF - 9 | Newly Established (Ha) | – | – | 24.47 | – |
|  | Harvested and Converted (Ha) | – | 1.45 | 23.01 | – |
|  | Net Change (tC) | – | –0.34 | –5.44 | 0.01 |
| CEF - 11 | Newly Established (Ha) | – | – | – | 697.26 |
|  | Harvested and Converted (Ha) | – | 20.60 | 384.73 | 530.67 |
|  | Net Change (tC) | – | –5.15 | –67.35 | –99.65 |
| CEF - 12 | Newly Established (Ha) | – | – | 152.35 | – |
|  | Harvested and Converted (Ha) | – | – | 151.95 | – |
|  | Net Change (tC) | – | – | –35.88 | 0.04 |
| CEF - 13 | Newly Established (Ha) | – | – | 106.04 | – |
|  | Harvested and Converted (Ha) | – | 1.71 | 103.67 | – |
|  | Net Change (tC) | – | –0.40 | –24.49 | 0.03 |
| CEF - 14 | Newly Established (Ha) | – | – | – | 142.84 |
|  | Harvested and Converted (Ha) | – | 4.23 | 138.57 | – |
|  | Net Change (tC) | – | –0.87 | –26.13 | –0.24 |
| CEF - 15 | Newly Established (Ha) | – | – | – | 175.24 |
|  | Harvested and Converted (Ha) | – | – | 37.68 | 94.56 |
|  | Net Change (tC) | – | – | –7.97 | –21.06 |
| CEF - 17 | Newly Established (Ha) | – | – | – | 8.55 |
|  | Harvested and Converted (Ha) | – | – | 6.83 | – |
|  | Net Change (tC) | – | – | –1.82 | –0.02 |
| CEF - 18 | Newly Established (Ha) | – | – | – | – |
|  | Harvested and Converted (Ha) | – | 10.01 | 112.30 | – |
|  | Net Change (tC) | – | –2.00 | –23.78 | 0.08 |
| CEF - 19 | Newly Established (Ha) | – | – | – | – |
|  | Harvested and Converted (Ha) | – | 2.83 | 10.45 | 93.84 |
|  | Net Change (tC) | – | –0.59 | –2.23 | –20.62 |
| CEF - 20 | Newly Established (Ha) | – | – | – | 14.37 |
|  | Harvested and Converted (Ha) | – | 7.40 | – | – |
|  | Net Change (tC) | – | –1.90 | 0.00 | –0.03 |
| CEF - 21 | Newly Established (Ha) | – | – | – | 173.78 |
|  | Harvested and Converted (Ha) | – | 7.80 | 80.44 | 83.65 |
|  | Net Change (tC) | – | –1.52 | –17.90 | –18.53 |
| CEF - 24 | Newly Established (Ha) | – | – | – | 18.33 |
|  | Harvested and Converted (Ha) | – | – | – | 17.84 |
|  | Net Change (tC) | – | – | – | –4.10 |
| CEF - 25 | Newly Established (Ha) | – | – | – | – |
|  | Harvested and Converted (Ha) | – | – | – | 133.27 |
|  | Net Change (tC) | – | – | – | –33.07 |
| CEF - 27 | Newly Established (Ha) | – | – | – | 34.47 |
|  | Harvested and Converted (Ha) | – | – | 51.89 | – |
|  | Net Change (tC) | – | – | –11.54 | –0.05 |
| CEF - 31 | Newly Established (Ha) | – | – | – | – |
|  | Harvested and Converted (Ha) | – | – | – | – |
|  | Net Change (tC) | – | – | – | – |
| TOTAL | Newly Established (Ha) | – | – | 796.72 | 1,556.83 |
|  | Harvested and Converted (Ha) | 50.12 | 393.87 | 1,341.23 | 1,060.45 |
|  | Net Change (tC) | –11.71 | –90.91 | –277.78 | –221.40 |

Note: 	CEF = carbon equivalent forest; Ha = hectares; tC = tonnes carbon.

## Annex 5: References

IPCC. 2014. Hiraishi T, Krug T, Tanabe K, Srivastava N, Baasansuren J, Fukuda M, Troxler TG (eds). 2013 Revised Supplementary Methods and Good Practice Guidance Arising from the Kyoto Protocol. Switzerland: IPCC.

Ministry for Primary Industries. 2014. National Exotic Forest Description as at 1 April 2014. Wellington: Ministry for Primary Industries.

Ministry for Primary Industries. 2015. Log and Roundwood Removal Statistics. Retrieved from: www.mpi.govt.nz/news-and-resources/statistics-and-forecasting/forestry (July 2015).

Ministry for the Environment. 2015. New Zealand’s Greenhouse Gas Inventory 1990–2013. Wellington: Ministry for the Environment.

New Zealand Government. 2011. Forest Management Reference Level Submission. Retrieved from http://unfccc.int/files/meetings/ad_hoc_working_groups/kp/application/pdf/newzealand_frml.pdf (22 February 2016).

UNFCCC. 2011. Report of the technical assessment of the forest management reference level submission of New Zealand submitted in 2011. Retrieved from http://unfccc.int/ resource/docs/2011/tar/nzl01.pdf (22 February 2016).

UNFCCC. 2012. Report of the Conference of the Parties serving as the meeting of the Parties to the Kyoto Protocol on its seventh session, held in Durban from 28 November to 11 December 2011: Addendum – Part 2: Action taken by the Conference of the Parties serving as the meeting of the Parties to the Kyoto Protocol at its seventh session. FCCC/KP/CMP/2011/10/Add.1.

# Annex 6: Additional information on the inventory system and completeness

## A6.1 	Quality assurance and quality control processes

The quality assurance and quality control (QA/QC) processes have a significant role in the preparation of the inventory, to ensure the core principles of transparency, accuracy, completeness, comparability and consistency are achieved. Table A6.1.1 describes the main QA/QC processes used in the preparation of the inventory. These processes are under continual review and improvement to ensure they are fit for purpose.

Table A6.1.1 	Quality assurance and quality control processes used in preparation of the inventory

| ID | QA/QC process or activity description |
| --- | --- |
| QA file | All external reviews of the whole or part of the inventory are documented in the QA file. Reviews are performed by qualified personnel, and the review records are included in the submission of the inventory to the United Nations Framework Convention on Climate Change. These reviews act as a source of ideas for inventory improvements. |
| QC 1 | Planned recalculations and improvements are approved by the reporting governance group that oversees all climate change reporting by the New Zealand Government. The role of this group is further described in chapter 1. |
| QC 2 | Planned improvements are peer reviewed prior to being implemented, when they affect the emission factor, parameter, methodology or activity data source. This is superseded in sectors that have a dedicated panel. |
| QC 3 | Tier 1 checklist QC sheets are completed to ensure transparency, accuracy, completeness, comparability and consistency principles are met. Examples are included in the submission of the inventory. |
| QC 4 | The chapter text for each sector is peer reviewed and follows the checklist provided to ensure that the peer review is comprehensive and consistent. |
| QC 5 | Recalculations that exceed a certain threshold (see figure A6.1.1) are analysed and clearly documented. This includes changes resulting from planned improvements, errors, recommendations from the expert review team, and changes to guidelines. |
| QC 7 | All sectors in the inventory are approved by members of the reporting governance group that oversees all climate change reporting by the New Zealand Government before being submitted to the National Inventory Compiler. |
| QC 10 | Common reporting format QC tools identify any potential issues with the data and are used to ensure the data standards are met. |
| Sector submission checks | Sector submissions are checked by the inventory agency prior to sector submission against the data and chapter standards. Any issues must be resolved before submitting. This enables the remainder of the inventory compilation to proceed smoothly because quality is assured. |

Figure A6.1.1 shows how these QA/QC processes align with the overall preparation of the inventory.

Figure A6.1.1 	How the quality assurance and quality control processes and products align with the preparation of the inventory

Planned improvements	Sectoral compilation

Plus unplanned improvements

Prepare common reporting format data and report sections, and improvements if applicable.

Internal quality control

- QC 3 Tier 1 checklist for common reporting format data.

- QC 4 chapter checklist for peer review of chapter.

- QC 10 CRF QC tools for data quality.

Deliver to the National Inventory Compiler:

- QC 7 approval of sector from reporting governance group manager

- include QC 2 and QC 5 forms as applicable.

Start with an idea, such as from a list of possible improvements (the ‘QA file’ could inform ideas).

Get improvements peer reviewed and complete. QC 2 peer-review change form if it affects:

emission factor, parameter, methodology or activity data source.

Include all planned improvements in QC 1 planned recalculations and improvements for the reporting governance group.

Fill out QC 5 recalculation form if it is a change to:

- emissions >500 kilotonnes

- methodology

- an emission factor

- emissions that result in the trend reversing.

Unplanned improvements, including recommendations from the expert review team, and resolution of any other issues as required.

Sector undergoes quality checks by the inventory agency.

Chapter and data checked against the quality standards, any issues are resolved before receiving confirmation of quality from the inventory agency.

The QA/QC processes of the inventory data from Tokelau include Tier 1 checks for accuracy, consistency and completeness as well as the peer reviews by personnel not involved in the inventory production. Figure A6.1.2 presents an overview of the entire process.

Figure A6.1.2 	Data processing, quality assurance and quality control processes during the inventory preparation of the data from Tokelau

| CRF | Common reporting format |
| --- | --- |
| MFAT | Ministry of Foreign Affairs and Trade |
| MfE | Ministry for the Environment (New Zealand) |
|  | QA/QC procedures performed by Tokelau NSO |
|  | QA/QC procedures performed by MfE |
|  | QA/QC procedures performed by different agencies in Tokelau and New Zealand |
| NIR | National inventory report |
| NSO | National Statistics Office (Tokelau) |
| QA/QC | Quality assurance and quality control |
| RGG | Reporting Governance Group |

The preliminary data quality checks for activity data emissions calculations are performed at the Tokelau National Statistics Office (NSO). The checked data files are then sent to the Ministry for the Environment (MfE) for further processing and analysis. Experts at MfE check the applicability of formulae and emission factor values used for the emissions estimates made at Tokelau and perform the remaining emissions estimates (for the Industrial Processes and Product Use and Waste sectors).

Results of the analysis and the emissions estimates are sent to Tokelau NSO for an additional quality check. After the Tokelau feedback is implemented, MfE prepares the common reporting format (CRF) input file with the data from Tokelau that undergoes data integrity checks using the bespoke QC tools (similar to other sectoral data from New Zealand, see the deliverable QC 10 in figure A.6.1.1). Additional QC procedures applied to the data from Tokelau include the final CRF completeness check as well as accuracy and consistency checks for the use of the data in the inventory chapter 8 and annex 7. Chapter 8 (Tokelau) is independently peer reviewed in both Tokelau and New Zealand by several experts who are not involved in the inventory preparation. The final stage of the QA/QC process is the approval by the Reporting Governance Group manager, which indicates that the Tokelau inventory contribution is ready for the national compilation. The entire process is presented in figure A.6.1.2.

## A6.2 	General assessment of completeness

### A6.2.1	Emissions reported as ‘NE’ (not estimated)

According to the United Nations Framework Convention on Climate Change (UNFCCC) reporting guidelines (UNFCCC, 2013), the notation key ‘NE’ (not estimated) signifies that emissions and/or removals occur but have not been estimated or reported. It can be applied for the following reasons:

- if emissions are insignificant, that is, they should not exceed 0.05 per cent of the national total greenhouse gas (GHG) emissions, and do not exceed 500 kilotonnes carbon dioxide equivalent (kt CO2-e) (paragraph 37(b) of the UNFCCC reporting guidelines)

- the total national aggregate of estimated emissions for all gases and categories considered insignificant shall remain below 0.1 per cent of the national total GHG emissions (paragraph 37(b) of the UNFCCC reporting guidelines)

- when an activity occurs in the Party but the 2006 IPCC Guidelines for National Greenhouse Gas Inventories (IPCC, 2006) do not provide methodologies to estimate emissions and removals (footnote 6 of the UNFCCC reporting guidelines (UNFCCC, 2013)). If this is the case, the category is considered to be non-mandatory, providing the emissions from the category have not been reported previously.

The UNFCCC reporting guidelines also state that, once emissions from a specific category have been reported in a previous submission, emissions from this specific category shall be reported in subsequent GHG inventory submissions (UNFCCC, 2013).

New Zealand’s gross emissions are 80,853.5 kt CO2-e in 2017. The threshold of 0.1 per cent for New Zealand’s 2019 submission is 80.9 kt CO2-e and the threshold of 0.05 per cent is 40.5 kt CO2-e. Both values are below 500 kt CO2-e.

Table A6.2.1 summarises New Zealand’s direct GHG emissions reported as ‘NE’ (not estimated) in the 2019 submission.

Table A6.2.1 	Summary of NE (not estimated) entries in 2019 submission

| CRF category code | Category | Gas | Explanation |
| --- | --- | --- | --- |
| Energy |  |  |  |
| 1.B.1.a.1.iii | Abandoned underground mines | CO2, CH4 | Methane (CH4) emissions from this category do not occur in the North Island of New Zealand and are not estimated for the South Island. Because the historical information is not available, New Zealand does not have any reliable information on activities related to CH4 emissions from abandoned mines to reliably report on it. A project is under way that evaluates abundant mines, which has been completed for the North Island but not for the South Island. Judging from the results for the North Island, emissions from the South Island can be also ‘NO’ (not occurring), so it is the only information we can use here. |
| 1.B.2.a.5 | Distribution of oil products | CO2, CH4 | This category is not mandatory: the 2006 IPCC Guidelines do not provide the default Intergovernmental Panel on Climate Change (IPCC) emission factor for calculating Tier 1 estimates of CH4 emissions from the distribution of refined oil products. New Zealand did not report an emissions estimate from this category prior to the 2018 submission. |
| 1.B.2.b.3 | Processing | CO2, CH4 | Fugitive emissions of CO2 and methane (CH4) have not been formally estimated, though a rough estimate of the likely level of emissions indicates that they are not significant. While emissions from the Kapuni Gas Treatment Plant may include traces of CH4, the level of these emissions has been determined to be insignificant in comparison with national emissions: a conservative estimate (using default emission factors from the 2006 IPCC Guidelines) gives approximately 1.5 carbon dioxide equivalent (kt CO2-e) per year. CH4: 625 Mm3 (Kapuni field production) * 9.7e-5 * 25 = 1.5 kt carbon dioxide equivalent (CO2-e). The conservative estimated value is below 0.05 per cent of New Zealand’s gross emissions. This would keep the national total aggregate of estimated emissions for all gases and categories considered insignificant below 0.1 per cent of the national total greenhouse gas emissions, which is in line with the paragraph 37(b) of the UNFCCC reporting guidelines. Carbon dioxide (CO2) from this category is mostly associated with venting through the chimney and, therefore, is reported under 1.B.2.c.1, as advised by the previous expert review team. However, there is a possibility of the presence of trace amounts of CO2 from processing due to leakage, which is estimated to be no higher than 0.1 per cent of vented CO2. The most conservative estimate of 0.1 per cent of vented CO2 from all categories is 0.26 kt, which is below 0.05 of the gross emissions (40.6 kt CO2-e) and thus can be considered insignificant. |
| Agriculture |  |  |  |
| 3.A.4 (for both New Zealand and Tokelau) | Poultry | CH4 | This category is not mandatory: the 2006 IPCC Guidelines state (page 10.27, vol 4-2) that the Tier 1 method for estimating CH4 emissions from enteric fermentation for poultry is not developed. Also, table 10-10 (page 10.28, vol 4-2) indicates that there is insufficient data to establish a CH4 emission factor for poultry for either developed or developing countries. |
| 3.B.2.5 | Indirect N2O emissions | N2O | According to footnote 6 in paragraph 37(b) of the United Nations Framework Convention on Climate Change (UNFCCC) reporting guidelines, this category is not mandatory for reporting. The 2006 IPCC Guidelines for determining indirect nitrous oxide (N2O) emissions do not provide a methodology for estimating emissions from leaching and run-off. In addition, indirect N2O emissions from leaching and run-off are insignificant in New Zealand, because almost all livestock are kept outdoors all year around on pasture. |
| 3.B.2.5 | N2O emissions per MMS | N2O | Direct N2O emissions from anaerobic lagoons (dairy and swine) and daily spread (swine) are reported under Agricultural soils. The 2006 IPCC Guidelines assume that negligible direct N2O emissions occur in anaerobic lagoons and daily spread, and only occur once the stored effluent is spread onto agricultural soil. For more information, see chapter 5, sections 5.3.2 (Direct nitrous oxide emissions from manure management) and 5.5.2 (Urine and dung deposited by grazing animals) of the NIR. According to footnote 6 in paragraph 37(b) of the UNFCCC reporting guidelines, this category is not mandatory for reporting. |
| 3.D.1.2.c | Other organic fertilisers applied to soils | N2O | Emissions from ‘Other organic fertilisers applied to soils’ are not estimated due to their insignificance. Emissions are roughly estimated to be 20 kt CO2-e (van der Weerden et al., 2014). Emissions are below the threshold of 0.05 per cent of the national total greenhouse gas emissions and do not exceed 500 kt CO2-e. |
| 3.I | Other carbon-containing fertilisers | CO2 | The 2006 IPCC Guidelines do not provide guidance for reporting on other carbon-containing fertilisers. Other carbon-containing synthetic fertilisers besides limestone, dolomite and urea are not applied to agricultural land in New Zealand. |
| Land Use, Land-Use Change and Forestry |  |  |  |
| 4. D.1 | Forest land, cropland, grassland and wetlands: Drainage and rewetting and other management of organic and mineral soils | CH4, N2O | No methodology is provided in the 2006 IPCC Guidelines for estimating emissions from this source category. According to footnote 6 in paragraph 37(b) of the UNFCCC reporting guidelines, this category is not mandatory for reporting. |
| 4.B.1 | Cropland remaining cropland/4(V) Biomass burning/ Wildfires/Cropland remaining cropland | CH4, N2O | New Zealand does not have sufficient information on biomass burning activities to reliably report on it. |
| 4.B.2 | Land converted to cropland/4(V) Biomass burning/ Wildfires/Land converted to cropland | CH4, N2O | New Zealand does not have sufficient information on biomass burning activities to reliably report on it. |
| 4.D.1 | Wetlands remaining Wetlands/4(V) Biomass burning/ Wildfires/Wetland remaining wetland | CH4, N2O | No IPCC guidance is provided for calculating Tier 1 estimates of carbon stock changes in organic soils for this land use category. New Zealand does not have sufficient information on biomass burning activities to reliably report on it. |
| Waste |  |  |  |
| 5.D.1 and 5.D.2 | Domestic wastewater and Industrial wastewater | Amount of CH4 flared | NE (not estimated) is used for activity data, because New Zealand does not have any information regarding the CH4 flaring in this source category. The amount of CH4 flared does not contribute to New Zealand's total emissions, but the implied emission factor only (as per the 2006 IPCC Tier 1 methodology provided in table 5D of the common reporting format tables). |

The estimate of emissions for all of New Zealand’s source categories marked as ‘NE’ (not estimated) results in 21.8 kt CO2-e, which is below the 0.1 per cent of the total emissions threshold (80.9 kt CO2-e).

### A6.2.2	Emissions reported as ‘IE’ (included elsewhere)

According to the UNFCCC reporting guidelines (UNFCCC, 2013), the notation key ‘IE’ (included elsewhere) signifies that emissions and/or removals for this activity or category are estimated and included in the inventory but not presented separately for this category.

Table A6.2.2 details where the notation key ‘IE’ (included elsewhere) has been used in this submission of the inventory.

Table A6.2.2 	Emissions reported using the ‘IE’ (included elsewhere) notation key

| CRF category code | Category | Reported under the following source category: | Notation key explanation |
| --- | --- | --- | --- |
| 1.A.2.a | Iron and steel – liquid fuels | 1.A.2.g.viii – Other – Liquid fuels | Disaggregated data do not exist. |
| 1.A.2.a | Iron and steel – solid fuels | 2.C.1 – Iron and steel production | All emissions from the use of coal in this category are included in the Industrial Processes and Product Use sector because the primary purpose of the coal is to produce iron. |
| 1.A.2.f | Non-metallic minerals – biomass | 1.A.2.g.viii – Other – Biomass | Disaggregated data do not exist. |
| 1.A.2.g.v | Construction – all fuels | 1.A.2.g.iii – Mining | Disaggregated data do not exist. |
| 1.A.3.b.ii–iv | Road transportation (other than ‘Cars’) – all fuels | 1.A.3.b.i – Cars | Disaggregated data do not exist. Disaggregation of carbon dioxide (CO2) emissions is included in the plan, but the implementation has not yet been completed. |
| 1.A.4.c.ii–iii | Agriculture/forestry/fishing – Off-road vehicles and other machinery | 1.A.4.c.i – Agriculture/ forestry/fishing – Stationary | Agriculture/forestry/fishing has not been disaggregated into stationary, mobile and fishing: data are not available. |
| 1.B.2.b.1 | Natural gas/exploration | 1.B.2.a.1 – Oil exploration | In New Zealand, exploration is not specifically aimed at obtaining oil or gas, that is, oil exploration is not separated from gas exploration by planning, processes, equipment or resources. Thus the exploratory wells are drilled without distinction of their purpose, that is, whether the expected outcome is oil, gas, both or none and there is no reliable way to predict which it would be to estimate proportions of mostly oil and mostly gas wells. In that sense, disaggregated data for oil and gas exploration do not exist. Considering that available emission factors for well drilling and testing also do not distinguish between oil and gas, all emissions from oil and gas exploration are placed in the same category. |
| 1.B.2.c.1.i–ii | Venting/oil and Venting/gas | 1.B.2.c.1.iii – Venting/ combined | The fields produce both oil and gas and, therefore, are reported as combined. Disaggregated data do not exist. |
| 1.B.2.c.1.i–ii | Flaring/oil and Flaring/gas | 1.B.2.c.1.iii – Flaring/ combined | The fields produce both oil and gas and, therefore, are reported as combined. Disaggregated data do not exist. |
| 2.A.3 | Glass production | 2.A.4.b – Other process uses of carbonates/Other uses of soda ash | Carbon dioxide emissions are reported in 2.A.4.b because this aggregates emissions from glass production with other uses of carbonates, due to confidentiality concerns for both glass and aluminium production. A very small number of firms in New Zealand are involved in these activities and use carbonates. |
| 3.A.4 | Enteric fermentation/ other/buffalo | 3.A.1.A – Dairy cattle | A small herd of around 200 buffalo was brought into New Zealand around 2007 for specialised cheese and dairy production. These buffalo are reported within the dairy herd so the notation key ‘IE’ is used from 2007 onwards. |
| 3.B.1.4 & 3.B.2.4 | Manure management/ other/buffalo | 3.B.1.A – Dairy cattle 3.B.2.A – Dairy cattle | For both nitrous oxide (N2O) and CH4 emissions, the notation key ‘NO’ (not occurring) is used up to 2006 because no buffalo were recorded in New Zealand before 2007. A small herd of around 200 buffalo was brought into New Zealand around 2007 for specialised cheese and dairy production. These buffalo are reported within the dairy herd so the notation key ‘IE’ is used from 2007 onwards. For more information, see chapter 5, section 5.1.4 (Minor livestock categories) of this national inventory report (NIR). |
| 3.B.2.5 | N2O Emissions per MMS | 3.D – Agricultural soils | Direct N2O emissions from anaerobic lagoons (dairy and swine) and daily spread (swine) are reported under Agricultural soils. |
| 3.D.1.2.b | Sewage sludge applied to soils | Included under the Waste sector 5.A.1.a | Direct N2O emissions from sewage sludge are reported under 5.A.1.a in the Waste sector. Sewage sludge activity data are obtained from water treatment industry surveys and do not disaggregate the amount of sludge used for different purposes. Due to the small amount of emissions coming from sewage sludge, further disaggregation of the activity data is considered resource prohibitive. Sewage sludge is a very small source of nitrogen, contributing 84 tonnes of nitrogen annually (van der Weerden et al., 2014). Emissions of N2O are less than 0.002 kilotonnes. |
| 3.E | Prescribed burning of savannas | Biomass burning (table 4(V) of LULUCF), category C. Grassland | Prescribed burning of savanna is reported under the Land Use, Land-Use Change and Forestry (LULUCF) sector. See chapter 6, section 6.11.5 of this NIR (Biomass burning (table 4(V) of LULUCF), category C Grassland). |
| 4.A.1/4(V) | Controlled burning/Forest land remaining forest land | Carbon dioxide emissions are captured by the general carbon stock change calculation if the fire-damaged area is harvested and replanted. If the stand is allowed to grow on but with a reduced stocking, the CO2 emissions are accounted for at the eventual time of harvest. | Carbon dioxide emissions are captured by the general carbon stock change calculation if the fire-damaged area is harvested and replanted. If the stand is allowed to grow on but with a reduced stocking, the CO2 emissions are accounted for at the eventual time of harvest. |
| 4.A.2/4(V) | Controlled burning/Land converted to forest land Wildfires/Land converted to forest land | Carbon dioxide emissions are captured by the general carbon stock change calculation if the fire-damaged area is harvested and replanted. If the stand is allowed to grow on but with a reduced stocking, the CO2 emissions are accounted for at the eventual time of harvest. | Carbon dioxide emissions are captured by the general carbon stock change calculation if the fire-damaged area is harvested and replanted. If the stand is allowed to grow on but with a reduced stocking, the CO2 emissions are accounted for at the eventual time of harvest. |
| 4.B.1/4(V) | Controlled burning/ Cropland remaining cropland | Included under the Agriculture sector | Carbon dioxide and CH4 emissions from burning of crop stubble are reported in the Agriculture sector. |
| 4.B.1/4(V) | Wildfires/Cropland remaining cropland | Any CO2 emissions from wildfires on non-forest land are likely to be offset by the subsequent carbon gain from the regrowth of biomass, which is also not accounted for. Alternatively, if the wildfire resulted in land-use change, then any CO2 emissions would be captured by the general carbon stock change calculation that is performed when land is converted to a new land use. | Any CO2 emissions from wildfires on non-forest land are likely to be offset by the subsequent carbon gain from the regrowth of biomass, which is also not accounted for. Alternatively, if the wildfire resulted in land-use change, then any CO2 emissions would be captured by the general carbon stock change calculation that is performed when land is converted to a new land use. |
| 4.B.2/4(V); 4.C.1/4(V); 4.C.2/4(V); 4.D.2/4(V) | Wildfires/Land converted to cropland Wildfires/Grassland remaining grassland Wildfires/Land converted to grassland Wildfires/Land converted to wetlands | Any CO2 emissions from wildfires on non-forest land are likely to be offset by the subsequent carbon gain from the regrowth of biomass, which is also not accounted for. Alternatively, if the wildfire resulted in land-use change, then any CO2 emissions would be captured by the general carbon stock change calculation that is performed when land is converted to a new land use. | Any CO2 emissions from wildfires on non-forest land are likely to be offset by the subsequent carbon gain from the regrowth of biomass, which is also not accounted for. Alternatively, if the wildfire resulted in land-use change, then any CO2 emissions would be captured by the general carbon stock change calculation that is performed when land is converted to a new land use. |
|  | Direct N2O emissions from nitrogen (N) inputs to managed soils/Inorganic N fertilisers and Direct N2O emissions from N inputs to managed soils/Organic N fertilisers from the following categories: | Included under the Agriculture sector. | New Zealand does not disaggregate data on nitrogen fertiliser by land use, therefore, all N2O emissions from organic and inorganic fertilisers are reported in the Agriculture sector. |
| 4.A.1/4(I); | Forest land remaining forest land | Included under the Agriculture sector. Included under the Agriculture sector | New Zealand does not disaggregate data on nitrogen fertiliser by land use, therefore, all N2O emissions from organic and inorganic fertilisers are reported in the Agriculture sector. All emissions from burning of crop stubble are reported in the Agriculture sector. |
| 4.D.1/4(I) | Wetlands remaining wetlands Land converted to wetlands |  |  |
| 4.D.2/4(I) | Settlements remaining settlements Land converted to settlements |  |  |
| 4.E.1/4(I) | Settlements remaining settlements |  |  |
| 4.E.2/4(I) | Land converted to settlements |  |  |
| 4.B.1/4(V) | Controlled burning/ Cropland remaining cropland |  |  |
| 4.C.1/4(V); 4.D.1/4(V) | Controlled burning/ Grassland remaining grassland Controlled burning/ Wetland remaining wetland Wildfires/Wetland remaining wetland | If due to temperate climate and rainfall, any CO2 emissions from burning on non-forest land are likely to be offset by the subsequent carbon gain from the regrowth of biomass, which is also not accounted for. Alternatively, if the fire resulted in land-use change, then any CO2 emissions would be captured by the general carbon stock change calculation that is performed when land is converted to a new land use. | This is not a significant activity in New Zealand due to its temperate climate and rainfall distribution, and any CO2 emissions from burning on non-forest land are likely to be offset by the subsequent carbon gain from the regrowth of biomass, which is also not accounted for. Alternatively, if the fire resulted in land-use change, then any CO2 emissions would be captured by the general carbon stock change calculation that is performed when land is converted to a new land use. |
| 4.C.2/4(V); 4.D.2/4(V) 4.E/4(V) | Controlled burning/Land converted to grassland Controlled burning/Land converted to wetlands Biomass burning/Land converted to settlements | Carbon dioxide emissions from the controlled burning of land converted to this category are captured by the general carbon stock change calculation that is performed when land is converted to a new land use. | Carbon dioxide emissions from the controlled burning of land converted to this category are captured by the general carbon stock change calculation that is performed when land is converted to a new land use. |
| 5.D.1 | Domestic wastewater | 5.A Solid waste | Activity data – sludge amounts are included in solid waste disposal because sludge is disposed to landfill. |
| 5.D.2 | Industrial wastewater | 5.A Solid waste | Activity data – sludge amounts are included in solid waste disposal because sludge is disposed to landfill. |
| Within the Tokelau sector 6, categories 1.A.3.b.i and 1.A.4.c.iii were reported elsewhere. | Road transport/gasoline and diesel oil | Domestic navigation | The number of petrol cars has, until recently, been very small in Tokelau (in 2018 only about 40 cars, 30 motorbikes, entire road network less than 10 kilometres). Census 2001 and prior record only four registered cars. Aluminium boats are the main means of family transport: there were, on average, 135 outboard motors over the period 1990–2017 travelling both outside and within the large lagoons. Therefore any petrol use for road transport is far outweighed by Domestic navigation, and is included there. |
| Within the Tokelau sector 6, categories 1.A.3.b.i and 1.A.4.c.iii were reported elsewhere. | Other category (1.A.4) | Domestic navigation | Only gas used for cooking is listed here. Amounts of liquid fuel use are miniscule compared with Domestic navigation and are included there. |

## Annex 6: References

IPCC. 2006. Eggleston HS, Buendia L, Miwa K, Ngara T, Tanabe K (eds). 2006 IPCC Guidelines for National Greenhouse Gas Inventories. IPCC National Greenhouse Gas Inventories Programme. Japan: Published for the IPCC by the Institute for Global Environmental Strategies.

UNFCCC. 2013. FCCC/CP/2013/10/Add.3. Report of the Conference of the Parties on its nineteenth session, held in Warsaw from 11 to 23 November 2013, Addendum; Decision 24/CP.19 Revision of the UNFCCC reporting guidelines on annual inventories for Parties included in Annex I to the Convention.

Van der Weerden A, de Klein C, Kelliher F, Rollo M. 2014. Reporting to 2006 IPCC Guidelines for N2O Emissions from Additional Sources of Organic N: Final Report. MPI Technical Report. Wellington: Ministry for Primary Industries.

# Annex 7: Tokelau

## A7.1 	Preliminary processing of raw data to obtain activity data for liquid and gaseous fuels from Tokelau

Table A7.1.1 	Allocation of 2018 fuel sales

Note: 	AA = Atafu, FF = Fakaofo; NN = Nukunonu.

Table A7.1.2 	Tokelau vehicle photo survey August–December 2018

## A7.2	Emissions estimate data and relevant supporting information by category for Tokelau

Tokelau CRF Table 1.A.1.a: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.1  Energy Industries][1.A.1.a  Public Electricity and Heat Production] (Part 1 of 3)

| [1. Energy][1.AA Fuel Combustion – Sectoral approach][1.A.1 Energy Industries][1.A.1.a Public Electricity and Heat Production] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 |
| Liquid fuels | TJ | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Liquid fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 |
| Liquid fuels | kt | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 |
| CH4 | kt | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 |
| Liquid fuels | kt | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 |
| N2O | kt | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 |
| Liquid fuels | kt | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 |
| Amount captured |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Liquid fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | t/TJ | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 |
| CH4 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 |
| N2O |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 |

Tokelau CRF Table 1.A.1.a: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.1  Energy Industries][1.A.1.a  Public Electricity and Heat Production] (Part 2 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.1 Energy Industries][1.A.1.a Public Electricity and Heat Production] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 |
| Liquid fuels | TJ | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 | 16.34 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Liquid fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 |
| Liquid fuels | kt | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 |
| CH4 | kt | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 |
| Liquid fuels | kt | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 | 0.000047 |
| N2O | kt | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 |
| Liquid fuels | kt | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 | 0.000009 |
| Amount captured |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Liquid fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | t/TJ | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 |
| CH4 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 |
| N2O |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 |

Tokelau CRF Table 1.A.1.a: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.1  Energy Industries][1.A.1.a  Public Electricity and Heat Production] (Part 3 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.1 Energy Industries][1.A.1.a Public Electricity and Heat Production] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 16.34 | 16.34 | 12.97 | 2.86 | 2.86 | 2.86 | 2.86 | 2.86 |
| Liquid fuels | TJ | 16.34 | 16.34 | 12.97 | 2.86 | 2.86 | 2.86 | 2.86 | 2.86 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Liquid fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 1.15 | 1.15 | 0.91 | 0.20 | 0.20 | 0.20 | 0.20 | 0.20 |
| Liquid fuels | kt | 1.15 | 1.15 | 0.91 | 0.20 | 0.20 | 0.20 | 0.20 | 0.20 |
| CH4 | kt | 0.000047 | 0.000047 | 0.000037 | 0.000008 | 0.000008 | 0.000008 | 0.000008 | 0.000008 |
| Liquid fuels | kt | 0.000047 | 0.000047 | 0.000037 | 0.000008 | 0.000008 | 0.000008 | 0.000008 | 0.000008 |
| N2O | kt | 0.000009 | 0.000009 | 0.000007 | 0.000002 | 0.000002 | 0.000002 | 0.000002 | 0.000002 |
| Liquid fuels | kt | 0.000009 | 0.000009 | 0.000007 | 0.000002 | 0.000002 | 0.000002 | 0.000002 | 0.000002 |
| Amount captured |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NO | NO | NO | NO | NO | NO | NO | NO |
| Liquid fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |  |  |
| Liquid fuels | t/TJ | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 |
| CH4 |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 | 2.85 |
| N2O |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 | 0.57 |

Tokelau CRF Table 1.A.3.b.i: [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i  Cars][Gasoline] (Part 1 of 3)

| [1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Gasoline] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/T | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

Note: this category is included under 1.A.3.b.iii. For explanation please refer to section.

Tokelau CRF Table 1.A.3.b.i: [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i  Cars][Gasoline] (Part 2 of 3)

| [1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Gasoline] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/T | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

Tokelau CRF Table 1.A.3.b.i: [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i  Cars][Gasoline] (Part 3 of 3)

| [1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Gasoline] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/T | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.A.3.b.i Diesel Oil: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.3  Transport][1.A.3.b  Road Transportation][1.A.3.b.i  Cars][Diesel Oil]  (Part 1 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Diesel Oil] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.A.3.b.i Diesel Oil: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.3  Transport][1.A.3.b  Road Transportation][1.A.3.b.i  Cars][Diesel Oil]  (Part 2 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Diesel Oil] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.A.3.b.i Diesel Oil: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.3  Transport][1.A.3.b  Road Transportation][1.A.3.b.i  Cars][Diesel Oil]  (Part 3 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.3 Transport][1.A.3.b Road Transportation][1.A.3.b.i Cars][Diesel Oil] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | IE | IE | IE | IE | IE | IE | IE | IE |
| Method |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.A.3.d Gas/Diesel Oil: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.3  Transport][1.A.3.d  Domestic Navigation][Gas/Diesel Oil] (Part 1 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.3 Transport][1.A.3.d Domestic Navigation][Gas/Diesel Oil] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 |
| CH4 | kg/TJ | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 |
| N2O | kg/TJ | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 |

CRF Table 1.A.3.d Gas/Diesel Oil: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.3  Transport][1.A.3.d  Domestic Navigation][Gas/Diesel Oil (Part 2 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.3 Transport][1.A.3.d Domestic Navigation][Gas/Diesel Oil] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 |
| CH4 | kg/TJ | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 |
| N2O | kg/TJ | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 |

CRF Table 1.A.3.d Gas/Diesel Oil: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.3  Transport][1.A.3.d  Domestic Navigation][Gas/Diesel Oil (Part 3 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.3 Transport][1.A.3.d Domestic Navigation][Gas/Diesel Oil] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 | 22.70 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 | 1.60 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 | 70.40 |
| CH4 | kg/TJ | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 | 6.65 |
| N2O | kg/TJ | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 | 1.90 |

CRF Table 1.A.4.b: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.4  Other Sectors][1.A.4.b  Residential] (Part 1 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.4 Other Sectors][1.A.4.b Residential] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 |
| Liquid fuels | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | TJ | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Liquid fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Gaseous fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.08 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| SO2 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Amount captured |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Liquid fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | t/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | t/TJ | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 |
| CH4 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kg/TJ | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 |
| N2O |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kg/TJ | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |

CRF Table 1.A.4.b: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.4  Other Sectors][1.A.4.b  Residential] (Part 2 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.4 Other Sectors][1.A.4.b Residential] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 |
| Liquid fuels | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | TJ | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Liquid fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Gaseous fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| SO2 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Amount captured |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Liquid fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | t/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | t/TJ | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 |
| CH4 |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kg/TJ | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 |
| N2O |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kg/TJ | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |

CRF Table 1.A.4.b: [1.  Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.4  Other Sectors][1.A.4.b  Residential] (Part 3 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.4 Other Sectors][1.A.4.b Residential] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 |
| Liquid fuels | TJ | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | TJ | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Liquid fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Gaseous fuels |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Liquid fuels | kt | IE | IE | IE | IE | IE | IE | IE | IE |
| Gaseous fuels | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| SO2 | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| Amount captured |  |  |  |  |  |  |  |  |  |
| CO2 | kt | NO | NO | NO | NO | NO | NO | NO | NO |
| Liquid fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kt | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |  |  |  |
| Liquid fuels | t/TJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | t/TJ | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 | 56.79 |
| CH4 |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kg/TJ | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 | 55.80 |
| N2O |  |  |  |  |  |  |  |  |  |
| Liquid fuels | kg/TJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Gaseous fuels | kg/TJ | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |

CRF Table 1.A.4.c.iii Gas/Diesel Oil: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.4  Other Sectors][1.A.4.c  Agriculture/Forestry/Fishing]
[1.A.4.c.iii  Fishing][Gas/Diesel Oil] (Part 1 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.4 Other Sectors][1.A.4.c Agriculture/Forestry/Fishing][1.A.4.c.iii Fishing][Gas/Diesel Oil] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.A.4.c.iii Gas/Diesel Oil: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.4  Other Sectors][1.A.4.c  Agriculture/Forestry/Fishing][1.A.4.c.iii  Fishing]
[Gas/Diesel Oil] (Part 2 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.4 Other Sectors][1.A.4.c Agriculture/Forestry/Fishing][1.A.4.c.iii Fishing][Gas/Diesel Oil] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.A.4.c.iii Gas/Diesel Oil: [1. Energy][1.AA  Fuel Combustion - Sectoral approach][1.A.4  Other Sectors][1.A.4.c  Agriculture/Forestry/Fishing][1.A.4.c.iii  Fishing]
[Gas/Diesel Oil] (Part 3 of 3)

| [1. Energy][1.AA Fuel Combustion - Sectoral approach][1.A.4 Other Sectors][1.A.4.c Agriculture/Forestry/Fishing][1.A.4.c.iii Fishing][Gas/Diesel Oil] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Fuel Consumption | TJ | IE | IE | IE | IE | IE | IE | IE | IE |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Method |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| CO2 | kt | IE | IE | IE | IE | IE | IE | IE | IE |
| CH4 | kt | IE | IE | IE | IE | IE | IE | IE | IE |
| N2O | kt | IE | IE | IE | IE | IE | IE | IE | IE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| CO2 | t/TJ | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA |
| N2O | kg/TJ | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 1.AB Gasoline: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Gasoline] (Part 1 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Gasoline] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| CO2 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 1.AB Gasoline: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Gasoline] (Part 2 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Gasoline] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| CO2 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 1.AB Gasoline: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Gasoline] (Part 3 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Gasoline] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 | 10.79 |
| Emission Factor |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 | 17.96 |
| Carbon content |  |  |  |  |  |  |  |  |  |
| C | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Carbon stored |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |
| C | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |
| C | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| CO2 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 1.AB Gasoline: [1.  Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Gas / Diesel Oil] (Part 1 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Gas / Diesel Oil] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 39.04 | 39.04 | 39.04 | 39.04 | 39.04 | 39.04 | 39.04 | 39.04 | 39.04 | 39.04 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 |
| CO2 | kt | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 |

CRF Table 1.AB Gasoline: [1.  Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Gas / Diesel Oil] (Part 2 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Gas / Diesel Oil] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 39.04 | 39.04 | 39.04 | 39.04 | 39.04 | 39.04 | 39.04 | 39.04 | 39.04 | 39.04 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 |
| CO2 | kt | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 | 2.7468525 |

CRF Table 1.AB Gasoline: [1.  Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Gas / Diesel Oil] (Part 3 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Gas / Diesel Oil] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.04 | 0.04 | 0.04 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.04 | 0.04 | 0.04 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 39.04 | 39.04 | 35.67 | 25.56 | 25.56 | 25.56 | 25.56 | 25.56 |
| Emission Factor |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 | 19.19 |
| Carbon content |  |  |  |  |  |  |  |  |  |
| C | kt | 0.75 | 0.75 | 0.68 | 0.49 | 0.49 | 0.49 | 0.49 | 0.49 |
| Carbon stored |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |
| C | kt | 0.75 | 0.75 | 0.68 | 0.49 | 0.49 | 0.49 | 0.49 | 0.49 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |
| C | kt | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 | 0.75 |
| CO2 | kt | 2.7468525 | 2.7468525 | 2.5097435 | 1.7984167 | 1.7984167 | 1.7984167 | 1.7984167 | 1.7984167 |

CRF Table 1.AB Other Kerosene: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Other Kerosene] (Part 1 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Other Kerosene] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| CO2 | kt | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 |

CRF Table 1.AB Other Kerosene: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Other Kerosene] (Part 2 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Other Kerosene] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| CO2 | kt | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 |

CRF Table 1.AB Other Kerosene: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Other Kerosene] (Part 3 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Other Kerosene] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 | 0.77 |
| Emission Factor |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 | 19.60 |
| Carbon content |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Carbon stored |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| CO2 | kt | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 | 0.06 |

CRF Table 1.AB LPG: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] (Part 1 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| CO2 | kt | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 |

CRF Table 1.AB LPG: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] (Part 2 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| CO2 | kt | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 |

CRF Table 1.AB LPG: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] (Part 3 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Liquefied Petroleum Gases (LPG)] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 | 1.32 |
| Emission Factor |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 | 15.48 |
| Carbon content |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Carbon stored |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |
| C | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| CO2 | kt | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 |

CRF Table 1.AB Lubricants: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Lubricants] (Part 1 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Lubricants] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| CO2 | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |

CRF Table 1.AB Lubricants: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Lubricants] (Part 2 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Lubricants] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 |
| Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 |
| Carbon content |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Carbon stored |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| CO2 | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |

CRF Table 1.AB Lubricants: [1. Energy][1.AB  Fuel Combustion - Reference Approach][Liquid Fuels][Lubricants] (Part 3 of 3)

| [1. Energy][1.AB Fuel Combustion - Reference Approach][Liquid Fuels][Lubricants] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Imports | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Exports | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| International bunkers | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Stock change | PJ | NO | NO | NO | NO | NO | NO | NO | NO |
| Apparent Consumption | PJ | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Conversion factor | TJ/unit | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 | 1000.00 |
| Calorific Value |  | GCV | GCV | GCV | GCV | GCV | GCV | GCV | GCV |
| Apparent Consumption | TJ | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 | 0.31 |
| Emission Factor |  |  |  |  |  |  |  |  |  |
| C | t/TJ | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 | 19.00 |
| Carbon content |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Carbon stored |  |  |  |  |  |  |  |  |  |
| C | kt | NO | NO | NO | NO | NO | NO | NO | NO |
| Net carbon emissions |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Fraction of carbon oxidized |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Emissions |  |  |  |  |  |  |  |  |  |
| C | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| CO2 | kt | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |

CRF Table 2.F.1.b HFC-134a Product Uses as Substitutes for ODS: [2.  Industrial Processes and Product Use][ 2.F  Product Uses as Substitutes for ODS][ 2.F.1  Refrigeration and Air conditioning][ 2.F.1.b  Domestic Refrigeration][ HFC-134a] (Part 1 of 3)

| [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.1 Refrigeration and Air conditioning][ 2.F.1.b Domestic Refrigeration][ HFC-134a] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | 0.08 | 0.09 | 0.09 | 0.09 | 0.10 | 0.10 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.02 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.02 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.b HFC-134a Product Uses as Substitutes for ODS: [2.  Industrial Processes and Product Use][ 2.F  Product Uses as Substitutes for ODS][ 2.F.1  Refrigeration 
and Air conditioning][ 2.F.1.b  Domestic Refrigeration][ HFC-134a] (Part 2 of 3)

| [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.1 Refrigeration and Air conditioning][ 2.F.1.b Domestic Refrigeration][ HFC-134a] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.11 | 0.11 | 0.12 | 0.12 | 0.13 | 0.14 | 0.14 | 0.15 | 0.16 | 0.17 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.03 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.03 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.b HFC-134a Product Uses as Substitutes for ODS: [2.  Industrial Processes and Product Use][ 2.F  Product Uses as Substitutes for ODS][ 2.F.1  Refrigeration
and Air conditioning][ 2.F.1.b  Domestic Refrigeration][ HFC-134a] (Part 3 of 3)

| [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.1 Refrigeration and Air conditioning][ 2.F.1.b Domestic Refrigeration][ HFC-134a] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.18 | 0.19 | 0.19 | 0.20 | 0.20 | 0.20 | 0.21 | 0.21 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air Conditioning] (Part 1 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| HFCs | t CO2 -e | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| HFC-23 | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| HFC-32 | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| HFC-125 | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| HFC-134a | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| SF6 | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| NF3 | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| HFCs and PFCs | t CO2 -e | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Aggregated F-gases | t CO2 -e | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning] (Part 2 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | NA | NA | NA | NA | NA | NA | NA | T1a | T1a | T1a |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | NA | NA | NA | NA | NA | NA | NA | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| HFCs | t CO2 -e | NO | NO | NO | NO | NO | NO | NO | 0.02 | 0.03 | 0.05 |
| HFC-23 | t | NO | NO | 0.01 | 0.05 | NO | NO | 0.25 | NO | NO | NO |
| HFC-32 | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.01 | 0.01 |
| HFC-125 | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.01 | 0.01 |
| HFC-134a | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.00 | 0.00 |
| SF6 | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| NF3 | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| HFCs and PFCs | t CO2-e | NO | NO | NO | NO | NO | NO | NO | 0.02 | 0.03 | 0.05 |
| Aggregated F-gases | t CO2 -e | NO | NO | NO | NO | NO | NO | NO | 0.02 | 0.03 | 0.05 |

CRF Table 2.F.1.f: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning] (Part 3 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |
| HFCs |  | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| HFCs |  | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| HFCs | t CO2-e | 0.07 | 0.09 | 0.10 | 0.12 | 0.14 | 0.16 | 0.17 | 0.19 |
| HFC-23 | t | 0.01 | NO | NO | NO | NO | NO | NO |  |
| HFC-32 | t | 0.01 | 0.02 | 0.02 | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 |
| HFC-125 | t | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 | 0.04 | 0.04 | 0.05 |
| HFC-134a | t | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| SF6 | t | NO | NO | NO | NO | NO | NO | NO | NO |
| NF3 | t | NO | NO | NO | NO | NO | NO | NO | NO |
| HFCs and PFCs | t CO2-e | 0.07 | 0.09 | 0.10 | 0.12 | 0.14 | 0.16 | 0.17 | 0.19 |
| Aggregated F-gases | t CO2-e | 0.07 | 0.09 | 0.10 | 0.12 | 0.14 | 0.16 | 0.17 | 0.19 |

CRF Table 2.F.1.f HFC-32: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-32] (Part 1 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-32] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-32: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-32] (Part 2 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-32] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | 0.02 | 0.04 | 0.06 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.01 | 0.01 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | 0.00305 | 0.00609 | 0.00914 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-32: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-32] (Part 3 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-32] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.08 | 0.10 | 0.12 | 0.14 | 0.17 | 0.19 | 0.21 | 0.23 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | 0.01 | 0.02 | 0.02 | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | 0.01218 | 0.015225 | 0.01827 | 0.021315 | 0.024969 | 0.028014 | 0.031059 | 0.034104 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-125: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-125] (Part 1 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-125] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-125: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-125] (Part 2 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-125] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | 0.03 | 0.06 | 0.08 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.01 | 0.01 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.01 | 0.01 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-125: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-125] (Part 3 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-125] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.11 | 0.14 | 0.17 | 0.19 | 0.23 | 0.26 | 0.28 | 0.31 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 | 0.04 | 0.04 | 0.05 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 | 0.04 | 0.04 | 0.05 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-134a: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-134a] (Part 1 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-134a] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-134a: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-134a] (Part 2 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-134a] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.00 | 0.01 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.00 | 0.00 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | 0.00 | 0.00 | 0.00 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.1.f HFC-134a: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.1  Refrigeration and Air conditioning][2.F.1.f  Stationary Air conditioning][HFC-134a] (Part 3 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.1 Refrigeration and Air conditioning][2.F.1.f Stationary Air conditioning][HFC-134a] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.01 | 0.01 | 0.01 | 0.01 | 0.02 | 0.02 | 0.02 | 0.02 |
| Remaining in products at decommissioning | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| From disposal | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 | 15.00 |
| Disposal loss factor | % | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.4.a: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.4  Aerosols][2.F.4.a  Metered Dose Inhalers] (Part 1 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | NA | NA | NA | NA | NA | T1a | T1a | T1a | T1a | T1a |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | NA | NA | NA | NA | NA | D | D | D | D | D |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions |  | NO | NO | NO | NO | NO | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| HFCs | t CO2 equivalent | NO | NO | NO | NO | NO | 0.12 | 0.71 | 1.17 | 1.73 | 2.52 |
| HFC-134a | t | NO | NO | NO | NO | NO | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| HFC-227ea | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Aggregated F-gases | t CO2 equivalent | NO | NO | NO | NO | NO | 0.12 | 0.71 | 1.17 | 1.73 | 2.52 |

CRF Table 2.F.4.a: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.4  Aerosols][2.F.4.a  Metered Dose Inhalers] (Part 2 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| HFCs |  | D | D | D | D | D | D | D | D | D | D |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions |  | 0.00 | 0.01 | 0.01 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| HFCs | t CO2-e | 2.96 | 6.35 | 12.51 | 15.70 | 16.10 | 16.86 | 17.28 | 17.42 | 17.92 | 18.44 |
| HFC-134a | t | 0.00 | 0.00 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| HFC-227ea | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Aggregated F-gases | t CO2-e | 2.96 | 6.35 | 12.51 | 15.70 | 16.10 | 16.86 | 17.28 | 17.42 | 19.92 | 18.44 |

CRF Table 2.F.4.a: [2.  Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.4  Aerosols][2.F.4.a  Metered Dose Inhalers] (Part 3 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |
| HFCs |  | T1a | T1a | T1a | T1a | T1a | T1a | T1a | T1a |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| HFCs |  | D | D | D | D | D | D | D | D |
| PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA |
| Unspecified mix of HFCs and PFCs |  | NA | NA | NA | NA | NA | NA | NA | NA |
| SF6 |  | NA | NA | NA | NA | NA | NA | NA | NA |
| NF3 |  | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions |  | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| HFCs | t CO2-e | 19.08 | 21.00 | 22.73 | 22.58 | 22.27 | 22.31 | 22.01 | 21.55 |
| HFC-134a | t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| HFC-227ea | t | NO | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Aggregated F-gases | t CO2-e | 19.08 | 21.00 | 22.73 | 22.58 | 22.27 | 22.31 | 22.01 | 21.55 |

CRF Table 2.F.4.a HFC-134a: [ 2.  Industrial Processes and Product Use][ 2.F  Product Uses as Substitutes for ODS][ 2.F.4  Aerosols][ 2.F.4.a  Metered Dose Inhalers][ HFC-134a] 
(Part 1 of 3)

| [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.4 Aerosols][ 2.F.4.a Metered Dose Inhalers][ HFC-134a] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Emissions | t | NO | NO | NO | NO | NO | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |

CRF Table 2.F.4.a HFC-134a: [ 2.  Industrial Processes and Product Use][ 2.F  Product Uses as Substitutes for ODS][ 2.F.4  Aerosols][ 2.F.4.a  Metered Dose Inhalers][ HFC-134a] 
(Part 2 of 3)

| [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.4 Aerosols][ 2.F.4.a Metered Dose Inhalers][ HFC-134a] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.00 | 0.00 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Emissions | t | 0.00 | 0.00 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |

CRF Table 2.F.4.a HFC-134a: [ 2.  Industrial Processes and Product Use][ 2.F  Product Uses as Substitutes for ODS][ 2.F.4  Aerosols][ 2.F.4.a  Metered Dose Inhalers][ HFC-134a] 
(Part 3 of 3)

| [ 2. Industrial Processes and Product Use][ 2.F Product Uses as Substitutes for ODS][ 2.F.4 Aerosols][ 2.F.4.a Metered Dose Inhalers][ HFC-134a] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Emissions | t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |

CRF Table 2.F.4.a HFC-227ea: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.4  Aerosols][2.F.4.a  Metered Dose Inhalers][HFC-227ea] 
(Part 1 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers][HFC-227ea] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.4.a HFC-227ea: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.4  Aerosols][2.F.4.a  Metered Dose Inhalers][HFC-227ea] 
(Part 2 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers][HFC-227ea] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Emissions | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |

CRF Table 2.F.4.a HFC-227ea: [2. Industrial Processes and Product Use][2.F  Product Uses as Substitutes for ODS][2.F.4  Aerosols][2.F.4.a  Metered Dose Inhalers][HFC-227ea] 
(Part 3 of 3)

| [2. Industrial Processes and Product Use][2.F Product Uses as Substitutes for ODS][2.F.4 Aerosols][2.F.4.a Metered Dose Inhalers][HFC-227ea] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount |  |  |  |  |  |  |  |  |  |
| Filled into new manufactured products | t | NO | NO | NO | NO | NO | NO | NO | NO |
| In operating systems (average annual stocks) | t | NO | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Emissions | t | NO | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| From manufacturing | t | NO | NO | NO | NO | NO | NO | NO | NO |
| From stocks | t | NO | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Recovery | t | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| Product manufacturing factor | % | NO | NO | NO | NO | NO | NO | NO | NO |
| Product life factor | % | NO | 100.00 | 100.0 | 100.0 | 100.0 | 100.0 | 100.0 | 100.0 |

CRF Table 2.G.3.a: [2.  Industrial Processes and Product Use][2.G  Other Product Manufacture and Use][2.G.3  N2O from Product Uses][2.G.3.a  Medical Applications] (Part 1 of 3)

| [2. Industrial Processes and Product Use][2.G Other Product Manufacture and Use][2.G.3 N2O from Product Uses][2.G.3.a Medical Applications] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Activity data |  |  |  |  |  |  |  |  |  |  |  |
| N2O imported | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Recovery |  |  |  |  |  |  |  |  |  |  |  |
| N2O | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| N2O | t/t | 1.03 | 1.03 | 1.03 | 1.03 | 1.03 | 1.03 | 1.03 | 1.03 | 1.03 | 1.03 |

CRF Table 2.G.3.a: [2.  Industrial Processes and Product Use][2.G  Other Product Manufacture and Use][2.G.3  N2O from Product Uses][2.G.3.a  Medical Applications] (Part 2 of 3)

| [2. Industrial Processes and Product Use][2.G Other Product Manufacture and Use][2.G.3 N2O from Product Uses][2.G.3.a Medical Applications] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Activity data |  |  |  |  |  |  |  |  |  |  |  |
| N2O imported | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Recovery |  |  |  |  |  |  |  |  |  |  |  |
| N2O | kt | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| N2O | t/t | 1.03 | 1.03 | 1.03 | 1.00 | 0.98 | 0.97 | 0.96 | 0.96 | 1.03 | 1.05 |

CRF Table 2.G.3.a: [2.  Industrial Processes and Product Use][2.G  Other Product Manufacture and Use][2.G.3  N2O from Product Uses][2.G.3.a  Medical Applications] (Part 3 of 3)

| [2. Industrial Processes and Product Use][2.G Other Product Manufacture and Use][2.G.3 N2O from Product Uses][2.G.3.a Medical Applications] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Activity data |  |  |  |  |  |  |  |  |  |
| N2O imported | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Method |  |  |  |  |  |  |  |  |  |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| N2O |  | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Recovery |  |  |  |  |  |  |  |  |  |
| N2O | kt | NO | NO | NO | NO | NO | NO | NO | NO |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| N2O | t/t | 1.00 | 1.03 | 0.98 | 1.04 | 1.01 | 1.00 | 1.02 | 0.99 |

CRF Table 3.A.3 Tokelau Swine: [ 3.  Agriculture][ 3.1  Livestock][ 3.A  Enteric Fermentation][ 3.A.3  Swine][ Other (please specify)][ Tokelau_Swine] (Part 1 of 3)

| [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.3 Swine][ Other (please specify)][ Tokelau_Swine] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.520 | 2.500 | 2.480 | 2.460 | 2.440 | 2.420 | 2.400 | 2.380 | 2.360 | 2.340 |
| Average gross energy intake | MJ/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Average CH4 conversion rate | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Weight | kg | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 |
| Feeding situation |  | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen |
| Milk yield | kg/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Work | h/day | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Pregnant | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Digestibility of feed | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Gross energy | MJ/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 3.A.3 Tokelau Swine: [ 3.  Agriculture][ 3.1  Livestock][ 3.A  Enteric Fermentation][ 3.A.3  Swine][ Other (please specify)][ Tokelau_Swine] (Part 2 of 3)

| [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.3 Swine][ Other (please specify)][ Tokelau_Swine] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.320 | 2.300 | 2.280 | 2.260 | 2.240 | 2.220 | 2.200 | 2.180 | 2.160 | 2.140 |
| Average gross energy intake | MJ/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Average CH4 conversion rate | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Weight | kg | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 |
| Feeding situation |  | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen |
| Milk yield | kg/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Work | h/day | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Pregnant | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Digestibility of feed | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Gross energy | MJ/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 3.A.3 Tokelau Swine: [ 3.  Agriculture][ 3.1  Livestock][ 3.A  Enteric Fermentation][ 3.A.3  Swine][ Other (please specify)][ Tokelau_Swine] (Part 3 of 3)

| [ 3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.3 Swine][ Other (please specify)][ Tokelau_Swine] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.120 | 2.100 | 2.080 | 2.060 | 2.040 | 2.020 | 2.000 | 1.980 |
| Average gross energy intake | MJ/head/day | NA | NA | NA | NA | NA | NA | NA | NA |
| Average CH4 conversion rate | % | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 | 1.50 |
| Additional information |  |  |  |  |  |  |  |  |  |
| Weight | kg | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 |
| Feeding situation |  | Pen | Pen | Pen | Pen | Pen | Pen | Pen | Pen |
| Milk yield | kg/day | NA | NA | NA | NA | NA | NA | NA | NA |
| Work | h/day | NO | NO | NO | NO | NO | NO | NO | NO |
| Pregnant | % | NA | NA | NA | NA | NA | NA | NA | NA |
| Digestibility of feed | % | NA | NA | NA | NA | NA | NA | NA | NA |
| Gross energy | MJ/day | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 3.A.3 Tokelau Poultry [ 3. Agriculture][ 3.1  Livestock][ 3.A  Enteric Fermentation][ 3.A.4  Other livestock][ Tokelau_Poultry] (Part 1 of 3)

| [3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.4 Other livestock][ Tokelau_Poultry] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 3.620 | 3.500 | 3.380 | 3.260 | 3.140 | 3.020 | 2.900 | 2.780 | 2.660 | 2.540 |
| Average gross energy intake | MJ/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Average CH4 conversion rate | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Weight | kg | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Feeding situation |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Milk yield | kg/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Work | h/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Pregnant | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Digestibility of feed | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Gross energy | MJ/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 3.A.3 Tokelau Poultry [ 3. Agriculture][ 3.1  Livestock][ 3.A  Enteric Fermentation][ 3.A.4  Other livestock][ Tokelau_Poultry] (Part 2 of 3)

| [3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.4 Other livestock][ Tokelau_Poultry] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.420 | 2.300 | 2.180 | 2.060 | 1.940 | 1.820 | 1.700 | 1.580 | 1.460 | 1.340 |
| Average gross energy intake | MJ/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Average CH4 conversion rate | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Weight | kg | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Feeding situation |  | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Milk yield | kg/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Work | h/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Pregnant | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Digestibility of feed | % | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Gross energy | MJ/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 3.A.3 Tokelau Poultry [ 3. Agriculture][ 3.1  Livestock][ 3.A  Enteric Fermentation][ 3.A.4  Other livestock][ Tokelau_Poultry] (Part 3 of 3)

| [3. Agriculture][ 3.1 Livestock][ 3.A Enteric Fermentation][ 3.A.4 Other livestock][ Tokelau_Poultry] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 1.220 | 1.100 | 0.980 | 0.860 | 0.740 | 0.620 | 0.500 | 0.380 |
| Average gross energy intake | MJ/head/day | NA | NA | NA | NA | NA | NA | NA | NA |
| Average CH4 conversion rate | % | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |
| CH4 |  | NA | NA | NA | NA | NA | NA | NA | NA |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CH4 |  | NA | NA | NA | NA | NA | NA | NA | NA |
| Emissions |  |  |  |  |  |  |  |  |  |
| CH4 | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | NE | NE | NE | NE | NE | NE | NE | NE |
| Additional information |  |  |  |  |  |  |  |  |  |
| Weight | kg | NA | NA | NA | NA | NA | NA | NA | NA |
| Feeding situation |  | NA | NA | NA | NA | NA | NA | NA | NA |
| Milk yield | kg/day | NA | NA | NA | NA | NA | NA | NA | NA |
| Work | h/day | NA | NA | NA | NA | NA | NA | NA | NA |
| Pregnant | % | NA | NA | NA | NA | NA | NA | NA | NA |
| Digestibility of feed | % | NA | NA | NA | NA | NA | NA | NA | NA |
| Gross energy | MJ/day | NA | NA | NA | NA | NA | NA | NA | NA |

CRF Table 3.B.1.3 Tokelau Swine: [3.  Agriculture][ 3.1  Livestock][ 3.B  Manure Management][ 3.B.1  CH4 Emissions][ 3.B.1.3  Swine][ Other (please specify)][ Pigs] (Part 1 of 3)

| [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.3 Swine][ Other (please specify)][ Pigs] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.520 | 2.500 | 2.480 | 2.460 | 2.440 | 2.420 | 2.400 | 2.380 | 2.360 | 2.340 |
| Allocation by climate region |  |  |  |  |  |  |  |  |  |  |  |
| Warm | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |
| Typical animal mass (average) | kg | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 |
| VS daily excretion (average) | kg dm/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 producing potential (average) | m^3/kg VS | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 |

CRF Table 3.B.1.3 Tokelau Swine: [3.  Agriculture][ 3.1  Livestock][ 3.B  Manure Management][ 3.B.1  CH4 Emissions][ 3.B.1.3  Swine][ Other (please specify)][ Pigs] (Part 2 of 3)

| [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.3 Swine][ Other (please specify)][ Pigs] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.320 | 2.300 | 2.280 | 2.260 | 2.240 | 2.220 | 2.200 | 2.180 | 2.160 | 2.140 |
| Allocation by climate region |  |  |  |  |  |  |  |  |  |  |  |
| Warm | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |
| Typical animal mass (average) | kg | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 |
| VS daily excretion (average) | kg dm/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 producing potential (average) | m^3/kg VS | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 |

CRF Table 3.B.1.3 Tokelau Swine: [3.  Agriculture][ 3.1  Livestock][ 3.B  Manure Management][ 3.B.1  CH4 Emissions][ 3.B.1.3  Swine][ Other (please specify)][ Pigs] (Part 3 of 3)

| [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.3 Swine][ Other (please specify)][ Pigs] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.120 | 2.100 | 2.080 | 2.060 | 2.040 | 2.020 | 2.000 | 1.980 |
| Allocation by climate region |  |  |  |  |  |  |  |  |  |
| Warm | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |
| Typical animal mass (average) | kg | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 | 80.00 |
| VS daily excretion (average) | kg dm/head/day | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 producing potential (average) | m^3/kg VS | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 | 18.50 |

CRF Table 3.B.1.4 Tokelau Poultry: [ 3.  Agriculture][ 3.1  Livestock][ 3.B  Manure Management][ 3.B.1  CH4 Emissions][ 3.B.1.4  Other livestock][ Tokelau_Poultry] Part 1 of 3)

| [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.4 Other livestock][ Tokelau_Poultry] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 3.620 | 3.500 | 3.380 | 3.260 | 3.140 | 3.020 | 2.900 | 2.780 | 2.660 | 2.540 |
| Allocation by climate region |  |  |  |  |  |  |  |  |  |  |  |
| Cool | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Temperature | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Warm | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |
| Typical animal mass (average) | kg | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| VS daily excretion (average) | dm/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 producing potential (average) | m^3/kg VS | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |

CRF Table 3.B.1.4 Tokelau Poultry: [ 3.  Agriculture][ 3.1  Livestock][ 3.B  Manure Management][ 3.B.1  CH4 Emissions][ 3.B.1.4  Other livestock][ Tokelau_Poultry] Part 2 of 3)

| [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.4 Other livestock][ Tokelau_Poultry] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 2.420 | 2.300 | 2.180 | 2.060 | 1.940 | 1.820 | 1.700 | 1.580 | 1.460 | 1.340 |
| Allocation by climate region |  |  |  |  |  |  |  |  |  |  |  |
| Cool | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Temperature | % | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Warm | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |
| Typical animal mass (average) | kg | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| VS daily excretion (average) | dm/head/day | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 producing potential (average) | m^3/kg VS | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |

CRF Table 3.B.1.4 Tokelau Poultry: [ 3.  Agriculture][ 3.1  Livestock][ 3.B  Manure Management][ 3.B.1  CH4 Emissions][ 3.B.1.4  Other livestock][ Tokelau_Poultry] Part 3 of 3)

| [ 3. Agriculture][ 3.1 Livestock][ 3.B Manure Management][ 3.B.1 CH4 Emissions][ 3.B.1.4 Other livestock][ Tokelau_Poultry] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Population | 1000s | 1.220 | 1.100 | 0.980 | 0.860 | 0.740 | 0.620 | 0.500 | 0.380 |
| Allocation by climate region |  |  |  |  |  |  |  |  |  |
| Cool | % | NO | NO | NO | NO | NO | NO | NO | NO |
| Temperature | % | NO | NO | NO | NO | NO | NO | NO | NO |
| Warm | % | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 |
| Typical animal mass (average) | kg | NA | NA | NA | NA | NA | NA | NA | NA |
| VS daily excretion (average) | dm/head/day | NA | NA | NA | NA | NA | NA | NA | NA |
| CH4 producing potential (average) | m^3/kg VS | NA | NA | NA | NA | NA | NA | NA | NA |
| Method |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| CH4 | kg/head/year | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |

CRF Table 5.C: [5.  Waste][5.C  Incineration and Open Burning of Waste]   (Part 1 of 3)

| [5. Waste][5.C Incineration and Open Burning of Waste] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions | kt CO2 equivalent | 0.16 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.14 | 0.14 | 0.14 | 0.14 |
| CO2 | kt | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.04 | 0.04 | 0.04 | 0.04 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| SO2 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |

CRF Table 5.C: [5.  Waste][5.C  Incineration and Open Burning of Waste]   (Part 2 of 3)

| [5. Waste][5.C Incineration and Open Burning of Waste] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions | kt CO2 equivalent | 0.14 | 0.14 | 0.14 | 0.14 | 0.14 | 0.14 | 0.14 | 0.14 | 0.14 | 0.14 |
| CO2 | kt | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| SO2 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |

CRF Table 5.C: [5.  Waste][5.C  Incineration and Open Burning of Waste]   (Part 3 of 3)

| [5. Waste][5.C Incineration and Open Burning of Waste] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CO2 |  | D | D | D | D | D | D | D | D |
| CH4 |  | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D |
| Emissions | kt CO2 equivalent | 0.14 | 0.13 | 0.13 | 0.13 | 0.13 | 0.13 | 0.14 | 0.14 |
| CO2 | kt | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| SO2 | kt | NE | NE | NE | NE | NE | NE | NE | NE |

CRF Table 5.C.2: [5.  Waste] [5.C  Incineration and Open Burning of Waste][5.C.2  Open Burning of Waste] (Part 1 of 3)

| [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount of Waste incinerated/open | kt | 0.59 | 0.55 | 0.55 | 0.55 | 0.55 | 0.55 | 0.52 | 0.52 | 0.52 | 0.52 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | D | D | D | D | D | D | D | D | D | D | D |
| CH4 | D | D | D | D | D | D | D | D | D | D | D |
| N2O | D | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.04 | 0.04 | 0.04 | 0.04 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| SO2 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kg/t | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 |
| CH4 | kg/t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| N2O | kg/t | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 5.C.2: [5.  Waste] [5.C  Incineration and Open Burning of Waste][5.C.2  Open Burning of Waste] (Part 2 of 3)

| [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount of Waste incinerated/open | kt | 0.52 | 0.52 | 0.52 | 0.52 | 0.52 | 0.52 | 0.52 | 0.52 | 0.52 | 0.52 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | D | D | D | D | D | D | D | D | D | D | D |
| CH4 | D | D | D | D | D | D | D | D | D | D | D |
| N2O | D | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| SO2 | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kg/t | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 |
| CH4 | kg/t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| N2O | kg/t | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 5.C.2: [5.  Waste] [5.C Incineration and Open Burning of Waste][5.C.2  Open Burning of Waste] (Part 3 of 3)

| [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount of Waste incinerated/open | kt | 0.52 | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 | 0.52 | 0.52 |
| Method |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CO2 | D | D | D | D | D | D | D | D | D |
| CH4 | D | D | D | D | D | D | D | D | D |
| N2O | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| SO2 | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| CO2 | kg/t | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 |
| CH4 | kg/t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| N2O | kg/t | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 5.C.2.2: [5.  Waste][5.C  Incineration and Open Burning of Waste][5.C.2  Open Burning of Waste][5.C.2.2  Non-biogenic] (Part 1 of 3)

| [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste][5.C.2.2 Non-biogenic] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount of Waste incinerated/open | kt | 0.59 | 0.55 | 0.55 | 0.55 | 0.55 | 0.55 | 0.52 | 0.52 | 0.52 | 0.52 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | D | D | D | D | D | D | D | D | D | D | D |
| CH4 | D | D | D | D | D | D | D | D | D | D | D |
| N2O | D | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.04 | 0.04 | 0.04 | 0.04 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kg/t | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 |
| CH4 | kg/t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| N2O | kg/t | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 5.C.2.2: [5.  Waste][5.C  Incineration and Open Burning of Waste][5.C.2  Open Burning of Waste][5.C.2.2  Non-biogenic] (Part 2 of 3)

| [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste][5.C.2.2 Non-biogenic] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount of Waste incinerated/open | kt | 0.52 | 0.52 | 0.52 | 0.52 | 0.52 | 0.52 | 0.52 | 0.52 | 0.52 | 0.52 |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | D | D | D | D | D | D | D | D | D | D | D |
| CH4 | D | D | D | D | D | D | D | D | D | D | D |
| N2O | D | D | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | kg/t | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 |
| CH4 | kg/t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| N2O | kg/t | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 5.C.2.2: [5.  Waste][5.C  Incineration and Open Burning of Waste][5.C.2  Open Burning of Waste][5.C.2.2  Non-biogenic] (Part 3 of 3)

| [5. Waste][5.C Incineration and Open Burning of Waste][5.C.2 Open Burning of Waste][5.C.2.2 Non-biogenic] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Amount of Waste incinerated/open | kt | 0.52 | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 | 0.52 | 0.52 |
| Method |  |  |  |  |  |  |  |  |  |
| CO2 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CO2 | D | D | D | D | D | D | D | D | D |
| CH4 | D | D | D | D | D | D | D | D | D |
| N2O | D | D | D | D | D | D | D | D | D |
| Emissions |  |  |  |  |  |  |  |  |  |
| CO2 | kt | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| CH4 | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Implied Emission Factor |  |  |  |  |  |  |  |  |  |
| CO2 | kg/t | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 |
| CH4 | kg/t | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| N2O | kg/t | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 5.D: [5.  Waste][5.D  Wastewater Treatment and Discharge] (Part 1 of 3)

| [5. Waste][5.D Wastewater Treatment and Discharge] | Unit | Base year (1990) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions | kt CO2 equivalent | 0.18 | 0.17 | 0.17 | 0.17 | 0.17 | 0.17 | 0.16 | 0.16 | 0.16 | 0.16 |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Population | 1000s | 1.700 | 1.600 | 1.600 | 1.600 | 1.600 | 1.600 | 1.500 | 1.500 | 1.500 | 1.500 |
| Protein consumption | kg/person/yr | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 |
| Fraction of nitrogen in protein |  | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 |
| Factor of non-consumed protein added to the wastewater |  | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 |
| Factor of industrial and commercial co-discharged protein into the sewer system |  | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 |
| Degree of utilization of modern, centralized WWT plants | % | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 5.D: [5.  Waste][5.D  Wastewater Treatment and Discharge] (Part 2 of 3)

| [5. Waste][5.D Wastewater Treatment and Discharge] | Unit | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D | D | D |
| Emissions | kt CO2 equivalent | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Population | 1000s | 1.500 | 1.500 | 1.500 | 1.500 | 1.500 | 1.500 | 1.500 | 1.500 | 1.500 | 1.500 |
| Protein consumption | kg/person/yr | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 |
| Fraction of nitrogen in protein |  | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 |
| Factor of non-consumed protein added to the wastewater |  | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 |
| Factor of industrial and commercial co-discharged protein into the sewer system |  | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 |
| Degree of utilization of modern, centralized WWT plants | % | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

CRF Table 5.D: [5.  Waste][5.D  Wastewater Treatment and Discharge] (Part 3 of 3)

| [5. Waste][5.D Wastewater Treatment and Discharge] | Unit | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) | (kt CO2-equivalent) |
| Method |  |  |  |  |  |  |  |  |  |
| CH4 |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| N2O |  | T1 | T1 | T1 | T1 | T1 | T1 | T1 | T1 |
| Emission Factor information |  |  |  |  |  |  |  |  |  |
| CH4 |  | D | D | D | D | D | D | D | D |
| N2O |  | D | D | D | D | D | D | D | D |
| Emissions | kt CO2 equivalent | 0.16 | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 | 0.16 | 0.16 |
| CH4 | kt | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| N2O | kt | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| NOx | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| CO | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| NMVOC | kt | NE | NE | NE | NE | NE | NE | NE | NE |
| Additional information |  |  |  |  |  |  |  |  |  |
| Population | 1000s | 1.500 | 1.400 | 1.400 | 1.400 | 1.400 | 1.400 | 1.500 | 1.500 |
| Protein consumption | kg/person/yr | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 | 32.45 |
| Fraction of nitrogen in protein |  | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 |
| Factor of non-consumed protein added to the wastewater |  | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 | 1.10 |
| Factor of industrial and commercial co-discharged protein into the sewer system |  | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 | 1.25 |
| Degree of utilization of modern, centralized WWT plants | % | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

# Annex 8: Agricultural emissions from fertilisers and by livestock type

## A8.1 	Agricultural emissions from fertilisers

Fertilisers provide the nutrients to grow and nourish pastures and crops. Nitrogen, phosphate, potassium and sulphur are the four most important nutrients for crop yields and sustainable food production.

New Zealand’s farmers use both organic and synthetic nitrogen (N) fertilisers. Two types of synthetic N fertilisers are used in New Zealand – urea and ammonium phosphates – mostly in a form of di-ammonium phosphate (DAP). Urea fertiliser accounts for most of the synthetic N fertiliser used in New Zealand. It is mainly applied to dairy pasture land to boost pasture growth during the autumn and spring months.

All fertilisers provide N inputs to agricultural soils that result in direct and indirect emissions of nitrous oxide (N2O) (see figure 5.5.1 in chapter 5). Urea also releases carbon dioxide (CO2).

Emissions from organic fertilisers are coming solely from animal manure. Most animal manure in New Zealand is excreted directly onto pasture, but some manure from dairy farms is kept in manure management systems and applied to soils as an organic fertiliser (see table 5.3.2 in chapter 5, for further details). Some manure is also collected but not stored; rather, it is spread directly onto pasture daily (e.g., swine manure and some dairy manure).

Emissions of N2O from all synthetic (including urea) N fertilisers are reported in categories 3.D.1.1 and 3.D.1.2 respectively. Emissions of CO2 from urea are not included under synthetic N fertilisers and are reported under a dedicated category 3.H.

##### 2017

In 2017, the combined effect of N synthetic and organic fertilisers amounted to 20.7 per cent of emissions from the Agricultural soils category and 6.1 per cent from all agricultural emissions.

Table A8.1.1 shows comparisons of both N2O and CO2 emissions from fertilisers to New Zealand’s national totals for each gas and New Zealand’s gross emissions.

Table A8.1.1 	Direct and indirect emissions by fertiliser in 2017

| Fertiliser type | Emissions |  | Percentage of |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | all N2O emissions from fertilisers | N2O emissions from Agriculture soils by gas | all emissions from Agriculture | All gross emissions |
|  | Gas/source | kt CO2-e | % | % | % | % |
| Synthetic N fertiliser | Direct N2O emissions | 1,365.6 | 77.6 | 15.9 | 3.5 | 1.7 |
|  | Urea | 1,019.5 | 57.9 | 11.9 | 2.6 | 1.3 |
|  | Other synthetic N fertilisers | 346.1 | 19.7 | 4.0 | 0.9 | 0.4 |
|  | Indirect N2O emissions from all synthetic N fertilisers | 294.7 | 16.7 | 3.4 | 0.8 | 0.4 |
|  | All N2O (direct + indirect) from synthetic N fertilisers | 1,660.3 | 94.3 | 19.4 | 4.3 | 2.1 |
|  | CO2 from urea | 588.3 | NA | NA | 1.5 | 0.7 |
| Organic fertiliser | Direct N2O emissions | 78.5 | 4.5 | 0.9 | 0.2 | 0.1 |
|  | Indirect N2O emissions | 32.0 | 1.8 | 0.4 | 0.1 | 0.0 |
|  | All N2O (direct + indirect) from organic fertilisers | 110.5 | 2.8 | 1.3 | 0.3 | 0.1 |

Note: 	Columns may not add up due to rounding.

##### 1990 – 2017

The total amount of fertilisers applied to agricultural soils in New Zealand has significantly increased since 1990. Synthetic N fertiliser applied to agricultural land has increased by 647.3 per cent since 1990, while the use of organic fertiliser has grown by nearly 178.2 per cent (figure A8.1.1, table A8.1.1)

Figure A8.1.1 	Use of fertilisers (tonnes (N)) in New Zealand in 1990 and 2017

21.4%

32.6%

46%

9.2%

15.2%

75.7%

Note: DAP = di-ammonium phosphate.

Table A8.1.2 	Use of fertilisers in New Zealand in 1990 and 2017

| Fertiliser type | 1990 |  |  | 2017 |  |  | Change in the use between 1990 and 2017 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Application | Percentage of |  | Application | Percentage of |  |  |  |
|  | tonnes (N) | synthetic N fertiliser | All fertilisers | tonnes (N) | synthetic N fertiliser | All fertilisers | tonnes | % |
| Inorganic N fertiliser (ammonium phosphates, for example, DAP) | 34,679 | 58.5 | 46.0 | 73,900 | 16.7 | 15.2 | 39,221.0 | 113.1 |
| Urea | 24,586 | 41.5 | 32.6 | 369,000 | 83.3 | 75.7 | 344,414.0 | 1,400.9 |
| Total synthetic N fertilisers (urea + ammonium phosphates) | 59,265 | 100.0 | 78.6 | 442,900 | 100.0 | 90.8 | 383,635.0 | 647.3 |
| Organic fertilisers (animal manure applied to soils) | 16,120 | NA | 21.4 | 44,838 | NA | 9.2 | 28,718.8 | 178.2 |

Note: 	DAP = di-ammonium phosphate. Columns may not add up due to rounding.

Between 1990 and 2017, N2O emissions from N synthetic fertiliser (both direct and indirect emissions, including urea) have increased by 510.9 per cent, while total emissions from these fertilisers (N2O and CO2) have increased by 623.1 per cent. For the same period, emissions from organic fertilisers increased by 200.1 per cent (see table A8.2.3).

In 1990 and 2017 respectively, 0.8 per cent and 4.3 per cent of agricultural emissions originated from N2O from synthetic N fertiliser. Total emissions from synthetic N fertiliser (including urea) have increased from 0.9 per cent to 5.8 per cent of agricultural emissions (see chapter 5 for further details).

Table A8.2.3 	Emissions from fertilisers in 1990 and 2017

|  |  |  | synthetic N fertilisers | Organic fertilisers |
| --- | --- | --- | --- | --- |
| 1990 | N2O emissions | kt CO2-e | 272.6 | 48.3 |
|  | CO2 emissions | kt | 39.2 | NA |
|  | Total emissions | kt CO2-e | 311.8 | 48.3 |
| 2017 | N2O emissions | kt CO2-e | 1,660.3 | 110.5 |
|  | CO2 emissions | kt | 588.3 | NA |
|  | Total emissions | kt CO2-e | 2,248.6 | 110.5 |
| Change in N2O emissions between 1990 and 2017 |  | kt CO2-e | 1,388.5 | 73.7 |
| Percentage change in N2O emissions between 1990 and 2017 |  | % | 510.9 | 200.1 |
| Change in all emissions between 1990 and 2017 |  | kt CO2-e | 1,937.6 | 73.7 |
| Percentage change in all emissions between 1990 and 2017 |  | % | 623.1 | 200.1 |

## A8.2 	Agricultural emissions by livestock type

This section covers distribution of GHG emissions from the Agriculture sector by livestock type in 1990, 2016 and 2017, including the changes in emissions. Table A8.2.1 shows total emissions of all GHGs across all categories of the Agriculture sector. For further details on emissions by gas and by category, refer to the common reporting format (CRF) tables (sector 3 – Agriculture).

Table A8.2.1 	Total emissions by livestock type in 1990, 2016 and 2017

| Livestock type | 1990 | 2016 | 2017 | 1990–2017 |  | 2016–2017 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | kt CO2-e |  |  | kt CO2-e | % | kt CO2-e | % |
| Dairy cattle | 7,869.4 | 18,231.7 | 18,199.1 | 10,329.8 | 131.3 | –32.5 | –0.2 |
| Non-dairy cattle | 7,084.8 | 6,474.2 | 6,562.8 | –522.0 | –7.4 | 88.6 | 1.4 |
| Sheep | 17,366.6 | 10,378.4 | 10,288.5 | –7,078.1 | –40.8 | –89.9 | –0.9 |
| Deer | 552.9 | 611.0 | 608.3 | 55.5 | 10.0 | –2.7 | –0.4 |
| Swine | 102.0 | 71.3 | 74.9 | –27.1 | –26.6 | 3.6 | 5.0 |
| Goats | 262.8 | 30.2 | 27.4 | –235.3 | –89.6 | –2.8 | –9.3 |
| Horses | 78.4 | 40.7 | 36.4 | –42.0 | –53.6 | –4.3 | –10.6 |
| Alpaca | 0.1 | 2.2 | 2.6 | 2.5 | 2,354.4 | 0.4 | 19.9 |
| Mules and asses | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Poultry (including all types of poultry) | 26.4 | 53.8 | 56.4 | 30.0 | 113.6 | 2.6 | 4.8 |
| Total, all livestock types | 33,343.2 | 35,893.6 | 35,856.6 | 2,513.4 | 7.5 | –37.1 | –0.1 |

Note: 	Columns may not add up due to rounding.

Value is for 2017. In 1990, the value was EF 7.4 kg CH4/head/year. Values for the intermediate years between 1990 and 2009 and for 2010–14 are interpolated and extrapolated based on an assumption that the dairy goat population has remained in a near constant state over time.

As was reported in the 2010 submission, that is, the first year that alpacas were included in New Zealand’s Greenhouse Gas Inventory (Ministry for the Environment, 2010).

Paragraph 33(c) of annex to Decision 2/CMP.7 contained in document FCCC/KP/CMP/2011/10/Add.1, p. 18.

MMS stands for a manure management system (see chapter 5).

MMS stands for a manure management system (see chapter 5).

The category names and CRF codes for source categories are consistent with New Zealand’s CRF tables. Only the tables that include reported emissions (by value, IE or NE) are included. For explanations and methodological issues, please refer to chapter 8.